IEEE P802.22
Wireless RANs

| Initial signal processing of captured DTV signals for evaluation of detection algorithms |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-08-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Suhas Mathur | Qualcomm | 5775 Morehouse Drive, San Diego, CA 92121 | (858)-651-4845 | smathur@qualcomm.com |
| Rahul Tandra | Qualcomm | 5775 Morehouse Drive, San Diego, CA 92121 | (858)-845-1970 | rtandra@qualcomm.com |
| Steve Shellhammer | Qualcomm | 5775 Morehouse Drive, San Diego, CA 92121 | (858) 658-1874 | shellhammer@ieee.org |
| Monisha Ghosh | Phillips | 345 Scarborough Road, Briarcliff Manor, NY 10510 | (914) 945-6415 | monisha.ghosh@philips.com |

Abstract

This document describes a method to process the captured ATSC DTV signals for the purpose of using the captured data for the purpose of evaluating detection schemes. This is important to ensure all evaluations are compared with respect to a common basis.  In particular, by using this process the signal-to-noise ratio (SNR) is calculated the same way for all simulations.

Some detection schemes may require knowledge of the pilot frequency. A separate estimator for the pilot frequency (not covered here) may be used for this purpose.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Revision History

| Rev | Date | Description |
| --- | --- | --- |
| R0 | August 15, 2006 | Initial document, giving a list of steps required to process the RF captures and MATLAB code for performing the processing |

- Steps involved in processing DTV signal files

The following describes a sequence of steps for processing captured DTV signals. The method used for encoding the RF captures (reproduced here for convenience) has been described in [1] as

The recorded DTV channels were sampled at 21.524476 Msamples/sec and down converted to a low central IF frequency of 5.381119 MHz (one fourth the sampling rate). The analog-to-digital conversion of the RF signal used a 10-bit or a 12-bit A/D. Each sample was encoded into a 2-byte word (signed int16 with a two’s complement format). To encode a field ensemble of 25 seconds, 1.05 G Bytes were needed. An 8 MHz bandwidth IF filter was used when capturing the signals.

Steps 1-3 are performed once for each DTV signal.  Steps 4-6 are performed multiple times so as to be able to produce multiple realizations of the noise.

| Step 1: | Read an appropriate number of samples from one of the DTV signal files. |
| --- | --- |
| Step 2: | Filter the signal using a passband filter with a 6 MHz bandwidth with a center frequency of. The filter may be either a Square Root Raised Cosine filter (that is normally used at a DTV transmitter) or a “brick wall” filter. |
| Step 3: | Measure the power in the received signal |
| Step 4: | Generate white noise sampled at 21.524476 MHz and filter it through the same filter used in Step 2. The noise power used is the receiver noise power. |
| Step 5: | Scale the signal power to meet the target SNR. |
| Step 6: | Add the filtered noise and the scaled and filtered signal |

NOTE: The length of signal samples from a DTV capture that is used for estimation of the signal power should be significantly larger to get an accurate estimate of the signal energy. The length of the sample sequence required for the actual detection algorithm might be smaller.

- MATLAB Code

Main signal processing block – takes as input the signal read from the DTV signal file, target SNR and type of filter to by used for filtering (brick wall or square root raised cosine). Produces as output, the signal + noise sequence.

function [signal_plus_noise_at_passband] = standard_block(target_SNR, signal, filter_flag); % filter_flag = 0 => Brick wall filter (approximate brick wall) % filter_falg = 1 => SQRC filter %declaring constants: filter_bandwidth = 6 ; % in Mhz sampling_freq = (684/286)*9; ; % in Mhz filter_center_freq = sampling_freq/4; % in Mhz ThermalNoise = -174; % dBm/Hz NoiseFigure = 11; % dB %Computing the noise power: NoisePSD = ThermalNoise + NoiseFigure; % dBm Bandwidth = sampling_freq * 10^6; %white noise at sampling rate of 21.52 Mhz NoisePower = NoisePSD + TodB(Bandwidth); % in dBm noise_scale_factor = sqrt(FromdB(NoisePower)); % Variance of the noise to be used %Genrating noise with the noise power computed above: generated_noise = noise_scale_factor * randn(1, length(signal)); %filtering the signal and the noise: %Arguments passed to the filter are: (signal to be filtered, passband width %of filter (in Mhz) , center frequency of filter (in Mhz), sampling rate in %Mhz) if (filter_flag == 0) filtered_signal = passband_brickwall(signal, filter_bandwidth, filter_center_freq , sampling_freq ); filtered_noise = passband_brickwall(generated_noise, filter_bandwidth, filter_center_freq , sampling_freq); else filtered_signal = passband_SQRC(signal, filter_bandwidth, filter_center_freq , sampling_freq); filtered_noise = passband_SQRC(generated_noise, filter_bandwidth, filter_center_freq , sampling_freq); end %scaling the signal to meet target_SNR: noise_power_of_filtered_noise = TodB(norm(filtered_noise)^2 /length(filtered_noise)); % in dBm power_in_filtered_signal = TodB(norm(filtered_signal)^2 / length(filtered_signal)); %in dBm required_signal_power = target_SNR + noise_power_of_filtered_noise; %in dBm signal_to_be_scaled_up_by = required_signal_power - power_in_filtered_signal; % in dB scaled_signal = sqrt(FromdB(signal_to_be_scaled_up_by)) * filtered_signal; %scaled signal %Adding filtered noise to scaled, filtered signal: signal_plus_noise_at_passband = scaled_signal + filtered_noise;

MATLAB function implementing a brick wall filter. Takes as input, the signal sequence to be filtered, the center frequency, the width of the filter and the sampling rate. Produces as output, the filtered signal.

function [filter_output] = passband_brickwall(signal, baseband_filter_width, f_center, f_s) signal_fft = fft(signal); fir = 10000; %length of filter impulse response = 2 * fir + 1 f_cutoff = baseband_filter_width / 2; %cutoff frwq of the equivalent LPF f_shift=f_center; % center freq of the passband filter W_c = 2*pi*(f_cutoff)/(f_s); n = [-1*fir: fir]; h_lpf_left = sin( W_c*[-1*fir:-1]) ./ (pi*[-1*fir:-1]); h_lpf = [h_lpf_left W_c/pi fliplr(h_lpf_left)]; h_lpf_passband = 2* (h_lpf.*cos(2*pi*(f_shift)/(f_s) *n)); H = fft(h_lpf_passband, length(signal)); filter_output = ifft(H .* signal_fft);

- MATLAB function ‘TodB’ used in the function ‘standard_block’ above

function PdB = TodB(P) % Converts from Linear scale to dB Scale PdB = 10*log10(P);

- MATLAB function ‘FromdB’ used in the function ‘standard_block’ above

function P = FromdB(PdB) % Converts from Linear scale to dB Scale P = 10^(PdB/10);

- References

- ATSC A/74 Recommended Practice Guideline Document entitled: “ATSC Recommended Practice:  Receiver Performance guidelines”, Sections 4.5.2 & 4.5.3