IEEE P802.22
Wireless RANs

| Initial signal processing of captured DTV signals for evaluation of detection algorithms |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-09-29 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Suhas Mathur | Qualcomm | 5775 Morehouse Drive, San Diego, CA 92121 | (858)-651-4845 | smathur@qualcomm.com |
| Rahul Tandra | Qualcomm | 5775 Morehouse Drive, San Diego, CA 92121 | (858)-845-1970 | rtandra@qualcomm.com |
| Steve Shellhammer | Qualcomm | 5775 Morehouse Drive, San Diego, CA 92121 | (858) 658-1874 | shellhammer@ieee.org |
| Monisha Ghosh | Philips | 345 Scarborough Road, Briarcliff Manor, NY 10510 | (914) 945-6415 | monisha.ghosh@philips.com |

Abstract

This document describes a method to process the captured ATSC DTV signals for the purpose of using the captured data for the purpose of evaluating detection schemes. This is important to ensure all evaluations are compared with respect to a common basis.  In particular, by using this process the signal-to-noise ratio (SNR) is calculated the same way for all simulations.

Some detection schemes may require knowledge of the pilot frequency. A separate estimator for the pilot frequency (not covered here) may be used for this purpose.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Revision History

| Rev | Date | Description |
| --- | --- | --- |
| R0 | August 15, 2006 | Initial document, giving a list of steps required to process the RF captures and MATLAB code for performing the processing |
| R1 | August 23, 2006 | Included MATLAB code for a square root raised cosine filter. Corrected some errors. |
| R2 | August 29, 2006 | Made an edit to step 2 in section 1 regarding the use of only a ‘brick wall’ filter. Added a section on the choice of filter. Edited the MATLAB code to remove provision for the use of a Square Root Raised Cosine filter. Deleted the MATLAB code for a Square Root Raised Cosine filter. |
| R3 | September 6, 2006 | Edited the document such that it is no longer mandatory to use a ‘brick wall ‘ filter. Instead, the signal in the 6 MHz band will be used for measuring signal power and for scaling the signal appropriately to meet the target SNR, receiver noise added and the signal + Noise filtered through a filter of the evaluator’s choice. The filter must, however, meet certain specs regarding the rejection of signal outside the 6 MHz band. The use of a practical filter with bounds on out of band attenuation and ripple is recommended. In section 2, specifications for a practical filter have been proposed (TBR). MATLAB code was edited to reflect these changes. Code for the practical filter to be used has not been included |
| R4 | September 7, 2006 | After a discussion on the conference call it was decided that there was no need for any information about a filter used in the standard processing. The group decided that all that was needed was a standard method of ensuring that the SNR was calculated correctly. Hence we pulled out of the document any information on the filter. This included the MATLAB code. |
| R5 | September 29, 2006 | Added a step to filter the signal plus noise |

- Steps involved in processing DTV signal files

The following describes a sequence of steps for processing captured DTV signals. The method used for encoding the RF captures (reproduced here for convenience) has been described in [1] as

The recorded DTV channels were sampled at 21.524476 Msamples/sec and down converted to a low central IF frequency of 5.381119 MHz (one fourth the sampling rate). The analog-to-digital conversion of the RF signal used a 10-bit or a 12-bit A/D. Each sample was encoded into a 2-byte word (signed int16 with a two’s complement format). To encode a field ensemble of 25 seconds, 1.05 G Bytes were needed. An 8 MHz bandwidth IF filter was used when capturing the signals.

Steps 1-3 are performed once for each DTV signal.  Steps 4-6 are performed multiple times so as to be able to produce multiple realizations of the noise.

| Step 1: | Read an appropriate number of samples from one of the DTV signal files. |
| --- | --- |
| Step 2: | Measure the signal power in the 6 MHz band. This can be done several ways. One can take an FFT of the signal and calculate the signal power in the 6 MHz band by adding up the power in all the frequency bins within the 6 MHz band. Alternatively, one can filter out the signal components outside the 6 MHz bandwidth and then calculate the resulting signal power. |
| Step 3: | Generate white noise sampled at 21.524476 MHz. The noise power used is the receiver noise power. |
| Step 4: | Estimate the noise power in the 6 MHz band. This can be done in several ways. The simplest method is scaling by the bandwidth. The noise in the 6 MHz band is 2.5375 dB less than the total noise power. |
| Step 4: | Scale the signal power so that the SNR within the 6 MHz band is at the specified value. SNR is defined as the signal power in the 6 MHz band divided by the noise power in the 6 MHz band. |
| Step 5: | Add the noise and the scaled signal. |
| Step 6: | Filter the signal plus noise with a bandpass filter centered at the IF frequency with a bandwidth of no greater than 6 MHz. |

NOTE: The length of signal samples from a DTV capture that is used for estimation of the signal power should be significantly larger to get an accurate estimate of the signal power. The length of the sample sequence required for the actual detection algorithm might be smaller.

- References

- ATSC A/74 Recommended Practice Guideline Document entitled: “ATSC Recommended Practice:  Receiver Performance guidelines”, Sections 4.5.2 & 4.5.3