IEEE P802.22
Wireless RANs

| Geolocation with Database Requirement Development |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-08-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Winston Caldwell | FOX | 10201 W. Pico Blvd. Los Angeles, CA 90064 | 310-369-4367 | Winston.caldwell@fox.com |

Abstract

This document shall serve as the repository for development of the 802.22 geolocation requirement that is to be used in conjunction with a database.  The definition for the geolocation cognitive capability is supplied.  Extracts from the 802.22 Requirements Document is supplied to reinforce the system’s dependence upon geolocation for successful operation.  The reasons that the 802.22 system needs to be aware of the positions of all of the devices operating within its network are discussed.  The goals of the Geolocation Tiger Team are presented along with the questions to which the Tiger Team is responsible for finding answers.  The various methods that can be used to determine a device’s positon is listed along with a brief summary of each technique.  Advances in geolocation technology is presented.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

GEOLOCATION w/ database

1.0 Definition

The ability to determine its location and the location of other transmitters, and then select the appropriate operating parameters such as the power and frequency allowed at its location.  Geolocation is a cognitive capability as defined by the FCC.

2.0 Requirements

As stated in the Requirements Document...

The WRAN system SHALL provide the means for the base station operator to assess interference potential by mapping the results of the spectrum occupancy performed by the base station and the CPEs against information on their physical location acquired through registration information or other geolocation mechanisms.

Information will be sent and collated at the base station where mapping of the interference situation will be made based on the sensing information from the CPEs and according to their respective geographic coordinates.

3.0 Needs for Position Awareness

It is necessary to know coordinate information accurately for all devices in the network.

- If a device is located inside or outside of a TV protected contour.

- How far the device is outside of the TV protected contour if the device is outside of the contour.

- Where CPEs are located in relation to one another in order to make a determination on whether you can rely on distributed sensing.

4.0 Goals of the Tiger Team

- Evaluate the proposals on geolocation.

- Produce a Q&A document used to ask questions to and receive answers from the Proposal Team.

- Develop a simulation model for geolocation.

- Answer the following questions:

5.0 Questions to be Answered

- How is device's position determined?

- Location precision?

- Time to acquire?

- Location information receiver sensitivity?

- Is the location data verifiable?

- How often should the data be verified?

- Is the location information correct?

- How does the position information interact with the database?

- How does the database get populated?

6.0 Methods to Determine Position

- GPS

- Galileo (finished 2010?)

- LORAN

- OMEGA

- Celestial navigation

- Professional install

- Registration information (geo-code)

- Triangulation

6.1 GPS

- GPS relies on triangulation from satellites.

- GPS signals contain pseudo-random code, a navigation message, and ephemeris information (for orbit drift from external gravitational forces).

- Distance is measured using the travel time of radio signals.

- Requires very accurate timing.

- If the clocks are off by a thousandth of a second, ranging calculations can be off by almost 200 miles (322 km).

- Errors can be caused by multi-path.

- Governmental intentional degradation ended on May1, 2000.

- Range is measured by multiplying the travel time by the speed of light.

- The location of the four transceivers must be known very accurately.

- The location of only three transceivers must be known if one of the resulting answers is unreasonable and can be rejected.

- Corrections need to be made for delays the signal might experience as it travels through the atmosphere.

6.1.1 Code-Phase GPS

- Code-phase GPS can have between 3 - 6 m of error.

- The difference in sync between transmitter and receiver is equal to the travel time.

- Each transceiver has a unique pseudo random code.

- The pseudo random code has a bit rate of about 1 MHz.

- The carrier frequency has a cycle rate of over 1 GHz (1000 times faster).

6.1.2 Carrier-Phase GPS

- Carrier-phase GPS can have 3 to 4 mm accuracy by measuring the code-phase accurately and determining the cycle that marks the edge of the timing pulse using the carrier-phase.

- Speed of light is approx. 186,000 but varies due to atmospheric conditions.

- Atmospheric-induced errors can be managed by a dual-frequency measurement through comparing the relative speeds of signals of two different frequencies.

6.2 Differential GPS

Differential GPS uses a stationary receiver of a precisely known position that calculates error corrections through a comparison of what the GPS travel time should be to what the GPS travel time actually is.  The stationary receiver transmits the error correction to the other local receivers of unknown position in order to make more accurate position determinations.  The stationary GPS receiver makes error correction calculations for each of the GPS satellites.

There exist many public agencies (i.e., US Coast Guard) that have established reference stations transmitting error corrections.

The FAA uses an additional geosynchronous satellite positioned over the US and approximately 24 reference stations that are scattered over the US and transmit error correction information.  Eventually, the FAA will install additional reference stations at every airport.

6.3 Assisted-GPS

A-GPS can be even more accurate in determining position if there is a cellular network available.

6.4 Hyperbolic Systems

Hyperbolic navigation systems work because the time/phase difference between the two signals from two different systems is constant along a hyperbolic path.

6.4.1 LORAN (LOng RAnge Navigation)

Two-dimensional geographic location is determined by calculating the time difference between a master station and the two secondary stations with which it has paired.  A time difference is calculated by identifying the TD line, a hyperbolic time difference curve, between the master station and one of its paired secondary stations.  The intersection of the TD lines identified by the time differences of the two pairings is the geographic position.  LORAN operates in the 90 – 110 kHz band.  The LORAN system is not considered accurate in inland areas since the low-frequency signals are degraded considerably by interference and propagation issues caused by land features and man-made structures.

6.4.2 Omega

A hyperbolic radio navigation technique that provided world-wide accuracy of four miles using an eight station chain that was scattered across the globe and transmitted very low frequency signals between 10 – 14 kHz.  An Omega receiver could determine its location by receiving signals from three of these stations.  Omega stations utilized guyed masts over 400 m high.  Omega permanently terminated in 1997.

6.4.3 Decca

A hyperbolic radio navigation system utilizing a master station and usually three slave stations, named Red, Green, and Purple, respectively.  Hyperbolic lines of position were determined by comparing the phase difference of the signals between the master and one of its slaves.  The Red, Green, and Purple stations created their own patterns which would be drawn on navigational charts as a set of hyperbolic lines in the appropriate color.  Position is plotted at the intersection of the hyperbola from the different patterns.  Each of the stations in the chain transmit at a different harmonic of the nominal frequency.

6.5 Celestial

Position is determined by measuring angles between celestial objects and the horizon and referring to tables in a nautical almanac.  A measured angle between a celestial object and the horizon is directly related to the celestial object's geographic position on the Earth and the observer.  A circular line of position indicates the points where the celestial object would be observed at a constant angle above the horizon at that instant.  Position is indicated by one of the two intersections of each of the circular lines of position produced using two different celestial objects.

6.6 Distance and Azimuth Beaconing

6.6.1 Distance Measuring Equipment (distance only)

DME sends paired pulses at a specific spacing from the CPE to the base station.  The base station then responds after a fixed delay with paired pulses back to the CPE with the same pulse spacing (to help discriminate from multiple interrogations) but at a different frequency.  The round-trip time is translated into distance.  Standard DME has an error of less than 360 m.

6.6.2 VHF Omni-directional Range (azimuth only)

VOR transmits two signals simultaneously.  One signal is constant and omni-directional as a reference phase and the other is directional and is rotated about the station.  The directional signal is constantly varied in phase through each rotation in a 0.03 second system cycle.  The two signals are only exactly in phase once during each rotation – when the directional signal is aligned to magnetic North.  The CPE receives both signals, examines the phase angle difference of the signals, and interprets the result as a radial from the station. In the US, VOR operates between 108 – 117.95 MHz.  VOR has less that 0.35º of error.  VOR also transmits a Morse code aural identifier at about 10 second intervals.

6.6.3 Tactical Air Navigation

TACAN is a navigation system used by the military that incorporates a DME system and a more accurate VOR system in order to determine position.  The range finding system is more accurate because its system supplies two waveforms at 15 Hz and 135 Hz as reference waveforms to make phase comparisons.  Non-radiating parasitic elements spin about the main TACAN radiating element at 900 rpm.  A single interior non-radiating element creates the 15 Hz amplitude modulated signal while a group of nine exterior non-radiating elements create the 135 Hz amplitude modulated signal.  A marker signal is transmitted every time the main lobe passes due magnetic East, misleadingly called the “N” marker.  Additional marker signals are transmitted after every 40º of rotation.

6.6.4 Microwave Landing System

MLS signals are transmitted on a single frequency through time sharing on one of two hundred  channels available between 5031 and 5090.6 MHz.  MLS transmits its signals by a narrow beam which sweeps across the coverage area at a fixed scan rate.  Azimuth and elevation can be calculated by measuring the time interval between sweeps.  MLS also utilizes DME for distance with an error of less than 30 m.

7.0 Software GPS

S-GPS allows for improvements in accuracy, speed, and consistency when determining position.  S-GPS removes the need of a special-purpose digital processor to extract the GPS signals, measure their timing, and calculate position.  Instead, all of these are performed in software while using the general-purpose processor.  S-GPS can have an error of less than 3 m.  Time for the first satellite fix can be less than 30 sec.  A hot start can be less than 4 sec.  S-GPS can incorporate A-GPS and can switch between standard GPS and A-GPS, if A-GPS is not available.

8.0 GPS Receivers

GPS receivers can provide for -185 dBW sensitivity (assisted, -174 dBW non-assisted) with 4.7 dB cascaded noise figure.  GPS receivers can tolerate -90 dBm co-channel interferer and 13 dBm interferer in the adjacent channel.