IEEE P802.22
Wireless RANs

| Interference-Based Bandwidth Constraints for Proposed IEEE 802.22 TG1 Transmitters |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Stephen Kuffner | Motorola | Schaumburg, IL, USA | +1-847-538-4158 | Stephen.Kuffner@ Motorola.com |

Abstract

This analysis determines the maximum allowed beacon bandwidth under the assumption that the beacon needs to be detected only if the low power licensed system is experiencing interference from the WRAN.  The maximum allowed bandwidth is calculated for interference from WRAN BS and CPEs with certain antenna height, fading, and EIRP assumptions.  Changes in this revision include higher beacon transmit EIRP to match expected unlicensed rules, higher beacon transmitter antenna gains for existing Part 74.861 transmit powers, and reduced WRAN EIRP for grouped subcarriers.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

### IEEE 802.22 TG1 Interference Scenario

Figure 1 shows an interference scenario used to determine the maximum bandwidth for the TG1 beacon transmitter.  The analysis is based on the assumption that, due to the proximity of the beacon transmitter and the wireless microphone receiver, the path loss PL between the WRAN and the microphone deployment will be roughly the same except for an impairment Δ that can be due to a difference in antenna height and an impairment F↓ due to multipath down-fading (note the WRAN interference into the microphone receiver could also be faded; an upfade F↑ would be the worst case here).  If the PL is such that the WRAN can interfere with the wireless microphone receiver, then the beacon bandwidth as defined by the analysis should be sufficient to provide detection of the beacon signal.  If the PL is high enough that the WRAN will not interfere with the microphone receiver, then the fact that the beacon transmitter is not detectable is acceptable since there would be no harmful interference.

- WRAN interference scenario:  BS into wireless mike receiver.

### WRAN Interference into Microphone Rx

The following equation describes the link between the WRAN transmission and the wireless microphone receiver:

10logPwBmBwunderbracealignlfraction of WRAN power inmicrophone Rx BW Bm−PL−Δ+F↑+Gm,rx<−174+10logBm+NFmmicrophone noise+INmax⇓PL>10logPwBmBw−Δ+174−10logBm−NFm−INmax+Gm,rx+F↑	(1)

I / Nmax is the maximum tolerable interference to noise ratio at the microphone receiver.  For 1 dB sensitivity degradation, this would be -5.9 dB.  However, for 20 dB D/U at -95 dBm as requested by Shure in their FCC Comments, interference at -115 dBm corresponds to the noise power in a 200 kHz microphone receiver BW with 6 dB NF.  Thus, the I / N max is more like 0 dB, which would result in a 3 dB sensitivity degradation.  Both I / N assumptions will be considered in the following analysis.  Pw is nominally 4 W (including Tx antenna gain; if higher power WRAN transmitters are allowed, this number will of course increase), and Bw is ~5.6 MHz for the current (Draft 0.1) modulation definition (1680 used subcarriers) and 6 MHz channel bandwidth (US).  Gm,rx is the microphone rack receive antenna gain.  F ↑ is the multipath upfade of the WRAN signal into the microphone receiver, and a value of 6 dB will be assumed here.

### Beacon Signal at WRAN Rx

The following equation describes the link between the beacon transmitter and the WRAN sensing receiver.

10logPb+Gb,tx−PL−F↓+Gw,sens>−174+10logBb+NFwWRAN noise+EsNo,min⇓10logPb+Gb,tx+Gw,sens+174−10logBb−NFw−EsNo,min−F↓>PL	(2)

Gw,sens is the WRAN sensing antenna gain and Gb,tx is the beacon transmitter antenna gain.  Es / No,min is the minimum tolerable faded channel Es /No that the WRAN receiver should demodulate at the desired packet error rate (F↓ is the downfade margin; this will be absorbed into the faded Es / No,min using packet error rate curves generated with a fading channel model).

Beacon Packet Error Rates

Beacon packet error rates of 1% and 10% will be considered in the analysis.  The Es /No,min for 1% PER is 31 dB, while for 10% PER it is 21 dB [1], [2].  These values will be used in Eq. (2).

Antenna Heights

The microphone receive antenna height (radiating center above ground level, RCAGL) will be fixed at hm = 1.5 m, while the beacon transmit RCAGL hb will be either 3 m (e.g. vehicle mounted) or 10 m (on an extended electronic news gathering truck mast).  The difference in propagation due to clutter height (hc = 10 m assumed) needs to be considered.  Propagation model P.1546 Eq. (23b) will be used to compute the path loss difference Δ.  The WRAN to microphone receiver link experiences an additional attenuation beyond the nominal P.1546 curves of

[3.2 + 6.2 log (f )] log (hm/hc),	(3a)

while the beacon transmitter experiences additional attenuation of

[3.2 + 6.2 log (f )] log (hb/hc).	(3b)

The difference in path loss between the beacon and WRAN and the microphone and WRAN is the difference of Eqs. (3b) and (3a),

Δ = [3.2 + 6.2 log (f )] log (hb/hm),	(3)

which is 16.8 dB for UHF/600 MHz and 14.4 dB for VHF/200 MHz for 10 m beacon height, and Δ = 6.1 dB or 5.3 dB for UHF and VHF respectively, at 3 m beacon height.

Beacon Bandwidth Determination

Substituting Eq. (1) into Eq. (2), the beacon to WRAN bandwidth ratio can be related to the beacon to WRAN power ratio;

10logPbPwBwBb+Δ+Gb,tx−Gm,rx−F↑+Gw,sens−NFw+NFm−EsNo,min+INmax>0⇓10logPbPw+Δ+Gb,tx−Gm,rx−F↑+Gw,sens−NFw+NFm−EsNo,min+INmax>10logBbBw.	(4)

RCAGLb = 10 m, 1% Beacon PER, I / N max = 0 dB

Substituting some parameter values into Eq. (4), assuming Pb = {250 mW for UHF, 50 mW for VHF}, Pw = 4 W, Bw = 5.6 MHz, Gb,tx = {7 dBi  UHF, 5.8 dBi VHF}, Gxm,rx = {0 dBi UHF, -6.5 dBi VHF}, F ↑ = 6 dB, Gw,sens = 0 dBi, ΔUHF = 16.8 dB, ΔVHF = 14.4 dB, NFw = 11 dB, NFm = 6 dB, Es /No,min = 31 dB and 3 dB desense of the microphone receiver (I / N max = 0 dB),

10log0.25 W4 W−12+16.8+7−0−6+0−11+6−31+0>10logBb5.6 MHz for UHF,10log0.05 W4 W−19+14.4+5.8−(−6.5)−6+0−11+6−31+0>10logBb5.6 MHz for VHF.	(5)

This gives Bb < 5.3 kHz for the UHF WRAN BS to microphone deployment link, and Bb < 2.07 kHz for VHF.  Alternatively, the Es / N o,min (that is, the minimum value of Es /No that the demodulator would have to tolerate at the given PER) margin/shortfall can be calculated for the existing symbol rate of 9.609 kHz:

10log0.25 W4 W−12+16.8+7−0−6+0−11+6−10log9609 Hz5.6 MHz+0>EsNo,min for UHF,10log0.05 W4 W−19+14.4+5.8−(−6.5)−6+0−11+6−10log9609 Hz5.6 MHz+0>EsNo,min for VHF.	(6)

For UHF, Es / N o,min < 28.4 dB (a shortfall of 2.6 dB below the 31 dB for 1% PER) while for VHF, Es / N o,min < 24.3 dB (a shortfall of 6.7 dB).  Again, this analysis assumes the WRAN power is spread uniformly over the 5.6 MHz modulation bandwidth.  For VHF, a link margin enhancement of 6.7 dB (e.g., a coding gain of 6.7 dB) at 1% PER could allow the use of the existing symbol rate.

Proceeding similarly for the other combinations of parameters, the link margin results for the existing 9609 Hz symbol rate are summarized in Table I and Table II.  This revision includes a different beacon transmitter constraint, in that Part 74.861 allows 250 mW transmit power but doesn’t limit antenna gain; a value of 7 dBi is considered reasonable for UHF and 5.8 dBi for VHF.  Note that the UHF link margins can be improved by 8 dB by using the WRAN link antenna as the beacon packet reception antenna, nominally assumed as 10 dBd minus 4 dB cable losses, and the VHF link margins can be improved by 6 dB using 6 dBd minus 2 dB cable losses [5].

- Link margins/shortfalls for WRAN BS detection of UHF TG1 beacons.  Note the (negative) shortfall values would be the required (positive) coding gains for the case of the existing 9609 Hz symbol rate.  Positive margins are shown in green, shortfalls in red.  The shown link margins can be improved by up to 8 dB using the WRAN link antenna as the beacon packet reception antenna.

- Link margins/shortfalls for WRAN BS detection of VHF TG1 beacons.  The shown link margins can be improved by up to 6 dB using the WRAN link antenna as the beacon packet reception antenna.

### CPE Interference into a Microphone Rx

The interference from a WRAN BS into a microphone receiver is of concern when there is no other sensing node, such as a CPE, in the vicinity.  This could happen in a sparsely populated area or just an area where there are no WRAN subscribers (e.g. there is DSL or cable in a neighborhood that could otherwise be covered by WRAN operation).  In some propagation conditions the wireless microphone might be outside of the range of interference from a base station, but still within the interference range of the CPE’s emissions.  The CPE transmit EIRP is still assumed to be 4 W, but since the WRAN system is OFDMA, the CPE could be transmitting its full power over fewer subcarriers, presumably as little as a two subchannels, about 56 subcarriers.  This comes from the following analysis.  Assume a 5 bps/Hz spectral efficiency for 64-QAM rate 5/6, which corresponds to 16.74 kbps per subcarrier, or 8.37 kbps assuming 50% TDD.  The number of subcarriers required to achieve 384 kbps uplink is 46, so with pilots assume 56 subcarriers or two subchannels.

Distributed Subcarriers

Thus the WRAN CPE could have 4 W distributed over 56 subcarriers or 71.4 mW/sc.  Assuming the subcarriers are randomly distributed, the probability that a subcarrier will fall within the microphone receiver 200 kHz bandwidth is 200 kHz/5.6 MHz = 3.57%.  Assuming a binomial distribution of subcarriers falling into the microphone receiver BW, the expected number of subcarriers per microphone receiver BW is 56 x .0357 = 2, or 142.8 mW (21.5 dBm).  This is the same power per 200 kHz as for the WRAN BS (4W x 200 kHz/5.6 MHz = 142.8 mW), so this case should result in the same beacon symbol rates and link margins as for the BS (see Table I and Table II).  With this binomial distribution, there’s a 95% confidence that there would be no more than four subcarriers per 200 kHz bandwidth (285.7 mW, 24.5 dBm).  The 3 dB higher interference power due to the extra two subcarriers would reduce the symbol rates or link margins by 3 dB.

- UHF beacon symbol rates and link margins for 95% confidence with distributed CPE subcarriers.  The values for 50% confidence are the same as Table I.  Note these link margins could improve by 8 dB by using the WRAN link antenna as the beacon reception antenna.

- VHF beacon symbol rates and link margins for 95% confidence with distributed CPE subcarriers.  The values for 50% confidence are the same as Table II.  Note these link margins could improve by 6 dB by using the WRAN link antenna as the beacon reception antenna.

Grouped or Contiguous Subcarriers

An even worse case to consider is the contiguous grouping of subcarriers, an optional mode in IEEE 802.22.  A related degradation could result if upstream subcarrier tiling is used, depending on the number of subcarriers per tile and whether the tiles are distributed across the TV channel.  For the contiguous grouping case, all 56 subcarriers of a subchannel could be grouped together, concentrating all 4 W into 187.5 kHz.  Recent motions in IEEE 802.22 have proposed limiting the EIRP using grouped subcarriers to 400 mW EIRP.  This new value will be used for the computations of link margin.

10log0.25 W0.4 W−2.04+16.8+7−0−6+0−11+6−31+0>10logBb200 kHz for UHF,10log0.05 W0.4 W−9.03+14.4+5.8−(−6.5)−6+0−11+6−31+0>10logBb200 kHz for VHF.	(8)

This gives Bb < 1.89 kHz for UHF, and Bb < 738 Hz for VHF.

Again, link margin shortfalls can be calculated.  See Table V and Table VI.

- UHF symbol rates with link margin shortfalls for WRAN CPE detection of TG1 beacons using contiguous subcarriers.  Note these link margins could improve by 8 dB by using the WRAN link antenna as the beacon reception antenna.

- VHF symbol rates with link margin shortfalls for WRAN CPE detection of TG1 beacons using contiguous subcarriers.  Note these link margins could improve by 6 dB by using the WRAN link antenna as the beacon reception antenna.

### Conclusion

An analysis has been performed to determine the maximum allowed beacon transmit bandwidth assuming that the beacon only has to be detectable if the microphone receiver is experiencing harmful interference.  An alternative perspective determines the amount of link margin/shortfall given the existing symbol rate of 9.609 kHz.  The analysis considered two conditions each for beacon antenna RCAGL (3 m and 10 m), beacon PER (1% and 10%), and microphone receiver desense (1 dB and 3 dB) for a total of 8 combinations per band (24 total considering two bands plus the proposed higher power VHF condition).  The analysis is pessimistic in that it considers the beacon signal to be down-faded while the interfering signal from the WRAN into the microphone receiver is assumed to be up-faded.

The only combination that consistently satisfied the link margin at UHF and VHF was for the conditions of 10 m beacon RCAGL, 10% beacon PER, and 3 dB microphone desense.  VHF was only able to satisfy a positive link margin for all cases if higher beacon transmit EIRPs were assumed.  Using the WRAN link antenna as the beacon packet reception antenna can further improve these link margins by 6 dB and 8 dB, respectively, for VHF and UHF.  Lastly, using FEC on the beacon packet can also improve the link margin by a comparable amount.  See [6].

References:

[1]	Document IEEE 802.22-07/0139r0, “Packet Error Rate Performance of the 802.22.1 Beacon with DQPSK Modulation in Multipath Profile A with Rayleigh Fading on Each Path,” D. Mazzarese, Mar. 2007.

[2]	Document IEEE 802.22-05/0055r7, “WRAN Channel Modeling,” E. Sofer, G. Chouinard, Sept. 2005.

[3]	Document IEEE 802.22-06/0171r0, “Channel Simulations for 802.22.1 Proposal,” P. Gorday, Sept. 2006.

[4]	Document IEEE 802.22-06/0234r4, “Complex Beacon Simulations,” David Mazzarese et al, Jan. 2007.

[5]	FCC OET Bulletin No. 69, “Longley-Rice Methodology for Evaluating TV Coverage and Interference,” February 2004.

[6]	Document IEEE 802.22-07/0148r0, “802.22 TG1 Link Margin Calculator,” S. Kuffner, March 2007.

It is assumed that a regulatory exception will be required for the beacon for VHF since presently a transmit power of only 50 mW is allowed.  This is a further 7 dB disadvantage compared to the UHF beacons, while the WRANs are allowed the same transmit power in both UHF and VHF.  Nonetheless the numbers will also be determined for VHF with 50 mW.

Note that the 71.4 mW/sc value used here would exceed the Part 15.247(e) unlicensed EIRP spectral density limit of 25 mW/3 kHz by a factor of 2.5x (4 dB).  This rule has not yet been applied to the TV whitespace.  With this rule, 4 W could be concentrated in a bandwidth of 480 kHz minimum, but Part 15.247(a)(2) specifies that the minimum 6 dB bandwidth for digital modulations should be at least 500 kHz.  It’s unclear how the modulation bandwidth of OFDMA is defined, so the per 3 kHz density may for distributed subcarriers.