IEEE P802.22
Wireless RANs

| MAC Management Messages for Efficient Sensing |  |  |  |  |
| --- | --- | --- | --- | --- |
| Last Revised - Date: 2007-01-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Vincent K. N. Lau | HKUST | Hong Kong, China | 852-2358-7066 | eeknlau@ece.ust.hk |
| Roger S. Cheng | HKUST | Hong Kong, China | 852-2358-7072 | eecheng@ece.ust.hk |
| Ross D. Murch | HKUST | Hong Kong, China | 852-2358-7044 | eermurch@ece.ust.hk |
| Wai Ho Mow | HKUST | Hong Kong, China | 852-2358-7070 | eewhmow@ece.ust.hk |
| Khaled Ben Letaief | HKUST | Hong Kong, China | 852-2358-7064 | eekhaled@ece.ust.hk |
| Linjun Lu | Huawei Technologies | Shenzhen, China | 0086-755-28973119 | lvlinjun@huawei.com |
| Soo-Young Chang | Huawei Technologies | Davis, CA, U.S. | 1-916 278 6568 | sychang@ecs.csus.edu |
| Jianwei Zhang | Huawei Technologies | Shenzhen, China | 86-21-68644808 | zhangjianwei@huawei.com |
| Lai Qian | Huawei Technologies | Shenzhen, China | 86-755-28973118 | qlai@huawei.com |
| Jianhuan Wen | Huawei Technologies | Shenzhen, China | 86-755-28973121 | wenjh@huawei.com |

Abstract

This document provides a minor but key modification to the existing MAC management messages for sensing, so as to avoid a dramatic increase in overhead when discontiguous channels are sensed.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Document Policy and Procedures: The contributor is familiar with the IEEE 802 Document Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of document(s), including document applications, provided the IEEE receives assurance from the document holder or applicant with respect to documents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of document information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if documented technology (or technology under document application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Document Committee Administrator at <patcom@ieee.org>.

| Co-Author(s): |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | Company | Address | Phone | email |
| Edward K. S. Au | HKUST | Hong Kong, China | 852-2358-7086 | eeedward@ece.ust.hk |
| Peter W. C. Chan | HKUST | Hong Kong, China | 852-2358-7086 | peter@ece.ust.hk |
| Ray R. Wang | HKUST | Hong Kong, China | 852-2358-7086 | wray@ece.ust.hk |
| Tianyu Wu | HKUST | Hong Kong, China | 852-2358-7086 | wuty@ece.ust.hk |
| Zhou Wu | Huawei Technologies | Shenzhen, China | 86-755-28979499 | wuzhou@huawei.com |
| Jun Rong | Huawei Technologies | Shenzhen, China | 86-755-28979499 | rongjun@huawei.com |
| Jian Jiao | Huawei Technologies | Beijing, China | 86-10-82882751 | jiao_jian@huawei.com |
| Mingwei Jie | Huawei Technologies | Shenzhen, China | 86-755-28972660 | jiemingwei@hauwei.com |

# Reference

[1]	IEEE P802.22™/D0.1 Draft Standard for Wireless Regional Area Networks Part 22: Cognitive Wireless RAN Medium Access Control (MAC) and Physical Layer (PHY) specifications: Policies and procedures for operation in the TV Bands.

# MAC Management Message for Efficient Sensing

## Introduction

In WRAN system, there are some channels that are more / less important to sense than the others. Due to the limited resource for sensing, it is desirable to have an intelligent and flexible scheme to allocate more resource to those important channels. To facilitate this, the BS needs to specify discontiguous channels for CPEs to sense. Unfortunately, the MAC management messages in the current draft [1] do not address discontiguous channels, which will impose a heavy overhead punishment on the systems employing the above-mentioned flexible sensing scheme. In this contribution, we propose a minor but key modification to the existing MAC management messages for sensing, so as to avoid a dramatic increase in overhead when discontiguous channels are sensed.

## Issues in Current Draft

We first explain by example why the specification of discontiguous channels is needed in ordered to maximize the capacity of distributed sensing in WRAN system, followed by addressing the issue in the current draft.

Consider the following example as shown in Figure 1.

Figure 1. An illustrative example showing why the specification of discontiguous channels is needed.

There are 7 contiguous channels in which TV signals / strong power incumbents are present in Channels 3, 4 and 5. If TV signals are present in these channels, the BS may spend very little or no resource to sense these channels since these incumbents are known a priori to be present (from the database or site survey). It is also possible that the incumbents are not fixed TV incumbents but only strong incumbent signals which may leave after some time (e.g. a television station's remote-news van, which is dispatched to somewhere in the WRAN cell and sends a signal back to the station). In this case, the BS does not have a priori information of its presence from the database. However, due to the strong signal of the incumbent, only few CPEs are sufficient to detect its presence very reliably. Most of the CPEs can save the sensing period to sense the other channels whose statuses are more uncertain. In this case, BS needs to specify discontiguous channels in different MAC/PHY stacks.

Referring to Section 6.8.22 of the current specification [1], a lot of overhead is needed to specify discontinuous to sense. In particular, one BLM-REQ message (c.f. Table 145 in [1]) can only facilitate one continuous set of channels. Therefore, N BLM-REQ messages with almost identical contents are required to specify N discontiguous channel intervals, which add a lot of overheads to the system.

It is noteworthy that an example scenario of discontiguous channels can be found in Section 6.21.1.5 of [1]. In particular, in response to out-of-band measurements previously requested by the BS, every CPE need to keep track of backup channels, which are completely disjoint from the current operational channels.

## Proposed Text Changes

In the following, the texts in red are to be deleted while those in blue are to be added to the refined BLM-REQ message.

Refined Table 145 [1]:

| Syntax | Size | Notes |
| --- | --- | --- |
| BLM-REQ_Message_Format() { |  |  |
| Management Message Type = 39 | 8 bits |  |
| Transaction ID | 16 bits | Shall be set to a nonzero value chosen by the BS sending the measurement request to identify the request/report transaction. |
| Starting Channel Number | 8 bits |  |
| Number of Channels | 8 bits |  |
| Interval-basis Channel List | Variable | Table xxx (in this document). |
| Confirmation Needed | 1 bit | Indicates whether or not the CPE is required by the BS to confirm, with a BLM-RSP message, the receipt of this message. 0 = No confirmation needed (default) 1 = Confirmation needed |
| Number of Single Measurement Requests | 3 bits | The number of single measurement requests contained in this message |
| Single Measurement Requests | Variable | A series of single measurement requests. See Table 146 [1]. |
| } |  |  |

Referring to the refined Table 145, the fields “Starting Channel Number” and “Number of Channels” are replaced by a new syntax “Interval-basis Channel List”, which can be used in identifying the channels that are contiguous or discontiguous. The table below shows the message format of the proposed Interval-basis Channel List. We can see that the proposed structure adds only one bit to the current structure, but it can save a lot of overheads when discontiguous channels are sensed.

Table xxx Interval-basis Channel List

| Syntax | Size | Notes |
| --- | --- | --- |
| Interval_basis_Channel_List_Format() { |  |  |
| Starting Channel Number | 8 bits | The index of the starting channel |
| Number of Channels | 8 bits | The number of channels in the current interval |
| Linker | 1 bit | 1: The next 17 bits follow the interval-basis structure 0: The Channel List structure is terminated here |
| } |  |  |

From the table above, it can be observed that the major difference is the presence of the one-bit linker, which can be used for indicating whether discontiguous channels are present. For all of the remaining syntaxes in the table, they remain unchanged and therefore it is fully compatible with all the other mandatory and optional features in the current draft.

## Conclusion

Sensing discontiguous channels is needed when the BS wants to distribute more resource to sense particular channels, instead of asking the CPEs to sense all channels with uniform resource. We have stated an example when some channels need more resource to sense than the others and an example scenario of discontiguous channels. By using the proposed Interval-basis Channel List structure for discontiguous channels, the BS does not need to send multiple BLM-REQ messages to convey virtually identical commands. If there is no discontiguous channel, there is only one bit overhead in which we believe that this one bit investment is worthwhile, since it saves the WRAN system from the risk of wasting resource when sensing discontiguous channels.