IEEE P802.22
Wireless RANs

| Spectrum Contention Algorithm |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-03 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Wendong Hu | STMicroelectronics | 1060 East Brokaw Road, San Jose, CA 95131 | 1-408-467-8410 | Wendong.hu@st.com |
| Liwen Chu | STMicroelectronics | 1060 East Brokaw Road, San Jose, CA 95131 |  | Liwen.chu@st.com |
| George Vlantis | STMicroelectronics | 1060 East Brokaw Road, San Jose, CA 95131 |  | George.vlantis@st.com |

Abstract

This document describes the spectrum contention algorithm for 802.22 WRAN self coexistence.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- Spectrum Contention Algorithm

Figure 1 illustrates the On-Demand Spectrum Contention (ODSC) algorithm, which is an optional feature in the MAC.

Starting with system initializations, a ready-to-operate WRAN system first performs channel evaluation and channel selection to detect and select incumbent-free channels. Operations of channel evaluation include channel sensing, measurement evaluations, measurement reporting, report processing, which are described in detail throughout Section Error: Reference source not found.

- —On-demand Spectrum Contention Algorithm

Then the WRAN system verifies whether non-exclusive sharing of the selected channels is feasible. Non-exclusive sharing of a selected channel is to share the selected channel through TPC (Transmit Power Control) such that WRAN systems sharing the same channel do not cause harmful interference to one another. Non-exclusive spectrum sharing method is feasible as long as the maximum achievable signal-to-interference-ratio (SIR) on the selected channel is higher than the required SIR of the WRAN for the supported services.

If non-exclusive sharing of the selected channels is feasible, the WRAN system then makes a channel switch decision and schedules data transmissions on the selected channels with appropriate TPC settings. The channel switch announcement and collision avoidance (specified in Error: Reference source not found and Error: Reference source not found) are performed on the selected channels.

On the other hand if non-exclusive sharing is not feasible, exclusive sharing of the selected channels is performed . When a channel is shared exclusively by multiple WRAN systems, only one WRAN system can operate on the channel at any time instance. The exclusive channel sharing is performed through spectrum contentions which are facilitated by inter-system coordination. By using CBP and inter base station communications, an 802.22 system can effectively communicate with other 802.22 systems for coordination of spectrum sharing of the selected incumbent-free channel without interrupting normal data transmissions.

If the selected incumbent-free channel is not occupied by another 802.22 system (or a licensed exempt system that the WRAN can communicate with), the WRAN ceases the effort of spectrum sharing on the selected channel. Otherwise, the WRAN system initiates the spectrum contention procedure to contend for the selected channel with the WRAN system that is occupying the channel. A WRAN would have to initialize the contention process with multiple neighboring systems if they operate on the same selected channel.

The basic components of the spectrum contention procedure include:

- Contention request

- Contention resolution and response

- Contention acknowledgment

The details of the spectrum contention procedure are specified in section 1.1.

After the spectrum contention procedure completes, the WRAN system winning the contention schedules data transmissions on the contended channel.  If the WRAN system winning the contention is not the current occupier of the contended channel (i.e. the WRAN system occupying the contended channel lost), channel switching to the contended channel is performed at the time agreed by both contending systems (more details later). If channel switching is to be performed, channel switch announcement and collision avoidance process take place before data transmissions are performed on the contended channels. The WRAN system that fails the contention shall not perform data transmissions on the contended channels. If the WRAN system losing the contention is the current occupier of the contended channel, the channel is released by such system at the time agreed by both contending systems.

In the data transmissions state of a WRAN system, as shown in Figure 1, one of two types of demands initiates an iteration of the spectrum sharing process:

- Internal demands

- External demands

Internal demands are generated by a WRAN system itself and are the consequences of the channel condition and workload condition analysis performed by the WRAN system. When the current channel condition (channel occupancy and channel quality) is not able to support the QoS of the given admitted service workloads, the WRAN generates an internal demand which initiates an iteration of the full spectrum sharing process so that a better channel or more channels can be acquired to satisfy the QoS requirements of the given workloads.

External demands are channel contention requests received from other WRAN systems. When an external demand is received, a WRAN system initiates spectrum contention iteration.

  - Spectrum Contention Procedure

In order to support on-demand spectrum contention, three messages are defined: spectrum contention request (SC_REQ), spectrum contention reply (SC_REP), spectrum contention acknowledgement (SC_ACK). The formats of these messages can be found in section 1.2. These messages are carried by CBP or inter base station communications.

Contention source is defined as the WRAN that requests to contend for the selected channel. Contention destination is defined as the WRAN that occupies the requested channel and receives the contention request.

Figure 2 illustrates the spectrum contention procedure.

- Spectrum Contention Procedure

After selecting the contended channel, the contention source sends SC_REQ to all its neighboring WRAN (contention destinations) that are operating on the requested channel. Whenever a new spectrum request is proposed, the sequence number of spectrum request is incremented by 1 (module 256). To guarantee that the contention destinations receive the spectrum contention request, the contention source may sends out SC_REQ more than one times. The repeated SC_REQs shall have the same sequence number.

If the contention source receives any SC_REP with REJECT indication from the contention destination, the contention source shall cancel the request by sending message SC_ACK with GIVEUP indication so as to allow the contention destinations continue to use the original working channels.

After receiving SC_REP messages with SUCCESS indication from all the contention destinations, the contention source shall acknowledge the SC_REPs by sending messages SC_ACK with OCCUPY indication notifying the contention destinations to release the original working channels at the given time as indicated in the SC_ACK messages. The message SC_ACK with OCCUPY indication also includes the start time of the quiet period. The contention source shall use the requested channel as the working channel at the time indicated in the SC_ACK message. To guarantee that the contention destinations receive the SC_ACK, the contention source may sends out SC_ACKs more than one times. The repeated SC_ACKs shall have the same sequence number. The SC_ACK messages select the same sequence number as the related SC_REQ. The contention source shall silently discard the repeatedly received SC_REPs.

After receiving a new SC_REQ, each contention destination runs a contention resolution algorithm to decide who will win the contention. The following figure gives the contention resolution algorithm.

The contention destination shall silently discard the repeatedly received SC_REQs.

After running the contention resolution algorithm, a contention destination replies with SC_REP to the contention source to indicate if the request is accepted. If the request is rejected, the reason is given in the reason field in the SC_REP message. If the SC_REQ is accepted, the contention destinations shall reject any other SC_REQ from any other contention sources until a SC_ACK is received. The SC_REP sent out with SUCCESS indication shall indicate the contention destination’s quiet period start time (TTQP). To guarantee the contention source receive the SC_REP, the contention destination may sends out SC_REPs more than one times. The repeated SC_REPs shall have the same sequence number.

After the contention destination sends out SC_REP, the contention destination shall wait for SC_ACK. If the contention destination does not receive any SC_ACK at the given time (timer Txx times out), the contention destination continues to use the original working channel.  If SC_ACK with GIVEUP indication is received, the contention destination continues to use the original working channel. If SC_ACK with OCCUPY indication is received, the contention destination shall release the original working channels at the given time being indicated by the SC_ACK message. The contention destination shall silently discard the repeatedly received SC_ACKs.

- Contention Resolution Algorithm

  - Spectrum Contention Messages

    - Spectrum Contention Request (SC_REQ)

This message (Table xxx) is sent by the contention source in order to contend the working channels with the contention destinations. This would be transmitted by the contention source once it has data transmission requirement and occupies no channel. If the current working channel can not satisfies the QOS requirement, the contention source can also send out this message.

Table xxx—SC_REQ

| Syntax | Size | Notes |
| --- | --- | --- |
| SC_REQ_Message_format() { |  |  |
| Management Message Type=XX | 8bits |  |
| Source Id | 48bits | The MAC address of the contention source |
| Destination Id | 48bits | The MAC address of the contention destination |
| Sequence number | 8bits | Incremented by 1 by the source whenever any of the following three fields change. The contention destinations shall discard the repeated SC_REQ being receveid |
| Spectrum contention number (SCN) | 32bits | A random number to show the priority to contend for the channel |
| Channel number | 8bits | The channel being requested by the contention source |
| Start time | 16bits | Starting from the next frame, the number frames after which the contention source requires to start operation on the requested channel. |
| } |  |  |

    - Spectrum Contention Reply (SC_REP)

This message (Table xxx) is sent by the contention destination in order to tell the contention source if the request is accepted. Typically this would be transmitted by the contention destination once it has receives a SC_REQ and runs the contention resolution algorithm.

Table xxx—SC_REP

| Syntax | Size | Notes |
| --- | --- | --- |
| SC_REP_Message_format() { |  |  |
| Management Message Type=XX | 8bits |  |
| Source Id | 48bits | Copy from the SC_REQ |
| Destination Id | 48bits | Copy from the SC_REQ |
| Sequence number | 8bits | Copy from the SC_REQ. The contention source shall discard the repeated SC_REP being receveid |
| Channel number | 8bits | The channel requested by the source BS |
| Result indication | 2bits | 00….SUCCESS (accept the request) 01….REJECT (reject the request) 10—11….reserved |
| Reason of Rejection | 6bits | The reason to reject the SC_REQ. This field can only be used when result==01 000000 - the current working period of the contention destination is too short 000001 - the contention destination has smaller spectrum occupation ratio than the contention source 000010 - The remaining time to the quiet period is too short 000011—111111 - reserved |
| Channel Release Time | 16 bits | Starting from the next frame, the number frames after which the channel can be released. |
| Time to quiet period (TTQP) | 16bits | If the contention destination accepts the request, this field indicates the start time of the following quiet period |
| } |  |  |

    - Spectrum Contention Acknowledgment (SC_ACK)

This message (Table xxx) is sent by the contention source in order to notify the contention destinations that the contention source will occupy the working channels or give up the request. Typically the contention source shall notify the contention destinations that it will occupy the channel if it receives SC_REP with a SUCCESS indication from all the contention destinations. Otherwise the contention source shall notify the contention destinations that it will give up the channel request if it receives SC_REP with a REJECT indication from anyone of the contention destinations.

Table xxx—SC_ACK

| Syntax | Size | Notes |
| --- | --- | --- |
| SC_ACK_Message_format() { |  |  |
| Management Message Type=XX | 8bits |  |
| Source Id | 48bits | The MAC address of the contention source |
| Destination Id | 48bits | The MAC address of the contention destination |
| Sequence number | 8bits | Same as the corresponding SC_REQ. The contention destinations shall discard the repeated SC_ACK being receveid |
| Channel number | 8bits | The channel being requested by the contention source |
| Start time | 16bits | Starting from the next frame, the number frames after which the contention source will start operation on the requested channel. |
| Occupation code | 2bits | 00….Occupy the channel 01….Give up the request 10—11….Reserved |
| Reserved | 6bits | The field shall be set to 0 |
| TTQP | 16bits | If the contention source occupies the channel, this field indicates the start time of the following quiet period |
| } |  |  |

References: