IEEE P802.22
Wireless RANs

| Draft Recommended Practice |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-01-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Winston Caldwell | FOX | 10201 W.Pico Blvd Los Angeles, CA 90064 | 310-369-4367 | Winston.caldwell@fox.com |

Abstract

This document will serve as the working document toward the draft 802.22.2 Recommended Practice.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# 802.22 WRAN Reference Model

The Wireless Regional Network standard developed under the P802.22 is aimed at point-to-multipoint wireless systems intended principally to extend broadband access to less populated rural areas where vacant channels in TV broadcast bands are likely to exist in larger quantity.  The use of these TV bands for broadband access has the advantage of providing for better propagation conditions to reach larger distances with reasonable transmission power.

The typical WRAN system operation will include a base station (BS) with an omni-directional or sectoral, vertically polarized antennas at 75 m above average ground level, and a number of Customer Premises Equipment (CPE)s for which all the RF parameters are remote controlled by the base station with their directional transmit and receive antenna and with an omni-directional sensing antenna at 10 m above average ground level.

The WRAN standard was developed to provide broadband access equivalent to the first generation of ADSL and cable modems to the rural population who would otherwise have no service except over telephone lines or satellite.  The standard was developed with the aim of providing a downsteam capacity of 1.5 Mbit/s and an upstream capacity of 384 kbit/s per subscriber with a service reliability of 50% location and 99.9% of the time at the edge of the coverage area.

The spectrum efficiency for the system varies from 1 bit/(s*Hz) for the most robust transmission (QPSK and FEC rate= ½) to 5 bit/(s*Hz) in the case of the closer-in line-of-sight CPEs (64QAM and FEC rate= 5/6), not counting the overhead needed for the system synchronization, TDD transition gaps, quiet periods, etc. which represents some 25% of the transmission capacity.  The lowest efficiency will be used for CPEs at the edge of the contour or in places hard to reach whereas, for easily reacheable terminals, a combination of higher modulation and FEC code will be used along with lower transmit power adjusted according to the ranging process and through Transmit Power Control (TPC).

For a decreasing subscriber density as a function of the distance from the BS, which represents the case for a typical rural town or village and its surrounding areas, it is reckoned that the average spectrum efficiency will be about 3 bit/(s*Hz).  This results in a total capacity of 18 Mbit/s in a 6 MHz channel for a simple omni-directional BS.  Assuming a TDD operation and a 50:1 over-subscription ratio typical of the first generation of ADSL/cable-modem, the number of subscribers that can be sustained by this omni-directional BS is 478 subscribers.

The area that the Wireless Regional Area network (WRAN) operator needs to cover should include enough subscribers to make it economically viable early in the process when the take-up rate is low.  This will define the potential subscriber base and the minimum population density for a sustainable business case.  For example, if the minimum number of subscribers to sustain the WRAN service is 90 and this represents 5% of the potential market in the area, the potential subscriber base would be 1800, which would correspond to a total population in the area of 4,500 people, assuming 2.5 people per household.

The use of a low power BS, as described in section 1.1.1. below, to cover this area would result in a lowest population density that can economically be covered by WRAN of 5 person/km2 because of its 16.8 km reach.  In the regions where the higher power BS, as described in section 1.1.2., can be deployed with its typical reach of 32 km, this lowest population density would reduce to 1.4 person/km2.   The upper limit of the range corresponds to the population density where other technologies start to make economic sense (e.g., ADSL and cable-modem at 60 person/km2).

With time, the number of subscribers in the area will increase and the WRAN operator will manage this increase by sectorizing his coverage area for a potential frequency re-use of 4 times to cover all his potential subscribers.

# Possible Scenarios

# Low Power Base Station WRAN System

Based on the proposal from the FCC NPRM 04-186, it is assumed that the CPEs and the BS will be limited by this 4 W EIRP to protect the incumbent services in the TV bands.  Since the BS tries to serve as many CPEs at the same time as possible in making effective use of the OFDMA multiplex, the BS EIRP will likely be kept close to the 4 W limit almost on a continuous basis.  Because of its lower transmission capacity, the CPE will only need to use a transmit power equivalent to the portion of the 4 W EIRP to secure a balanced RF transmission in both directions between the BS and the CPE.  The only time when the CPE EIRP will come close to the rated maximum is when this CPE is allowed to use the full channel capacity in the upstream direction and is located such that it uses its TPC at full capacity.  This would occur in the case of a CPE at the edge of coverage being given the full channel upstream capacity for, for example, uploading of a video program overnight.

# Higher Power Base Station WRAN System

[Also based on the proposal from the FCC NPRM 04-186 that the EIRP from the unlicensed devices should be limited to 4 W to protect the incumbent services in the TV bands, it is assumed that, since the BS will be professionally installed, a BS would be allowed to use higher transmission power to serve their multiple CPE terminals. (doesn’t make sense to me /Winston/)]  The OFDMA modulation used in the 802.22 standard will allow the CPEs to use only a portion of the carrier multiplex to transmit its data towards the BS.  This portion will depend on the capacity required at that moment.

The standard has defined that the minimum portion that could be used by a CPE is one sub-channel which contains [54 carriers whereas the complete multiplex contains 1728 carriers for a total of 32 subchannels].  This means that in order to establish a communication with a CPE at the edge of the contour, the Base Station would need to modulate the carriers belonging to this communication channel with the most robust parameters such as QPSK and FEC rate= ½ in order to reach as far as possible.  On the return channel, the CPE would modulate the carriers of its sub-channel with the same parameters and transmit at the maximum 4 W EIRP.  Hence, in order to establish a balanced RF link, the BS will need to transmit with a corresponding EIRP, that is 4 W EIRP per 54 carriers.

Since the BS needs to serve as many CPEs as possible, it will try to use all the 1728 carriers of its multiplex at the same time.  The most demanding situation will be when the BS has to transmit 4 W EIRP on all its sub-channels at the same time in the case when it happens to serve 32 CPEs located at the edge of the coverage.  This will amount to a total BS EIRP of 32*4 = 128 W to establish balanced RF transmission channels toward the 32 CPEs allowed to use their prescribed maximum 4 W EIRP on their return link.  The BS will normally operate at lower power since a number of its CPEs will be closer and operate at lower transmit power due to their TPC settings.

This higher BS transmit EIRP will be taken into consideration in the protection of the DTV receivers at the planning stage by extending the keep-out distances for co-channel and adjacent channels from the nearby TV and DTV protected contours to cover this extra 15 dB EIRP, and at the professional installation stage by making sure that the BS is located at 10(15/20)= 5.66 times the nominal 10 m minimum separation between the 4 W CPE and the nearest DTV receiving installation.  The BS antenna will therefore need to be at 56.6 m from the closest DTV receiving installation.  This should not be onerous because it is expected that the BS antenna height is expected to be some 75 m above ground to provide for the necessary reach.  The professional installer therefore needs to consider the reach of the BS as well as the proximity of TV receivers in deciding on the antenna height.  Buying the land over these 60 m around the Base Station would be another simple means of avoiding any potential interference problem.  Note:  This paragraph describes extending the range of interference exception.

This additional 15 dB in EIRP will also need to be considered in setting the sensing thresholds and/or the number of CPEs involved in sensing for detecting the incumbents to make sure that the additional interference range is taken into account.  This is especially true for detecting Part 74 wireless microphones in the extended area around the BS.

# Classes of Base Stations

During the WRAN standard development, it was suggested to define a number of BS classes [Cairns session].  [A first class was proposed for EIRP up to 4 W for which professional installation would be optional].  A second class was for BS with EIRP up to 4 W that would need to be professionally installed.  A third class was for the BS that would allow the CPEs to operate at 4 W EIRP in a balanced transmission as described in section 1.1.2..  This class would clearly need professional installation and special care for extra keep-out distances from TV protected contours for co-channel and adjacent channel operation as well as extended minimal separation distance to the closest TV receiving installation as explained in section 1.1.2..  Special measures will also be needed to make sure that sensing of incumbent operation will be more acute to compensate for the higher power.  Other classes with medium power in the range between 4 W and 128 W that would require professional installation were also proposed.

# Deployment

# System

# Planning of Service

Since the WRAN systems are to provide broadband access services while protecting the incumbent services in the TV bands from interference through geo-location paired with a database containing station information, sensing, and dynamic frequency selection features included in the standard, careful planning of the service will be needed to avoid excessive future service disruption from these DFS features.  More careful planning of the systems will avoid future unexpected situations.  Service continuity and reliability will indeed depend on the quality of this initial planning.  [If the systems are truly ‘unlicensed’, I am not sure that this will really do the job if coexistence among unlicensed systems is not fully taken care of since any other unlicensed system could come in and disturb the well planned WRAN system.  This needs further discussion.]

# Advanced Propagation Model

Although the original FCC propagation curves contained in Part 73 were used to define the TV protected contours and the ITU-R Rec. P1546 propagation model was used in the system studies for the development of the 802.22 standard, more precise propagation models such as TIREM that take into consideration actual terrain data should be used in the deployment planning of WRAN systems.  More precise coverage areas as well as potential interference situations that need to be avoided such as unexpected line-of-sight conditions will be found through precise propagation computer predictions.  These better propagation tools may allow possible system implementation where rough flatland estimates would have declared it impossible.

      - Database

        - Incumbent Station Database

To facilitate the planning of new WRAN systems, a database of incumbent operation in the area will need to be available in an accurate and up-to-date form.  The incumbents should make sure that the information contained in the database is accurate to avoid potential interference resulting from inadequate WRAN system planning.  Since the sensing and DFS features of the WRAN system will ultimately provide the protection at the cost of poorer WRAN service continuity and reliability, the WRAN operators will need to be aware of the accuracy of these incumbent databases.  Since both parties could be affected, it would be appropriate that the accuracy and maintenance of such databases be under government or third party responsibility.

          - Channel Information

All the TV channels occupied in an extended area (how far out?) should be documented [along with the position of the TV transmitter, the antenna directivity, HAAT and ERP (might be redundant with use of polygons)].  Because of the level of power involved, a TV transmitter may affect a WRAN system operation over a radius of [around 400] km and more.  Similar to area frequency manager database for Part 74 or the FCC cdbs database.

          - Polygons

As an alternative to populating the incumbent station database with station operation parameters, it could be populated by TV protected contours in the form of contour polygons represented by the coordinates of all apexes.  These contours will need to be defined with the agreement of the local regulator so that the right considerations such as the TV received signal level, the TV interference level, the Designated Market Area, etc. are taken into account.  This will allow the future WRAN operators to calculate the precise keep-out distances for the co-channel or adjacent channel operation and identify precisely which TV receiving installations can claim protection on which channels from WRAN.

          - Standardized Format

The format of the databases should be harmonized in the various regions so that standardized computer tools used for planning the WRAN systems can be used without special modifications.  Harmonization should also be done across regions where local broadcasting can be affected by the same WRAN systems.

        - WRAN Base Station Database

As a minimum, a registry of the BSs in operation in an area with their coordinates and operating characteristics should be constituted and made publicly available (e.g., on a Web site).

The location and technical parameters of the BS are to be provided for inclusion in a database that will be publicly accessible.

      - Separation Distances

        - DTV Separation

Proposers should be mindful in terms of system architectural considerations that, in the interest of coexistence and avoidance of interference to incumbent TV operations, WRAN BSs will have to be located outside the noise-limited protected contour of TV stations that use the same channel or first adjacent channels.  Proposers should also note that exact required separation distances out from the noise-limited protected contour of TV stations that use the same channel or first adjacent channels may vary in different regulatory domains.

In the case of second adjacent channels and beyond, the WRAN BS will need to be located at some minimum distance from the closest TV receiver.

In the case of a WRAN system operating on a co-channel or first adjacent channels basis (N, N±1), specific keep-out distances will need to be observed based on a simplified flat-land propagation prediction model (ITU-R Rec. P.1546).  The actual values are contained in the Table 1 for DTV and Table 2 for NTSC.  In the cases where more accurate propagation prediction models using terrain database information indicate that smaller keep-out distances than indicated could be assumed, discussions with the local regulator and the broadcaster potentially affected may be undertaken to come to an agreement.

It is also possible to use smaller separation distances than those indicated in the Table as long as the maximum EIRP of the WRAN transmitting device (i.e., maximum value of its TPC range) is scaled accordingly.  This tapering of EIRP toward the TV protected contour can be pre-defined at the time of installation for setting the BS maximum EIRP and can be entirely controlled from the Base Station for the CPEs as part of an additional constraint to the normal ranging and TPC process.  The amount of tapering in dB from the maximum allowed EIRP will be defined as follows from the fraction of the actual distance to the protected contour to the distance indicated in the table:

Tapering = Path loss exponent * 10*log(actual distance/distance in the table)   (dB)

where:		path loss exponent = 3.0   [tbd]

Reference: Doc. 219

Might implement a methodology as opposed to supplying actual values.  We should explain how to compute.  The following is an example based on some assumptions, such as transmitter height.  This comes from WRAN spreadsheet 22-04-0002-15 spreadsheet.

When the WRAN system operates on alternate channels (N±2 and beyond), the BS and the CPEs that are limited to a 4 W maximum EIRP will be allowed to operate within the protected contour of the TV station down to a distance of 10 m from the closest TV receiving installation as indicated in Tables 1 and 2.  For higher EIRP Base Stations, this minimum distance will need to be scaled according to the excess EIRP (√(EIRP/4) ).  The allowed maximum EIRP from the WRAN transmit devices will also need to be scaled according to the EIRP profile as defined in section 2.1.2 to take into account the vulnerabilities of the typical TV receivers on taboo channels.

Table 1:  Distance from the DTV protected contour

| WRAN into DTV | D/U | CPE | 4 W BS | 128 W BS |
| --- | --- | --- | --- | --- |
| Co-channel, N | 23 dB | 3.3 km* | 16.1 km | 32.9 km |
| Adjacent channel, N-1 | -28 dB | 126 m | 485 m | 1.8 km |
| Adjacent channel, N-1 | -26 dB | 147 m | 566 m | 1.8 km |
| N±2 and beyond (see EIRP profile in section zzz) | D/U taboo values and -8 dBm | 10 m from DTV RX | 10 m from DTV RX | 56.6 m from DTV RX |

Note: F(50,10), ITU-R Rec. P.1546, 75 m BS antenna HAAT

* 16 dB CPE antenna front-to-back ratio assumed.

Table 2:  Distance from the NTSC Grade B contour

| WRAN into NTSC | D/U | CPE | 4 W BS | 128 W BS |
| --- | --- | --- | --- | --- |
| Co-channel, N | 4 dB | 1.5 km | 2.8 km | 7.9 km |
| Adjacent channel, N-1 | -14 dB | 44 m | 44 m | 226 m |
| Adjacent channel, N-1 | -17 dB | 31 m | 31 m | 171 m |
| N±2 and beyond (see EIRP profile in section zzz) | D/U taboo values and -8 dBm | 10 m from TV RX | 10 m from TV RX | 56.6 m from TV RX |

Note: F(50,10), ITU-R Rec. P.1546, 75 m BS antenna HAAT.

Table 2:  Distance from the NTSC Grade B contour

We need also to define separation distances for DVB-T, PAL, SECAM, etc. while taking into consideration that rules in other regulatory domains can be different, such as D/U ratios.

It should be realized that, although the keep-out distances for co-channel and adjacent channel operation were calculated from the consideration of interference for the WRAN terminals into DTV or NTSC reception, the interference in the reverse direction will also need to be considered by the WRAN system planner.  Table 3 gives these distances for a 1 dB WRAN receiver desensitization.  As can be seen from the large separation distance, the co-channel interference from the DTV signal into WRAN reception is much more severe than in the WRAN into DTV direction due to the larger power of the TV transmitter.  However, the WRAN system planner may decide to allow his receivers to suffer more than the 1 dB desensitization.  BS and CPEs could then be located closer than the distances given in Table 3.  This system would have to make use of higher operating values for the TPC parameters and/or lower transmission capacity because of the use of more robust modulation.  Nonetheless, the system planner will still have to meet the minimum distances specified in Table 1.

Table 3:  Distance for the DTV transmit station

| DTV into WRAN | Desens. | CPE | BS |
| --- | --- | --- | --- |
| Co-channel, N | 1 dB | 406 km | TBD for 75m |
| Adjacent channels, N±1 | 1 dB | 91.2 km | TBD for 75 m |
| N±2 and beyond | 1 dB | 6.6 km | TBD for 75 m |

Note: ITU-R Rec. P.1546, 1 MW ERP and 300 m HAAT DTV station

In the case of the adjacent channel, the WRAN interference into DTV reception is the constraining case.  The separation distance needed to prevent DTV interference into the WRAN service is less than the distance to the protected contour (from what?).  The table also indicates that CPEs and base stations located too close to the TV transmitter would suffer desensitization from its out-of-band emissions.  Again, the WRAN system planner may decide to allow his receivers to suffer more that the 1 dB desensitization.  The BS and CPEs could then be located closer than the distances given in Table 2.   The system would have to make use of  higher operating values for the TPC parameters and/or lower transmission capacity because of the use of more robust modulation.

[Since TV broadcasting uses horizontal polarization, a means to reduce the level of interference into TV reception from the nearby WRAN transmit devices in the case of alternate channel operation is to use vertical polarization.  It has been found that, for typical UHF TV antennas, the maximum coupling between a 4 W WRAN transmitter and the TV receiver located at 10 m produces a level of -26 dBm at the input of the TV receiver.  In order to achieve this level, the antenna orientation needs to be kept within a tolerance of 10º in the polarization plane.  It was also found that the worse coupling occurs when the antenna elevation difference between the WRAN transmit and TV receive installations result in elevation angles close to 15º.  Better isolation can be achieved when the two antennas are at the same height.

The coupling between WRAN and TV reception for alternate channel operation should be considered in terms of absolute levels of power at the input of the TV receiver rather than an additional discrimination to be added to the TV receive antenna gain.  There will be a tendency for the TV receiving installation to use a lower gain receive antenna when the local TV signal power is higher.  The important point is to keep the WRAN power level at the input of the TV receiver below the saturation level of the TV receiver. Comes from Docs. 230, 231, 232]

        - Part 74 Separation

Once Part 74 operation has been identified through sensing or other means and its location has been identified through geolocation or other means, 4 Watt CPEs will either have to change channel in a radius of X km or will have to reduce their maximum TPC limit.  The amount of tapering in dB from the maximum allowed EIRP will be defined as follows from the fraction of the actual distance to the [protected contour (have to find a new term relating to Part 74)] to the distance indicated in the table:

Tapering = Path loss exponent * 10*log(actual distance/distance in the table)   (dB)

where:		path loss exponent = 3.0   [tbd]

    - EIRP Profile

In the case of alternate channel use when the CPE can be located as close as 10 m to the nearest TV receiver, special measures will need to be taken to protect the TV receiver from saturation (-8 dBm level).  Extensive tests have demonstrated that additional protection will be required for specific channel separation to protect the operation of the TV receivers.  Such protection is expressed in terms of a dB reduction of the maximum EIRP level of the TPC range allowed for the nearby WRAN device.  The maximum range corresponds to 4 W which produces a –26 dBm level at the input of the TV receiver.  Such additional constraints will reduce this maximum coupling level by a specified number of dBs to allow unimpaired TV reception in the most demanding conditions - at the edge of the protected contour.  The following tables indicate the values for these reductions from the nominal WRAN EIRP levels applicable at the CPEs and at the BS.

[Tables to be produced once the TV receiver non-linearity measurements have been completed.]

This section needs to be updated with the results from receiver sensitivity and interference testing from OET, CRC, Fox, etc.

[Do we use the NTSC taboo channel values for NTSC?  Are there values available for other TV and DTV systems? Depends on the laws of the other countries.]

    - Interference Mechanisms

      - Out-of-Band Emissions

It is assumed that the level of out-of-band emission from a WRAN CPE is such that it will result in a TV receiver desensitization of no more than 1 dB in the most demanding receiving condition - at the edge of the protected contour.  A 41 dB(uV/m) field strength as required for DTV reception at the noise limited contour corresponds to a signal level of –83.9 dBm at the input of the DTV receiver.  With a required SNR of 15.2 dB and a margin of 5.9 dB to limit the impact to 1 dB desensitization, this results in a level of –105 dBm.   A 4 W CPE using vertical polarization and located at 10 m from a DTV receiving installation produces a –26 dBm at the input of the DTV receiver.  1 dB receiver desensitization will be achieved with a –26 – 105 = 79 dB out-of-band rejection at the output of the CPE.

Since the minimum field strength at the Grade B contour of the NTSC coverage is 64 dB(uV/m), the out-of-band rejection requirement necessary for DTV will be the driving factor.

Values are needed for other TV systems.

      - Third-Order Intermodulation

Special attention will be needed from the WRAN operator to avoid 3rd order intermodulation products from two high power signals present at the TV receiver that fall in the selected channel due to non-linearities in the TV receiver front-end.  From the geolocation capabilities of the WRAN system, the availability of the incumbent database and the results of RF sensing, the WRAN operator will be in a position to know within which protected contours each of his CPEs are located.  A simple calculation considering the ‘2A-B’ intermodulation scenario will indicate which channels would generate an intermod component falling on a protected channel at the nearby TV receivers.  The use of these channels should be avoided for the given CPE and, if it needs to be used because of the lack of other available channels, an additional 15 dB [tbd] reduction in the maximum EIRP level of the TPC range will need to be applied.  A representative diagram might help demonstrate.

# Installation

# System

    - Chassis

Chassis radiation is assumed to be and SHOULD be insignificant (e.g. <= -20 dB) compared to radiation from the intended path via the antenna.

# Base Station

Base stations SHALL be designed to be professionally installed.

Base stations SHALL be professionally installed and maintained.

"Professional installation" means that the installation must be supervised or inspected by a trained, competent professional, such as e.g. a NARTE Certified EMC Engineer, an SBE Certified Professional Broadcast Engineer, or a Registered Professional Engineer.

The base station SHALL be professionally installed by a party that will determine the device’s geographic location and the available unused TV channels at that location.

The installing party SHALL configure the base station to operate on only unused (might need to more clearly define “unused”) channels.

# Antennas

Base station antenna systems SHALL be designed and installed to support the requirements of 15.1.1.7, 15.1.6, and 15.1.7 in the FRD.

# Transmit Antenna

        - Pattern

# Directivity

(To reduce the vertical sidelobes of the pattern toward ground to avoid saturating nearby TV receivers as described in section V.i.c and to meet Part 15.209 and 1 dB TV receiver desensitization.)

# Polarization

The antenna has to be oriented orthogonal to the orientation of the nominal television receive antennas in the area in the polarization plane within a tolerance of 3 deg.

      - Receive Antenna

        - Pattern

          - Directivity

          - Polarization

The antenna has to be oriented orthogonal to the orientation of the nominal television receive antennas in the area in the polarization plane within a tolerance of 3 deg

      - Sense Antenna

        - Pattern

The base station SHALL sense licensed transmissions using an omni-directional antenna (might consider electronically steerable antennas – to achieve higher gain sensing in all directions)

          - Gain

The base station SHALL sense licensed transmissions using a sense antenna with a gain of 0 dBi or greater (where all losses between the antenna and the input to the receiver are included) in any azimuthal direction and polarization.

          - Polarization

The antenna has to have similar gain in both horizontal and vertical polarizations.

# RF Safety Limits

FCC OET Bulletin 65, Annex 2, Table 2:

# Cable Protection

FCC OET Bulletin 65, Annex 2, Table 2:

# Customer Premises Equipment (CPE)

CPEs MAY be user installable (plug and play) but operators MAY choose to provide for professional installation.

For the system to operate correctly, proper CPE installations SHALL be verifiable.  Means MAY be provided to allow verification of proper CPE installation remotely from the base station. Proposers are strongly encouraged to submit ideas for mechanisms by which such verification could be done automatically. If such a mechanism is adopted, it SHALL ensure that CPEs will not transmit absent reliable verification.  In the absence of any automated verification method, visual verification of proper installation will be necessary.  This paragraph needs to be worded more appropriately for this document.

# Antennas

Sensing thresholds specified in section 15.1.3 in FRD are based on these assumptions.  Installation and deployment requirements should be further elaborated in proposals with the understanding that, due to the unique requirement of assuring incumbent services are protected, they will be incorporated in a normative manner in the standard.

CPE antenna systems SHALL support the requirements of 15.1.1.7 of FRD (sensing thresholds), 15.1.6 of FRD (maximum transmitted EIRP) and 15.1.7 of FRD (out of band emissions). Proposers SHOULD comment on how to integrate the various elements of antenna systems to meet these requirements.

Verification that the CPE antennas are permanently installed outdoors on a fixed structure and properly aligned is required.

Verification that the CPE antennas are installed in a fixed outdoor location, and remains in the location where it was installed is required.

# Transmit Antenna

A minimum separation distance of 10 m between the CPE transmit antenna and DTV receiving antennas is required.

# Installation Height

The CPE transmit antenna SHALL be assumed to be installed outdoors at a nominal height of 10 meters above ground level and co-located with the sense antenna.  Proposals may discuss the implications of WRAN antenna installations at heights other than 10 meters above ground level.

Need to be installed outside to minimize the blockage and keep the TPC level and or maximize the transmit capacity (since more efficient modulation can be used)as much as possible .

        - Pattern

          - Gain

Main beam coupling may exist between the antennas for channels N±2 and beyond inside the noise-limited protected contour.

          - Orientation

Verification that the CPE transmit antenna is oriented properly (pointed at the base station) to minimize the CPE transmit power resulting from the use of local TPC is required.

The antenna needs to be oriented toward the base station of the selected service provider.  It is possible that if a new service provider is selected for technical or other reasons, the antenna may need to be re-oriented toward the new service provider antenna.  Proper orientation will allow the TPC level to be kept to its minimum and or maximize the transmit capacity (since more efficient modulation can be used).

          - Front-to-Back

In the case of co-channel and first adjacent channel operation, both antennas are assumed to be looking away from each other since the base station has to be located at a certain distance outside the noise-limited protected contour of the DTV station as indicated in the previous section.  The back-lobe rejection of both antennas can therefore be relied upon.

          - Polarization

WRAN CPE transmit antennas SHOULD be polarized orthogonally to nearby DTV receive antennas to provide

additional isolation through cross-polarization discrimination.

For channels N±2 and beyond, polarization discrimination is the only way to increase isolation based on the fact that the CPE transmit antenna and the DTV receive antenna will be orthogonally polarized.

Some antenna polarization discrimination MAY be assumed and proposers SHOULD document their assumptions in this regard.

The possibility of some signal depolarization between the two antennas SHOULD be considered as well.

The antenna has to be oriented vertically in the polarization plane within a tolerance of 10 deg.  In order to maximize the polarization discrimination toward a nearby TV receiving installation, it is preferable to mount the antenna at the same height as the nearby TV receive antenna.

        - Tamper Resistant

        - Tamper Resistant

Provisions SHALL be made to prevent the use of antenna equipment that could impair or defeat the requirements of the preceding two paragraphs and proposers SHALL elaborate their proposed approaches.

      - Receive Antenna

        - Installation Height

        - Pattern

          - Gain

          - Orientation

          - Front-to-Back

          - Polarization

The antenna has to be oriented orthogonal to the orientation of the nominal television receive antennas in the area in the polarization plane within a tolerance of 10 deg.

      - Sense Antenna

        - Installation Height

The CPE sense antenna SHALL be assumed to be installed outdoors at a nominal height of 10 meters above ground level and co-located with the transmit antenna.  Proposals may discuss the implications of WRAN antenna installations at heights other than 10 meters above ground level.

Special attention SHALL be given to the installation of the outdoor antenna to make sure that the sensing capabilities of the CPE are not impacted by local obstructions.

        - Pattern

It is assumed that the sense antenna will be omni-directional.

The sensing antenna has to be omnidirectional in the horizontal plane with a maximum gain variation of 1 dB (NPRM response by 802.18).

          - Gain

CPEs SHALL sense licensed transmissions using an antenna with 0 dBi gain or greater (where all losses between the antenna and the input to the receiver are included) in any azimuthal direction and polarization.

The sensing antenna gain has to be at least 0 dBi (NPRM response by 802.18).  In case where it is not possible, the lower gain

          - Orientation

          - Front-to-Back

In the case of co-channel and first adjacent channel operation, both antennas are assumed to be looking away from each other since the base station has to be located at a certain distance outside the noise-limited protected contour of the DTV station as indicated in the previous section.  The back-lobe rejection of both antennas can therefore be relied upon.

          - Polarization

WRAN CPE transmit antennas SHOULD be polarized orthogonally to nearby DTV receive antennas to provide

additional isolation through cross-polarization discrimination.

For channels N±2 and beyond, polarization discrimination is the only way to increase isolation based on the fact that the CPE transmit antenna and the DTV receive antenna will be orthogonally polarized.

Some antenna polarization discrimination MAY be assumed and proposers SHOULD document their assumptions in this regard.

The possibility of some signal depolarization between the two antennas SHOULD be considered as well.

The sensing antenna needs to receive equally on both polarizations for TV sensing and Part 74 wireless microphone.

        - Tamper Resistant

Provisions SHALL be made to prevent the use of antenna equipment that could impair or defeat the requirements of the preceding two paragraphs and proposers SHALL elaborate their proposed approaches.

# RF Safety Limit

FCC OET Bulletin 65, Annex 2, Table 2:

# Cable Protection

FCC OET Bulletin 65, Annex 2, Table 2:

# RF Receive Chain  (The following might instead be placed in an annex for assumption reference.)

# Noise Figure

2.5 dB assumed

# Filtering Loss

1 dB + 0.5 dB antenna coupling loss

# LNA Gain

20 dB

# Cable Loss

3.5 dB + 0.2 dB connector loss

# Receiver Noise Figure

8dB

# Overall Figure of Merit: G/T

-17 dB (1/K)

# Operation

- System

- Continuous Monitoring

- Software

Devices must include features to ensure that only software that is approved with a device can be loaded into this device, and the software may not allow the user to operate the device with parameters outside those that were approved.  “Software” in this context includes the software that selects a device’s operating frequency, determines a device’s geographic location, identifies TV channels that are vacant, and accesses information in a related database.

- Distributed Sensing

Sensing reliability is improved  by the collaborative sensing from a number of well selected CPEs based on their geo-location relative to the presumed origin of the signal since all this information will be available at the base station.  The sensing scheme needs to be considered more as a component of a more complex distributed sensing system than an independent device that, on its own, needs to meet the sensing requirement.

- Base Station

Base stations SHALL be professionally operated.

This standard should support different classes of base stations with different conditions of operation such as transmit power levels, etc., where permitted by regulatory domains.

Proposers SHALL elaborate in detail the technical characteristics of each class proposed (transmit EIRP, out of band emissions mask, required separation distance from TV receivers, EIRP profile if required, etc.).

Proposers SHALL provide analysis and simulation to justify the proposed technical parameters of each class proposed and provide a very high degree of confidence that those parameters will not cause harmful interference to licensed services.

    - Requests to the CPE

The base station SHALL be able to request local interference sensing and channel scanning from a CPE at any time, especially for a new CPE trying to get registered with a base station, to verify that its local interference situation would allow it to access the network.

    - Maximum Effective Isotropic Radiated Power

The default class of base station SHALL operate at a maximum of 4 W EIRP.

    - Duty Cycle

Base stations SHALL support 100% transmit duty cycle at rated power.

    - Monitoring

The base station or its operator SHALL periodically access a TV channel availability database derived from current FCC data and computational software to ensure that the channels on which the device operates remain unused.

      - Manual Monitoring

      - Automatic Monitoring

Information will be sent and collated at the base station where mapping of the interference situation will be made based on the sensing information from the CPEs and according to their respective geographic coordinates.  Solutions to avoid interference will be deduced at the base station and measures will be signalled to the CPEs such as channel change or power reduction.

Sensing reliability is improved by the collaborative sensing from a number of well selected CPEs based on their geo-location relative to the presumed origin of the signal since all this information will be available at the base station.  The sensing scheme needs to be considered more as a component of a more complex distributed sensing system than an independent device that, on its own, needs to meet the sensing requirement.

  - Customer Premises Equipment

    - Maximum Effective Isotropic Radiated Power

4 Watt maximum CPE transmit EIRP.

Accordingly, the maximum transmitted CPE EIRP will have to be limited (by capping the TPC range) to a value that is related to the channel separation between WRAN and TV operations.

- Duty Cycle

CPEs SHALL support 100% transmit duty cycle at rated power to support cases where maximum upstream throughput is required from a terminal at the edge of the coverage area.

Annex 1

Planning of Service Procedure Example

- Acquire the necessary tools and information.

  - WRAN BS operating parameters

    - Antenna characteristics

    - Intended height of the radiation center AGL

    - Operating channel

  - Incumbent receiver characteristics

    - Receivers are outdoors at 9 m height.

    - Receiver antenna patterns use a cosine exponent equal to 4.

  - Incumbent database

    - TV transmitter operating parameters are from the 05/09/05 FCC CDBS database.

    - Only co-channel and adjacent channel interference was considered (“taboo” channel relationships were not considered).

    - Only high power TV stations were considered (low power or “class A” TV stations were not considered).

  - Propagation analysis tool

    - 8-Vestigial Side-Band (8-VSB) Advanced Television Systems Committee modulation for 802.22 systems.

    - Desired to undesired ratios (D/U) from the Federal Communications Commission (FCC) Office of Engineering and Technology Bulletin 69.

    - 30 meter United States Geological Survey terrain data.

    - Population from 2000 Topographically Integrated Geographic Encoding and Referencing system census data.

    - Terrain Integrated Rough Earth Model propagation prediction model (TIREM).

    - Long Term Fading, Environmental Science Services Administration (ESSA) Technical Report ESSA Research Laboratories 79-Institute for Telecommunication Sciences 67.

    - Surface of the Earth Electrical Characteristics from International Telecommunication Union Recommendation 527-3.

    - 802.22 coverage simulations use 99.9 % time availability.

    - Digital television (DTV) interference analysis, desired and undesired signals use 90 % and 10 %, respectively for time availability.

    - Analog interference analysis desired and undesired signals use 50 % and 10 %, respectively for time availability.

    - Other assumptions regarding transmitter and receiver parameters for link budgets are from 802.22 document number 22-04-0002-10-0000.

- Identify the area of interest to deploy the WRAN service.

In this example, a location in Linden, TN was chosen.

- Determine a useable channel by plotting the co- and adjacent channel incumbent service noise-limited contours in the area of interest.

Note: The figures above do not accurately represent final post-transition DTV service or associated service contours.

Current co- and adjacent channel DTV noise-limited contours were plotted in the Linden, TN area for channels 25 through 30.  In the figures above, the black circle is a 40 km radius circle with center at a site in Linden, TN of high terrain elevation.  The red contours are the co- and adjacent channel DTV noise-limited contours.  It can be seen in the figures above that channels 26 through 29 are unusable for the Linden, TN site.  It appears that channels 25, 29, and 30 are possible because the selected site would not be contained within either a co- or adjacent channel DTV noise-limited contour.

For this example channel 30 is selected.

- Analyze the interference potential from the planned WRAN system into the nearby DTV service.

Assumptions:

The purpose of this analysis was to simulate the deployment of an Institute of Electronics and Electrical Engineering (IEEE) 802.22 system operating within the television (TV) broadcast bands from both a coverage consideration and an interference consideration.

Both coverage and interference simulations were performed in different locations in the United States.  These locations were selected to represent rural areas outside of both co-channel and adjacent channel TV noise limited contours.  Although the locations were outside of TV noise limited contours, co-channel, and usually adjacent channel, contours were nearby.

The coverage and interference simulations were performed using News Corporation’s Fox Technology Group’s propagation prediction software.

Coverage plots were created for each of the six locations analyzed as well as interference plots.  For each location, plots were created to describe the interference caused by an 802.22 base station into a TV receiver and the interference caused by a TV transmitter into an 802.22 consumer premises equipment (CPE).  Occasionally additional plots were created for a location to illustrate a particularly interesting interference example.

References:

National Association of Radio Telecommunications Engineers

Electro Magnetic Compatibility

Society of Broadcast Engineers