IEEE P802.22
Wireless RANs

| Draft Recommended Practice |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-11-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Winston Caldwell | FOX | 10201 W.Pico Blvd Los Angeles, CA 90064 | 310-369-4367 | Winston.caldwell@fox.com |
| Gerald Chouinard | Communications Research Center | 3701 Carling Avenue Ottawa, Canada K2H-8S2 | 613-998-2500 | Gerald.chouinard@crc.ca |

Abstract

This document will serve as the working document toward the draft 802.22.2 Recommended Practice.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# 802.22 WRAN Reference Model

The Wireless Regional Network standard developed under the P802.22 is aimed at point-to-multipoint wireless systems intended principally to extend broadband access to less populated rural areas where vacant channels in TV broadcast bands are likely to exist in larger quantity than in more populated areas.  The use of these TV bands for broadband access has the advantage of providing for better propagation conditions to reach larger distances with reasonable transmission power.

The typical WRAN system operation will include a Base Station (BS) and a number of Customer Premises Equipments (CPEs).  During the development of the IEEE 802.22 standard, the BS is assumed to have an Omni-directional or sectoral vertically polarized antenna at 75 m above average ground level.  The CPE is assumed to have a directional transmit and receive antenna and an Omni-directional sensing antenna at 10 m above average ground level.  All of the RF parameters of the CPE are remote controlled by the BS.

The WRAN standard was developed to provide a broadband access equivalent to the first generation of ADSL and cable modems to the rural population who would otherwise have no service except over telephone lines or satellite.  The standard was developed with the aim of providing a minimum peak downstream capacity of 1.5 Mbit/s and a minimum peak upstream capacity of 384 kbit/s per subscriber with a service reliability of 50% location and 99.9% time at the edge of the coverage area [these rates might be different now (esp. the lower rate)].

The spectrum efficiency for the system varies from 1 bit/(s*Hz) for the most robust transmission (QPSK and FEC rate= ½) to 3.24 [need to reflect change in FRD] bit/(s*Hz) in the case of the closer-in line-of-sight CPEs (64QAM and FEC rate= 5/6), not counting the overhead needed for the system synchronization, TDD transition gaps, quiet periods, etc.. The overhead represents some 25% of the transmission capacity.  The lowest efficiency will be used for CPEs at the edge of the contour or in places hard to reach.  A combination of higher modulation and FEC code will be used for easily reacheable terminals.  Transmit power will be adjusted according to the ranging process and through Transmit Power Control (TPC).

The average spectrum efficiency will be about 1.8 bit/(s*Hz) for a typical rural town or village and its surrounding areas where the subscriber density will decrease as a function of the distance from the BS.  This efficiency results in a total capacity of about 10 Mbit/s in a 5.6 MHz channel for a simple Omni-directional BS.  Assuming TDD operation with a 1.5 Mbit/s downstream and 384 kbit/s upstream capacity per CPE and a 40:1 over-subscription ratio, which is typical of the first generation of ADSL/cable-modem, the number of subscribers that can be sustained by this Omni-directional BS is 478 subscribers1.

Using an unlicensed low power BS as described in section 1.1.1., the system would reach 16.7 km.  A higher power BS as described in section 1.1.2. would reach 30.7 km under the same propagation conditions   With time, the number of subscribers in the area will increase and the WRAN operator would manage this increase by adding omnidirectional base stations operating on different TV channels or by sectorizing his coverage area to take advantage of potential frequency re-use.

# Possible Scenarios

# Licensed-Exempt Low Power Base Station WRAN System

Based on the proposal from the FCC NPRM 04-186, it is assumed that the unlicensed CPEs and BS will be limited by 4 W EIRP to help in protecting the incumbent services in the TV bands.  Since it is intended for the BS to serve as many CPEs at the same time as possible in making effective use of the OFDMA multiplex, the BS EIRP will likely be kept close to the 4 W limit almost on a continuous basis.  Because of its lower transmission capacity, the CPE will only need to use a transmit power equivalent to the portion of the 4 W EIRP to secure a balanced RF transmission in both directions between it and the BS with which it is associated.  The only time when the CPE EIRP will come close to the rated maximum is when this CPE is allowed to use the full channel capacity in the upstream direction and is located such that it uses its TPC at full capacity.  This would occur in the case of a CPE at the edge of coverage being given the full channel upstream capacity for uploading large files overnight, for example.

# [Higher Power Base Station WRAN System] [Could instead title, “Licensed WRAN System,” to address domains outside of FCC regulation]

Also based on the proposal from the FCC NPRM 04-186 that the EIRP from the license-exempt devices should be limited to 4 W to help in protecting the incumbent services in the TV bands, it is assumed that, since the BS will be professionally installed, a BS would be allowed to use higher transmission power whilst still protecting the incumbents to serve their multiple CPE terminals to secure a balanced RF transmission in both directions between the BS and the CPEs that are allowed to transmit up to 4 W EIRP.  Because of the OFDMA modulation used in the 802.22 standard the CPEs will be able to use only the portion of the carrier multiplex that they need to transmit their data towards the BS.  This portion will depend on the capacity required at that moment and will be able to use up to 4 Watt EIRP.

The standard has defined that the minimum portion that could be used by a CPE in its upstream is one sub-channel which contains 24 data 54 carriers whereas the complete multiplex contains 1440 carriers for a total of 60 subchannels2.  However, in order to provide for the minimum rated upstream capacity of 384 kbit/s at the edge of coverage, a minimum of 6 or 7 sub-channels would be needed by the CPE depending on the cyclic prefix used assuming that all the useful data symbols in a frame can be dedicated to the upstream traffic (6 sub-channels for cyclic prefixes 1/16 and 1/32 while 7 symbols would be needed for cyclic prefixes ¼ and 1/8).  Note that in those cases where a lower upstream data rate is needed, the BS can in fact reduce the upstream capacity allocation of the CPE down to only 1 sub-channel which corresponds to 16.8 kbaud/s for the minimum allocation of 7 upstream symbols per frame).

In order to establish a communication with a CPE at the edge of the contour, the BS would need to modulate the carriers belonging to this communication channel with the most robust parameters such as QPSK and FEC rate= ½ in order to reach as far as possible.  On the return channel, the CPE would modulate the carriers of its sub-channel with the same parameters and transmit at the maximum 4 W EIRP.  Hence, in order to establish a balanced RF link, the BS will need to transmit with an EIRP that will correspond to 60 sub-channels rather than the 6 or 7 sub-channels on the upstream and compensate for any difference in receiver sensitivity and receive antenna gain between that available at the CPE receiver and the BS receiver.   For example, in the case of a cyclic prefix of 1/8 and for a 3 dB better receiving G/T at the BS compared to that of the CPEs, the EIRP of the BS will be:

EIRPBS = 4 * 60/7 *2 = 68.6 Watts

This BS EIRP will allow

balanced RF transmission with the CPEs using 4 W EIRP on their return link to provide for the required 384 kbit/s upstream capacity (in fact, 436.8 kbit/s when the entire frame can be dedicated to the upstream).  The BS will normally operate at this power to provide service to the fringe CPEs eventhough lower power would normally be needed to reach the closer-in CPEs since the power differential among the carriers in the ODFM transmission on the downlink should be minimized for best ODFDM decoding at the CPEs.  Conversely, the TPC settings will make sure that  lower power is transmitted from the closer-in CPEs so that the differential in carrier power is minimized as the upstream transmissions are received at the BS.

If the WRAN service was to be designed to serve CPEs at the edge of the coverage area that can provide the minimum upstream data rate, i.e., 16.8 kbit/s which corresponds to a single sub-channel during the minimum 7 consecutive symbols neede for channel training, with 4 Watt EIRP (i.e., VoIP service), the power of the base station would have to be:

EIRPBS = 4 * 60/1 *2 = 480 Watts

and the WRAN coverage could then extend to some 39 km for 4 Watt CPEs and the 480 Watt base station EIRP.  The specification of the maximum EIRP of the base station therefore depends on the definition of the minimum service at the edge of the WRAN coverage area.

.

This higher BS transmit EIRP will be taken into consideration in the protection of the DTV receivers at the planning stage by extending the keep-out distances for co-channel and adjacent channels from the nearby TV and DTV protected contours to cover this extra 12 to 21 dB EIRP.  This higher BS transmit EIRP will also be taken into consideration at the professional installation stage by making sure that the BS is located at  √ (68.6/4)= 4.14 to √(480/4)= 11 times the nominal 10 m minimum separation between the 4 W CPE and the nearest DTV receiving installation.  The BS antenna will therefore need to be at 41.4 to 110 m from the closest DTV receiving installation.  This should not be onerous because it is expected that the BS antenna height is expected to be some 75 m above ground to optimize the service coverage of the WRAN system.  The professional installer therefore needs to consider the service coverage range of the BS as well as the proximity of TV receivers in deciding on the antenna height.  Buying the land over these 40 to 110 m around the BS would be another simple means of avoiding any potential interference problem.

With this extended range of EIRP, the minimum distances to the transmit antenna of the BS will need to be increased tp meet the RF safety limit.  Such RF safety limit is contained in the FCC OET Bulletin 65 and is specified to be at 0.2 mW/cm2 in the VHF bands and F(MHz)/1500 mW/cm2 for the UHF band.  This limit corresponds to 28 cm for 4 Watt EIRP, 1.15 m for 68.6 Watt EIRP and 3.05 m for 480 Watt EIRP.  Since the base station is to be professionally installed, the antenna height above average terrain will amply cover for such minimum distance for RF safety.

The TG1 beacon has been designed so that any WRAN device, BS or CPE, using a maximum of 4 Watt EIRP can detect it within the distance at which the WRAN device could create interference to wireless microphone operation.  However, the increase in interference potential due to the additional 12 to 21 dB in EIRP for the higher power base stations will increase the range of potential interference beyond the range where TG1 beacons can be sensed by the BS.  Special considerations will be needed to make sure that there is a sufficient number of CPEs involved in distributed sensing in this extended interference range around the BS for reliably detection of the TG1 beacons.

# Keep-Out Distances

The keep-out distances in this section are listed in the d(n) (m) notation, representing the keep-out distance, d, for the channel relationship between the channel used by the incumbent service and the channel used by the WRAN device, n.  Since a WRAN device is not allowed to operate co- or adjacent channel to an incumbent service, inside the protected contour of the incumbent service, the keep-out distances provided in this section are for n = 0 (co-channel, N), +1 (upper-adjacent channel, N+1), and -1 (lower-adjacent channel, N-1).  The keep-out distances are different depending on the type of incumbent service, the output power of the WRAN device, and whether the WRAN device is a BS or a CPE.

d(n)

Incumbent

Transmitter

Protected Contour

WRAN Interference Range

Keep-Out

Distance

WRAN

Device

Figure 1: Keep-Out Distance

Figure 1 is a general graphic illustration describing the keep-out distance.  The green triangle at the center of the figure is the incumbent service transmitter.  The green line represents the protected contour.  The red triangle in Figure 1 is the WRAN device.  The red line represents the interference range of the WRAN device.  Harmful interference occurs as soon as the red line overlaps the green line.  In Figure 1 the WRAN device is located at the minimum distance from the protected contour so that overlap, and therefore interference, does not occur.  This minimum distance is the keep-out distance.  The keep-out distance, d(n) is shown in Figure 1 as the distance from the protected contour to the black line surrounding the protected contour.

If there is a TV operation on channel N, a WRAN device outside of the protected contour of that TV station:

- Shall not transmit on channel N within a distance of d(0) (m) of the protected contour with a transmitted EIRP of 4W.

- May transmit on channel N within a distance of d(0) (m) of the protected contour as long as the transmitted EIRP is reduced below 4 W by a rule that models the directivity of the CPE transmit antenna toward the protected contour and the reduced loss in propagation based on an agreed upon propagation model so that the proper co-channel D/U ratio is not exceeded at the protected contour. A minimum distance (d_min) to the protected contour within which no transmission would be allowed is specified to cover for the limited precision of the geo-location technology (typically 100 m).

- Shall not transmit within a distance of d(-1) (m) of the protected contour on channel N-1 with a transmitted EIRP of 4 W. The distance d(-1) (m) being typically in the range of the geo-location precision, it is not allowed to use a reduction of the TPC range to transmit within this distance.

- Shall not transmit within a distance of d(+1) (m) of the protected contour on channel N+1 with a transmitted EIRP of 4 W. The distance d(+1) (m) being typically in the range of the geo-location precision, it is not allowed to use a reduction of the TPC range to transmit within this distance.

According to calculations1, Table 1 provides the required keep-out distances for a CPE transmitting at 4 W EIRP to meet the D/U ratios at the protected contour as shown in Tables 4-6 in in Section 3.

Table 1: Keep-Out Distances d(n) (m) for a 4 W CPE

|  | Incumbent Service |  |
| --- | --- | --- |
| Channel Relationship, n | DTV | Analog TV |
| 0 | 3,100 | 1,500 |
| +1 | 134 | 31 |
| -1 | 115 | 44 |

In particular, this means that no co-channel or first adjacent channel operation is allowed within the protected contour by any WRAN device. But operation outside the TV protected contour is allowed with a constraint on the keep-out distance between the device and the contour when a device is transmitting at 4 W EIRP.

Since they are based on the propagation loss from the higher elevation BS transmit antenna and possibly higher EIRP, the minimum separation distances for d(0), d(-1) and d(+1) for the base station will need to be greater than that for the CPE.

According to calculations1, Table 2 provides the required keep-out distance for a BS transmitting at 4 W EIRP to meet the D/U ratios at the protected contour as shown in Tables 4-6 in in Section 3.

Table 2: Keep-Out Distances d(n) (m) for a 4 W BS

|  | Incumbent Service |  |
| --- | --- | --- |
| Channel Relationship, n | DTV | Analog TV |
| 0 | 16,000 | 8,400 |
| +1 | 566 | 189 |
| -1 | 485 | 243 |

According to calculations1, Table 3 provides the required keep-out distance for a BS transmitting at 100 W EIRP to meet the D/U ratios at the protected contour as shown in Tables 4-6 in in Section 3.

Table 3: Keep-Out Distances d(n) (m) for a 100 W BS

|  | Incumbent Service |  |
| --- | --- | --- |
| Channel Relationship, n | DTV | Analog TV |
| 0 | 31,200 | 17,800 |
| +1 | 1,600 | 561 |
| -1 | 1,400 | 707 |

# EIRP Profile

The EIRP Profile in this section is defined in terms of f(n) (dBW) notation, representing the hard limit for maximum EIRP, f, for the channel relationship between the channel used by the incumbent service and the channel used by the WRAN device, n.

This EIRP profile defines the maximum EIRP limit that a WRAN CPE or BS must not exceed in order to avoid causing harmful interference.  The EIRP profile is calculated assuming a reference minimum distance of 10 m between the WRAN device and a TV receiving installation.  The EIRP profile is used as a template to calculate the maximum allowed EIRP for each TV channel where an incumbent is present in the area as a function of the channel relationship between the TV operation and the WRAN operation.  The EIRP profile is provided at the end of this section for use in the US in Tables 7 and 8.  The EIRP profile will ultimately need to be reviewed and confirmed by the local regulatory body and stored in the database service.

If there is a TV operation on channel N, a WRAN device located within the protected contour of that TV station:

- Shall not transmit on channel N

- Shall not transmit on channel N-1

- Shall not transmit on channel N+1

- Shall meet a maximum transmitted EIRP constraint on alternate channels (N±2 and beyond), as defined by the superposition combination of the EIRP profile for each channel relationship.

In the case of second adjacent channel relationships and beyond, a WRAN device can be located close to the a TV receiving installation.  Special measures, such as the use of vertical polarization and the reduction of the maximum EIRP for WRAN systems, will need to be taken to protect the TV receiver from saturation (e.g., -8 dBm level), taboo channel interference, and third-order intermodulation.  Such protection is expressed in terms of a maximum EIRP in dBW.  Calculations1 assuming the values provided in OET 69 have shown that a WRAN device transmitting at 4 W EIRP on a TV channel in the UHF band would result in -4 dBm received power at the input of a TV receiver 10 m from the TV receiving antenna if the two antennas are pointed at each other.  Field testing2 has shown that the use of cross-polarization at 10 m separation between a vertically polarized transmit antenna and a horizontally polarized receive antenna can reduce the effective gain of the link on a TV channel in the UHF band by 14 dB including local multipath.  The inclusion of the 14 dB polarization discrimination results in a total received power of -18 dBm at the input of the TV receiver.

The EIRP profile is derived by equation (1).

EIRP=minE - 20log(615 / f) -D/U + I - 114.8) dBW	(1),

EIRP – maximum effective Isotropic radiated power

E – field strength at the edge of the protected contour (dBu)

f – mid-band frequency of the operating channel (MHz)

D/U – desired-to-undesired field strength ratio (dB)

I – minimum antenna discrimination (dB)

EIRPREG – regulated maximum effective Isotropic radiated power

The EIRP for a WRAN device must never exceed the EIRP determined using equation (1).  The EIRP determined by equation (1) can never be greater that the maximum EIRP set by local regulations.  EIRPREG in the US is 6 dBW.  However the resulting EIRP should be further limited using equation (1) to prevent taboo channel and third-order intermodulation interference.

Equation (1) assumes that because the WRAN device is located inside the protected contour, the device could be located at a minimum distance of 10 m from the nearest protected receiving installation.  Therefore the EIRP is calculated assuming a separation distance of 10 m, free-space propagation, and minimum antenna discrimination since the actual location of the device cannot be controlled.

Table 4 lists the field strength at the edge of the protected contour according to US regulations.

Table 4: Field Strength at the Edge of the Protected Contour According to US Regulations

| TV Band | TV Service | Field Strength (dBu) |
| --- | --- | --- |
| L-VHF | Analog | 47 |
|  | Digital | 28 |
| H-VHF | Analog | 56 |
|  | Digital | 36 |
| UHF | Analog | 64 |
|  | Digital | 41 |

Table 5 lists the desired-to-undesired field strength ratios for second adjacent channel relationships and beyond (taboo channels) based on the ATSC A/74 DTV receiver recommended performance3 for DTV while Table 6 is according to OET Bulletin #694 for analog TV.

Table 5: DTV D/U for Second Adjacent Channel Relationships and Beyond

| Channel Relationship | D/U (dB) |
| --- | --- |
| N+/-2 | -48.2 |
| N+/-3 | -56.4 |
| N+/-4 | -64.7 |
| N+/-5 | -70.8 |
| N+/-6 to N+/-13 | -69.7 |
| N+/-14 and 15 | -55.3 |

Since the ATSC A/74 Recommendation only provides the D/U for moderate and weak desired signal levels, the D/U values provided in Table 5 for a desired signal level at the edge of the protected contour were linearly (in dB) extrapolated from the weak and moderate values.

Table 6: Analog TV D/U for Second Adjacent Channel Relationships and Beyond

| Channel Relationship | D/U (dB) |
| --- | --- |
| N-2 | -24 |
| N+2 | -28 |
| N-3 | -30 |
| N+3 | -34 |
| N-4 | -34 |
| N+4 | -25 |
| N-7 | -35 |
| N+7 | -43 |
| N-8 | -32 |
| N+8 | -43 |
| N+14 | -33 |
| N+15 | -31 |

Table 7 contains the minimum antenna discrimination that can be assumed when calculating the EIRP for a CPE located inside a protected contour.

Table 7: Minimum Antenna Discrimination

| TV Band | TV Service | Antenna Discrimination (dB) |
| --- | --- | --- |
| L-VHF | Analog | 6 |
|  | Digital | 10 |
| H-VHF | Analog | 6 |
|  | Digital | 12 |
| UHF | Analog | 6 |
|  | Digital | 14 |

The following tables indicate examples of an EIRP profile for a WRAN device operating on various channel relationships to TV operation using the US values provided above.  Table 8 assumes that the device is located inside a protected contour of a DTV service operating in the UHF band.  Table 9 assumes that the device is located inside a protected contour of an analog TV service operating in the UHF band.  If a channel relationship is not provided, it does not need to be considered.

Table 8: EIRP Profile, f(n) for a CPE Located inside a UHF DTV Protected Contour in the US

| Channel Relationship, n | EIRP (dBW) |
| --- | --- |
| 0 | -100, Operation not Allowed |
| +/-1 | -100, Operation not Allowed |
| +/-2 | -11.6 |
| +/-3 | -3.3 |
| +/-4 | 4.9 |
| +/-5 | 6 |
| +/-6 to +/-13 | 6 |
| +/-14 and +/-15 | -4.5 |

Table 9: EIRP Profile, f(n) for a CPE Located inside a UHF Analog TV Protected Contour in the US

| Channel Relationship, n | EIRP (dBW) |
| --- | --- |
| 0 | -100, Operation not Allowed |
| +/-1 | -100, Operation not Allowed |
| -2 | -20.8 |
| +2 | -16.8 |
| -3 | -14.8 |
| +3 | -10.8 |
| -4 | -10.8 |
| +4 | -19.8 |
| -7 | -9.8 |
| +7 | -1.8 |
| -8 | -12.8 |
| +8 | -1.8 |
| +14 | -11.8 |
| +15 | -3.8 |

# Database Service

The database service is an essential component of the cognitive capabilites of the WRAN system to determine the correct operating parameters.  The database service helps to assure that the WRAN system does not cause harmful interference into the incumbent services and to assure that the WRAN system makes the most efficient use of the available spectrum for self-coexistence purposes.

Figure 2 shows the inputs and outputs for the database service and the communication between it and IEEE 802.22 using primitives.

Figure 2: Database Service Inputs and Outputs

SME-MLME-DB-INDICATION.indication

SME-MLME-DB-REG.request

Maximum EIRP

List of Available

TV Channels

Location Information

(lat/lon)

Transmit Antenna

Characteristics

Database Service

IEEE 802.22

The information that the database service accepts as input are the latitude, longitude, and the antenna characteristics for a device that is attempting to associate to the WRAN network.  This information is passed to the database service using the SME-MLME-DB-REG.request IEEE 802.22 primitive

As a result, the database service outputs the list of available TV channels and the EIRP profile.  The list of available TV channels and the EIRP profile that are outputted depend on the information contained in the database and on the interference mechanisms that are applicable based on local regulations.  This information is passed to IEEE 802.22 using the SME-MLME-DB-INDICATION.indication IEEE 802.22 primitive.

  - Databases

A database documenting the existence of broadcast incumbents will need to be developed and be made available on-line.  This database could also contain information describing the operation of other WRAN systems in the area.  One could use this database during the planning of the WRAN system deployment; while it is an 802.22 system requirement that the BS communicates with an existing database during operation.  A database helps to determine spectrum availability and to avoid harmful interference to the incumbent services.  A database is only as effective as the information contained in it; therefore, this information should be as accurate and up-to-date as possible.

    - Incumbent Database

The incumbent station database contains information describing the operation of protected services in the area, including the television broadcast services, wireless microphone operation, and safety of life land mobile operation. The incumbents should make certain that the information in the incumbent database that describes their station is contained in the incumbent database to ensure that the service coverage area is protected.  Since both WRAN service providers and incumbents could be affected by the information contained in the database, it would be appropriate that the development of such databases involve the incumbents, the potential WRAN service providers and the local regulators to determine the exact extent of the protection; and that the maintenance and administration of such databases be under government or third party responsibility.  Regardless of how the information in the incumbent database is formatted, the database service will use this information to generate the protected contour for the incumbent service. The protected contour defines a boundary within which broadcast receivers need to be protected from interference.  WRAN devices are restricted from operating co- or adjacent channel both within the protected contour and from an additional distance beyond the protected contour.  The database would limit the WRAN device to a decreased maximum EIRP for co- or adjacent channel operation if the WRAN device is located within this separation distance.  For channel relationships beyond co- and adjacent channel, in which case the device can be located inside a contour, the database would return a decreased maximum EIRP to avoid taboo channel interference.

      - Channel Information

All of the TV channels occupied in an extended area out to a radius defined by the interference range of any associated WRAN device should be contained in an incumbent database along with the geographic location of the transmitter, the transmit antenna pattern, height of the center of radiation (Above Ground Level (AGL)), and the ERP for each incumbent service present.  The database service would collect this information pertaining to an incumbent service and could construct the protected contour on the fly.

      - Polygons

As an alternative to populating the incumbent station database with station operation parameters, the database could be populated with pre-computed protected contours in the form of polygons that are represented by the coordinates of all the apexes of a contour.  Each contour must also be identified by a channel number.  These polygons that represent a protected contour must be computed according to the local regulations or agreed upon by negotiations with all of the interested parties.

      - Standardized Format

The database queries should be in SQL format and globally harmonized so that standardized computer tools could be used for planning the WRAN systems as well as during normal operation.  These queries should allow determination of the maximum EIRP for both BSs and CPEs in any location within the area that the database is supposed to cover.  These databases should make sure that there is consistentcy and continuity among the various local databases so that they perfectly overlap or stitch together.

    - WRAN Base Station Database

As a minimum, a registry of the BSs in operation in an area with their coordinates and operating characteristics should be constituted and made publicly available (e.g., on a website).  This information could either be incorporated into the incumbent database or it could be contained in a separate available database.  The WRAN base station database would help nearby WRAN systems coexist and make the most efficient use of the available spectrum.

The latitude, longitude, technical parameters such as the transmit/receive antenna pattern, the antenna height, the EIRP, and the unique identifier of the BS are to be provided for inclusion in a database that will be available for interference calculations and for coexistence purposes.

  - Protected Contour

The database service uses the information contained in the database to determine the boundary within which WRAN devices operating on co- or adjacent channel relationships to incumbent services are prohibited from operating.  Co- and adjacent channel operation is also prohibited by WRAN devices operating at full-power within the separation distance (as specified in Section X).  The database service detects that the registering WRAN device is within this separation distance and returns the reduced maximum EIRP allowed for the device for the co- and adjacent channels.  The protected contours would need to be defined with the agreement of the local regulator so that the right considerations such as the TV received signal level, the TV interference level, the Designated Market Area, etc. are taken into account.  The protected contour for DTV4 and analog TV5 in the US is determined using method described in the regulations.The fact that these contours could be simple, near-circular contours such as the ones used by the FCC in the US and up to rather complex contours resembling a "Swiss cheese" when precise topography is included would also be useful.  Different levels of precision could be used in different countries

The TV contours to be protected from WRAN interference are stored in a database of polygon points defined by latitude, longitude and altitude coordinates maintained at the WRAN base station (see the 802.22 WRAN Recommended Practice). These polygons will have been established from other databases such as those defining the true geographical limits of the protected contours of TV broadcast stations operating in the vicinity of the WRAN.  In the USA, the TV protected contour is the ‘Grade B protected contour’ for NTSC channels, and the ‘noise-limited protected contour’ for ATSC channels, and equivalent definitions will prevail in other parts of the world for other TV standards.

  - Examples

Implementation examples of the database service are shown in Annex 1 of this document.

  - Database Service Inputs

The database service accepts the SME-MLME-DB-REG.request IEEE 802.22 primitive provided by IEEE 802.22 as input.  The information in this primitive includes the type of device (BS or CPE), the type of query (device registration or device query), and both the location information and the transmit antenna characteristics for the device.

    - Location Information (Latitude/Longitude)

The location information is a required input to the database service.  This input is formatted as an ASCII string according to the NMEA 0183 standard.

    - Transmit Antenna Characteristics

The transmit antenna characteristics input could include the antenna gain, the azimuth pattern, and the elevation pattern for horizontal and vertical polarizations.  If the transmit antenna characteristics are not provided as an input, the database service should assume that the transmit antenna is Isotropic.  The height of the transmit antenna above ground level in meters (m) could also be provided to the database service.  If the height information for the transmit antenna is not provided, the database service should assume 75 m above ground level for a BS and 10 m above ground level for a CPE.

  - Database Service Outputs

The database service sends its output to IEEE 802.22 using the SME-MLME-DB-INDICATION.indication IEEE 802.22 primitive.  The information in this primitive includes a return of the location information and an array with  the list of available TV channels and the maximum EIRP for each of the available TV channels.

    - List of Available TV Channels

The list of available TV channels is an output of the database service.  Any channel not on the list of available TV channels cannot be considered by the WRAN system for operation.

    - Maximum Allowed EIRP

The database service outputs the maximum EIRP allowed for each channel on the list of available TV channels at the WRAN device location.  The database service computes the maximum EIRP values for a WRAN device using the location information for the device and the EIRP profile method described in section 3.

  - Calculation of the List of Available TV Channels and the Corresponding Maximum Allowed EIRP

WRAN CPEs and BSs are not allowed to operate on the same channel or on the first adjacent channels of a TV operation within the protected contour.  However they can operate co-channel or adjacent channel outside this protected contour as long as they are located at sufficient keep-out distances beyond this protected contour.  If the CPE is located outside of nearby protected contours and beyond the specified keep-out distance given in section 2, the WRAN device can operate at the maximum regulated transmitted EIRP.  WRAN CPEs or BSs can only be located outside of the protected contour but within the range of the keep-out distance if they reduce their maximum EIRP accordingly to protect TV operation as shown in section 4.6.3.1.

WRAN CPEs and BSs operating on a second adjacent channel or beyond can be located inside the protected contour as long as they meet the maximum transmit EIRP limits resulting from the application of the EIRP profile.

The maximum transmitted EIRP is determined by the two following steps:

- Determine the maximum transmitted EIRP for each device on each TV channel from the constraint of TV operations in each channel: fill in Table 9 per TV channel column by column, using the flowchart of Figure 3.

- Fill in Table 10 using the most contraining values in each row of Table 9.

This section presents the method to determine the maximum allowed transmitted EIRP for a single WRAN device transmitting in a single TV channel.  The maximum transmitted EIRP limit is determined according to the EIRP profile if the device is inside the protected contour.  Other constraints can be added on top of this maximum allowed transmitted EIRP to further restrict the maximum level of EIRP, but in no case shall the actual transmitted EIRP exceed the maximum allowed transmitted EIRP determined by this method.

Table 9 shows an example of how the maximum transmitted EIRP for a single WRAN device is computed by the database service from the knowledge of TV operations. Each column is filled in turn. Given a TV operation on TV channel N-2, and given that the device on channel N is located within that protected contour, it is determined that transmissions by the device on TV channels N-3, N-2 and N-1 are not allowed. The maximum transmitted EIRP of that individual device (conditioned on the assumption that it would be the only device transmitting in a given TV channel) is determined by the EIRP profile at +2 to +6 on TV channels N to N+4. It is assumed that the out-of-band emission mask specified in section 8.13 of the 802.22 standard6 meets the constraints on adjacent TV channel emissions when a device is transmitting in a given TV channel. The values in bold font in Table 9 illustrate the method to compute the maximum transmitted EIRP for a single device from constraints on all TV channels, by taking the minimum of all constraints on each row.  Although the example presented in Table 9 illustrates the process for a range of 7 TV channels (N-3 to N+4), the actual process will need to encompass a greater  range of channel relationships as specified by the EIRP profile in section 3.

The flowchart of the decisions made to fill-in one column of Table 9 is shown in Figure 33 [The flowchart does not include intermod considerations]. It shall be repeated for each device and for each TV channel. For a given device and for a given TV channel N, the process described in Figure 33 shall be repeated for each distinct TV operation identified in channel N. This situation may occur when a device is located near the edge of coverage of two non-overlapping TV operations on the same channel, particularly in the case of low power TV transmitters. The most constraining power limits from all distinct TV operations is reported in the column corresponding to channel N in Table 9. The parts of the flowchart represented with solid lines shall be implemented by the database service. The parts represented with dashed lines are optional. “d” is the known distance from the location of the CPE to the protected contour of the TV operating on channel N.

The EIRP profile is defined as f(n) in dBW units. It represents the maximum transmitted EIRP constraint on channel N, where N is the channel number used by the TV station, and M=N-n is the TV channel number used by the WRAN. Hence, the maximum allowed transmitted EIRP on channel N-n is f(N-M) = f(n), expressed in dBW, inside the protected contour when there is a TV operation on channel N. In this example, disallowed operation on channel N-n would be represented by f(n) = -100 dBW. Hence, it is required that f(0) = f(-1) = f(1) = -100 dBW to indicate that co- and adjacent channel operation within the protected contour is prohibited.

- Flowchart of the decision tree for determining the maximum transmitted EIRP limit on every TV channel for a single WRAN device at a given location to protect TV operations on channel N

- — Individual WRAN device maximum transmitted EIRP from each individual TV operation

| TV channel number | N-3 | N-2 | N-1 | N | N+1 | N+2 | N+3 | N+4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TV operation | No TV station | One TV station | No TV station | One TV station | No TV station (a known TV station is off) | No TV station | One TV station | No TV station |
| WRAN device location | Outside protected contour | Inside protected contour | Outside protected contour | Outside protected contour (with d > d[0]) | Inside protected contour | Outside protected contour | Inside protected contour | Outside protected contour |
| device operation on TV channel N-3 | 6 dBW | Not allowed (adjacent band) | 6 dBW | 6 dBW | 6 dBW | 6 dBW | f(-6) dBW | 6 dBW |
| device operation on TV channel N-2 | 6 dBW | Not allowed | 6 dBW | 6 dBW | 6 dBW | 6 dBW | f(-5) dBW | 6 dBW |
| device operation on TV channel N-1 | 6 dBW | Not allowed (adjacent band) | 6 dBW | 6 dBW | 6 dBW | 6 dBW | f(-4) dBW | 6 dBW |
| device operation on TV channel N | 6 dBW | f(2) dBW | 6 dBW | 6 dBW | 6 dBW | 6 dBW | f(-3) dBW | 6 dBW |
| device operation on TV channel N+1 | 6 dBW | f(3) dBW | 6 dBW | 6 dBW | 6 dBW | 6 dBW | f(-2) dBW | 6 dBW |
| device operation on TV channel N+2 | 6 dBW | f(4) dBW | 6 dBW | 6 dBW | 6 dBW | 6 dBW | Not allowed (adjacent band) | 6 dBW |
| device operation on TV channel N+3 | 6 dBW | f(5) dBW | 6 dBW | 6 dBW | 6 dBW | 6 dBW | Not allowed | 6 dBW |
| device operation on TV channel N+4 | 6 dBW | f(6) dBW | 6 dBW | 6 dBW | 6 dBW | 6 dBW | Not allowed (adjacent band) | 6 dBW |

Once Table 9 has been completely filled for one WRAN device, the next step is to determine the most constraining power limit per row. Each row corresponds to a hypothesis of transmission from the device in a given TV channel. In the example of Table 9, the most constraining power limits are highlighted in bold font. It was assumed that d is larger than keep-out distances d(0), d(-1) and d(+1), and that f(-2)<f(-3) and f(2)<f(3). The final values are reported in Table 10.

- — Individual WRAN device maximum transmitted EIRP from all TV operations

| WRAN device operation on TV channel | Maximum transmit EIRP |
| --- | --- |
| N-3 | -100 dBW |
| N-2 | -100 dBW |
| N-1 | -100 dBW |
| N | f(+2) dBW |
| N+1 | f(-2) dBW |
| N+2 | -100 dBW |
| N+3 | -100 dBW |
| N+4 | -100 dBW |

- Interference

  - Interference

    - Interference Mechanisms

      - Out-of-Band Emissions

It is assumed that the level of out-of-band emissions from a WRAN CPE is such that it will result in a TV receiver desensitization of no more than 1 dB in the most demanding receiving condition - at the edge of the protected contour.  A 41 dB(uV/m) field strength as required for DTV reception at the noise limited contour corresponds to a signal level of –83.9 dBm at the input of the DTV receiver (depends on numerous factors – need to be more specific /Winston/ based on the OET 69 Planning Factors).  With a required SNR of 15.2 dB and a margin of 5.9 dB to limit the impact to 1 dB desensitization, this results in a level of –105 dBm.   A 4 W CPE using vertical polarization and located at 10 m from a DTV receiving installation produces a –18 dBm at the input of the DTV receiver.  1 dB receiver desensitization will be achieved with a –18 – 105 = 87 dB out-of-band suppression at the output of the CPE.

Since the minimum field strength at the Grade B contour of the NTSC coverage is 64 dB(uV/m), the out-of-band rejection requirement necessary for DTV will be the driving factor.

Values are needed for other TV systems. [This section may not be necessary.  We do not need to justify the emissions mask in the standard.  If this section is needed, a methodology to calculate may be more appropriate.  This section might be attempting to state the necessary out-of-band rejection of a CPE to operate.]

      - Third-Order Intermodulation

Special attention will be needed from the WRAN operator to avoid third-order intermodulation products from two  signals present at the TV receiver that fall in the selected channel due to non-linearities in the TV receiver front-end.  From the geolocation capabilities of the WRAN system, the availability of the incumbent database and the results of RF sensing, the WRAN operator will be in a position to know within which protected contours each of his CPEs are located.  A simple calculation considering the ‘2A-B’ intermodulation scenario will indicate which channels would generate an intermod component falling on a protected channel at the nearby TV receivers.  The use of these channels should be avoided for the given CPE.  If the CPE needs to use the channel because of the lack of other available channels, an additional 15 dB [tbd] reduction in the maximum EIRP level of the TPC range will need to be applied.  A representative diagram might help demonstrate.

# Coexistence

  - Coexistence

# Coexistence Policy

[If the systems are truly ‘unlicensed’, I am not sure that this will really do the job if coexistence among unlicensed systems is not fully taken care of since any other unlicensed system could come in and disturb the well planned WRAN system.  This needs further discussion. The intent is that the standard takes care of self-coexistence by sharing the capacity among the overlapping base stations.  Since the implementation of the BSs is to be done professionally, the consideration of service continuity and reliability will have taken place at the planning stage of the new BS wrt to incumbent protection as well as self-coexistence with already existing WRAN systems.][BS should be designed to operate as both unlicensed and licensed] [Even better, Part 91 as used for the 3.6 GHz band: non-exclusive light licensed.]

# Deployment

This section is intended to be used as a guide by the owner of the WRAN service and the surveyor who selects the area of deployment for the WRAN system.

The WRAN systems are to provide broadband access services while protecting the incumbent services in the TV bands from interference.  Although WRAN systems employ interference mitigation techniques such as geo-location technology paired with an incumbent database containing TV station, Part 74, and WRAN information, sensing, and dynamic frequency selection (DFS), careful planning of the service will be needed to avoid excessive future service disruption.  Protection margins should be used in the planning of the systems to avoid future unexpected situations.  Service continuity and reliability will indeed depend on the quality of this initial planning.

# Coverage and Interference Prediction Model

Although the original FCC coverage curves contained in Part 73 are used to define the TV protected contours and the Recommendation ITU-R P.1546 propagation model was used in the system studies for the development of the 802.22 standard, a more precise coverage and interference model should be used in the deployment planning of WRAN systems.  More precise coverage computer predictions will result in accurate coverage areas and will identify complex interference situations that need to be avoided.

      - Characteristics

The coverage and interference prediction model that should be utilized in the planning of a WRAN service should incorporate at least the following:

- Point-to-point propagation model.

- Desired to undesired protection ratios (D/U) for the various channel relationships considered should be selected from a scientific reference that provides results from analysis examining the specific modulations used by both the desired and undesired signals.

- Transmitting and receive antenna pattern characteristics for the incumbent, BS, and CPEs.

- Height of the transmitting antenna Above Ground Level (AGL).

- 30 meter terrain data.

- Population from current census data.

- K-factor and other atmospheric effects on signal fading.

- Ground cover (wetlands, desert, tropical, etc)

- Ground clutter (trees, urban buildings, etc).

- Long term fading effects, such as those described in Environmental Science Services Administration (ESSA) Technical Report ESSA Research Laboratories 79-Institute for Telecommunication Sciences 67.

- Surface of the Earth electrical characteristics, such as those described in the Recommendation ITU-R P.527-3.

      - Statistics

All coverage and interference prediction is a statistical exercise.  Depending on the statistics, the coverage and interference prediction model will return different results.  Coverage and interference prediction is statistically multidimensional and is typically represented in terms of percent of locations and in percent of time.  When performing coverage and interference prediction, a system planner should use the following statistical information.

- 802.22 coverage simulations should use 99.9 % time availability.

- Digital television (DTV) interference analysis should use 90 % and 10 % for time availability for desired and undesired signals, respectively.

- Analog interference analysis desired and undesired signals use 50 % and 10 %, respectively for time availability.

The protected contours are defined by 50% of locations and 90% of time.

# Selection of Deployment Location

Proposers should be mindful in terms of system architectural considerations that, in the interest of coexistence and avoidance of interference to incumbent TV operations, WRAN BSs will have to be located outside the keep-out distance that is beyond co- and adjacent channel protected contours of TV stations as specified in Section 2.  The keep-out distances that have been provided in this document are calculated by making certain assumptions.  Because of these assumptions, deploying a WRAN system according to the specified keep-out distance will neither guarantee avoidance of interference to incumbents nor from incumbents into the WRAN.  Fortunately, during the planning stage of the system, there is an opportunity to identify a set of areas of interest for potential deployment.  Specific data that would influence RF propagation, including the difference in height between transmitter and receiver, terrain, ground cover, and atmospheric characteristics, could be collected for one of these deployment areas of interest.  A system planner should utilize an RF propagation prediction tool (as described in section 4.1.1.1) that accounts for this specific data that more accurately describes the characteristics of an interested deployment area.  Proper engineering utilizing a more accurate RF propagation prediction tool might result in keep-out distances that are larger than the values given in Section 2.  The system planner should respect the larger keep-out distance to avoid causing interference to an incumbent receiver located inside of a protected contour.

In the case where the WRAN system is to operate on second adjacent channel relationships or beyond relative to a local TV operation, the WRAN BS and CPEs will be allowed to operate inside the protected contour.  The BS will need to be located at some minimum distance from the closest TV receiver to avoid interference.  The reference minimum distance from a TV receiver is assumed to be 16 m for the CPEs, which is based on the reference minimum separation for an outdoor transmitter as stated by US regulations.  The allowed maximum EIRP of the WRAN transmit devices will also need to be scaled according to the EIRP profile as defined in section 3 to avoid taboo channel and third-order intermodulation interference.

It should be realized that, although the keep-out distances for co-channel and adjacent channel operation were calculated from the consideration of interference for the WRAN terminals into DTV or analog TV reception, the interference in the reverse direction will also need to be considered by the WRAN system planner.

To demonstrate the implications of receiving harmful interference by a TV station, Table 11 gives the minimum distances that will result in a 1 dB WRAN receiver desensitization.

Table 11:  Distance for the DTV transmit station

| DTV into WRAN | Desens. | CPE | BS |
| --- | --- | --- | --- |
| Co-channel, N | 1 dB | 406 km | TBD for 75m |
| Adjacent channels, N±1 | 1 dB | 91.2 km | TBD for 75 m |
| N±2 and beyond | 1 dB | 6.6 km | TBD for 75 m |

Note: ITU-R Rec. P.1546, 1 MW ERP and 300 m HAAT DTV station

As can be seen from the large distances in comparison to the values given in section 2, the co-channel interference from the DTV signal into WRAN reception is much more severe than in the WRAN into DTV direction due to the larger power of the DTV transmitter.  Because an incumbent transmitter can transmit at such a high power, it may cause hamful interference into a WRAN system over a radius of about 400 km or greater.  However, the WRAN system planner may decide to reduce the extent of his WRAN coverage and allow his receivers to suffer more than the assumed 1 dB desensitization.  BS and CPEs could then be located closer than the distances given in Table 11. Nonetheless, the system planner will still have to meet the keep-out distances specified in Section 2.

In the case of the adjacent channel, the WRAN interference into DTV reception might be the constraining case.  The minimum distance needed to prevent DTV interference into the WRAN service may be less than the distance to the protected contour depending on the relative selectivity of the WRAN receiver.  Table 12 (missing table) also indicates that CPEs and BSs operating on N+/-2 and beyond that are located too close to a TV transmitter may suffer desensitization from the out-of-band emissions from the TV transmissions.  Again, the WRAN system planner may decide to reduce the extent of his WRAN coverage and allow his receivers to suffer more that the 1 dB desensitization.  The BS and CPEs could then be located closer than the minimum distances given in Table 12 (missing table).  It is likely, however, that considering the extent of the out-of-band rejection of the DTV transmitter in these alternate channels, the WRAN receivers will be saturated long before this desensitization due to the DTV out-of-band signal take place.  [some more calculations needed here.]

The coupling between WRAN and TV reception for alternate channel operation should be considered in terms of absolute levels of power at the input of the TV receiver rather than an additional discrimination to be added to the TV receive antenna gain since there will be a tendency for the TV receiving installation to use a lower gain receive antenna when the local TV signal field strength is higher.  The important point is to keep the WRAN power level at the input of the TV receiver below the saturation level of the TV receiver. Comes from Docs. 230, 231, 232

Once Part 74 operation and its location has been identified 4 Watt CPEs will have to avoid the occupied channel, be located beyond a radius of 4 km from the Part 74 receiver, or reduce their maximum TPC limit accordingly.  The amount of tapering in dB from the maximum allowed EIRP will be defined as follows from the fraction of the actual distance to the [protected contour (have to find a new term relating to Part 74)] to the distance indicated in the table:

Tapering = Path loss exponent * 10*log(actual distance/distance in the table)   (dB)

where:	path loss exponent = 3.0   [tbd]

# Installation

After the WRAN system planner completes the suggested deployment process described above in Section 7, the professional installer should follow the suggestions provided in this section for a proper installation.  This section also describes the characteristics of the WRAN system devices that should be installed.    The 802.22 standard was designed while assuming the WRAN system characteristics provided in this section.

# System

The WRAN system is comprised of a BS and CPEs.

The chassis radiation for all system devices is assumed to be and should be insignificant (e.g. <= -20 dB) compared to radiation from the intended path via the antenna.

# Base Station

It is an IEEE 802.22 requirement that BSs shall be professionally installed and maintained.

"Professional installation" means that the installation must be supervised or inspected by a trained, competent professional, such as e.g. a NARTE Certified EMC Engineer, an SBE Certified Professional Broadcast Engineer, or a Registered Professional Engineer.

The professionally installed BS shall determine its geographic location automatically using satellite-based geo-location technology and the available TV channel list for its location by interfacing with the database service as described in Section 4.

The installing party shall configure the BS to operate only on a channel that is available according to the database service and is not occupied according to the spectrum sensing capability.

# Antennas

Base station antenna systems shall be designed and installed to support the requirements of 15.1.1.7, 15.1.6, and 15.1.7 in the Functional Requirements Document (FRD).  During the design of the 802.22 standard, it was assumed that the BS antennas are installed 75 m AGL.

# Transmit/Receive Antenna

# Gain

It is assumed that the BS transmit/receive antenna is Omni-directional with 12 dB of gain elevation gain.

# Polarization

It is assumed that the antenna is oriented orthogonal to the orientation of the nominal television receive antennas in the area in the polarization plane within a tolerance of 3 deg.  In other words, if the TV receiver antenna uses horizontal polarization, the BS transmit/receive antenna should use vertical polarization.

      - Sensing Antenna

        - Gain

The BS shall sense licensed transmissions using a sense antenna with a gain of 0 dBi or greater (where all losses between the antenna and the input to the receiver are included) in any azimuthal direction and polarization.

        - Polarization

The antenna has to have similar gain in both horizontal and vertical polarizations.

# RF Safety Limits

The RF safety limits that are required in the US are provided by FCC OET Bulletin 65, Annex 2, Table 2:

Table : RF Safety Limits According to US regulations

# Cable Protection

Cable protection that is required in the US is provided by FCC OET Bulletin 65, Annex 2, Table 2:

Table : Cable Protection Requirement According to US Regulations

# Customer Premises Equipment (CPE)

CPEs MAY be user installable (plug and play) but operators MAY choose to provide for professional installation.

For the system to operate correctly, proper CPE installations SHALL be verifiable.  Means MAY be provided to allow verification of proper CPE installation remotely from the base station. Automatic system verification of correct installation is beneficial.  CPEs shall not transmit absent reliable verification.  In the absence of any automated verification method, visual verification of proper installation will be necessary.

# Antennas

Sensing thresholds specified in section 15.1.1.7. in the FRD are based on the assumptions that the CPEs are permanently installed outdoors on a fixed structure and are properly aligned.

CPE antenna systems SHALL support the requirements of 15.1.1.7 of FRD (sensing thresholds), 15.1.6 of FRD (maximum transmitted EIRP) and 15.1.7 of FRD (out of band emissions).

Verification that the CPE antennas are permanently installed outdoors on a fixed structure and are properly aligned is required.

Verification that the CPE antennas remain in the location where they were installed is required.

# Transmit Antenna

A minimum separation distance of 10 m between the CPE transmit antenna and DTV receiving antennas is required.

# Installation Height

The CPE transmit antenna SHALL be assumed to be installed outdoors at a nominal height of 10 meters above ground level and co-located with the sense antenna.

The CPE transmit antenna needs to be installed outside to minimize the blockage, minimize the TPC level and maximize the transmit capacity (since more efficient modulation can be used) as much as possible.

        - Pattern

          - Gain

The transmit antenna of the CPE will most likely use a directional antenna pattern.  Main beam coupling may exist between the antennas for channels N±2 and beyond inside the noise-limited protected contour.

          - Orientation

Verification that the CPE transmit antenna is oriented properly to minimize the CPE transmit power resulting from the use of local TPC is required.

The antenna needs to be oriented toward the base station of the selected service provider.  It is possible that if a new service provider is selected for technical or other reasons, the antenna may need to be re-oriented toward the new service provider antenna.  Proper orientation will allow the TPC level to be kept to its minimum and or maximize the transmit capacity (since more efficient modulation can be used).

          - Front-to-Back

In the case of co-channel and first adjacent channel operation, the CPE transmit antenna and the TV receive antenna are assumed to be looking away from each other since the BS has to be located at a certain distance outside the noise-limited protected contour of the TV station.  The back-lobe rejection of both antennas can lower the potential of interference.

          - Polarization

WRAN CPE transmit antennas SHOULD be polarized orthogonally to nearby DTV receive antennas to provide

additional isolation through cross-polarization discrimination.

For channels N±2 and beyond, polarization discrimination is the only way to increase isolation based on the fact that the CPE transmit antenna and the DTV receive antenna will be orthogonally polarized.

[Some antenna polarization discrimination MAY be assumed.]

The antenna has to be oriented vertically in the polarization plane within a tolerance of 10 deg.  In order to maximize the polarization discrimination toward a nearby TV receiving installation, it is preferable to mount the antenna at the same height as the nearby TV receive antenna.

        - Tamper Resistant

Provisions SHALL be made to prevent the use of antenna equipment that could impair or defeat the requirements of the preceding two paragraphs and proposers SHALL elaborate their proposed approaches. [Edit this paragraph].

      - Receive Antenna

        - Installation Height

        - Pattern

          - Gain

          - Orientation

          - Front-to-Back

          - Polarization

The antenna has to be oriented orthogonal to the orientation of the nominal television receive antennas in the area in the polarization plane within a tolerance of 10 deg.

      - Sense Antenna

        - Installation Height

The CPE sense antenna SHALL be assumed to be installed outdoors at a nominal height of 10 meters above ground level and co-located with the transmit antenna.

Special attention SHALL be given to the installation of the outdoor antenna to make sure that the sensing capabilities of the CPE are not impacted by local obstructions.

        - Pattern

It is assumed that the sense antenna will be omni-directional.

The sensing antenna has to be omnidirectional in the horizontal plane with a maximum gain variation of 1 dB.

          - Gain

CPEs SHALL sense licensed transmissions using an antenna with 0 dBi gain or greater (where all losses between the antenna and the input to the receiver are included) in any azimuthal direction and polarization.

The sensing antenna gain has to be at least 0 dBi.

          - Orientation

          - Front-to-Back

In the case of co-channel and first adjacent channel operation, both antennas are assumed to be looking away from each other since the base station has to be located at a certain distance outside the noise-limited protected contour of the DTV station as indicated in the previous section.  The back-lobe rejection of both antennas can therefore be relied upon. [Edit this paragraph].

          - Polarization

WRAN CPE transmit antennas SHOULD be polarized orthogonally to nearby DTV receive antennas to provide

additional isolation through cross-polarization discrimination.

For channels N±2 and beyond, polarization discrimination is the only way to increase isolation based on the fact that the CPE transmit antenna and the DTV receive antenna will be orthogonally polarized.

[Some antenna polarization discrimination MAY be assumed.

The possibility of some signal depolarization between the two antennas SHOULD be considered as well.]

The sensing antenna needs to receive equally on both polarizations for TV sensing and Part 74 wireless microphones.

        - Tamper Resistant

Provisions SHALL be made to prevent the use of antenna equipment that could impair or defeat the requirements of the preceding two paragraphs.

# RF Safety Limit

FCC OET Bulletin 65, Annex 2, Table 2:

# Cable Protection

FCC OET Bulletin 65, Annex 2, Table 2:

# RF Receive Chain  (The following might instead be placed in an annex for assumption reference.)

# Noise Figure

2.5 dB assumed

# Filtering Loss

1 dB + 0.5 dB antenna coupling loss

# LNA Gain

20 dB

# Cable Loss

3.5 dB + 0.2 dB connector loss

# Receiver Noise Figure

8dB

# Overall Figure of Merit: G/T

-17 dB (1/K)

# Operation

- System

- Continuous Monitoring

- Software

Devices must include features to ensure that only software that is approved with a device can be loaded into this device, and the software may not allow the user to operate the device with parameters outside those that were approved.  “Software” in this context includes the software that selects a device’s operating frequency, determines a device’s geographic location, identifies TV channels that are vacant, and accesses information in a related database.

- Distributed Sensing

Sensing reliability is improved by the collaborative sensing from a number of well selected CPEs.  Information  on the geo-location of the CPE relative to the presumed origin of the signal will be available at the BS.  [The sensing scheme needs to be considered more as a component of a more complex distributed sensing system than an independent device that, on its own, needs to meet the sensing requirement.]

- Base Station

BSs SHALL be professionally operated.

Different classes of BSs with different conditions of operation such as transmit power levels, etc., are supported where permitted by regulatory domains.

    - Requests to the CPE

The BS SHALL be able to request local interference sensing and channel scanning from a CPE at any time.  When a new CPE attempts to register with a BS,, the BS shall verify that the local interference potential from the CPE would allow the CPE to access the network.

    - Maximum Effective Isotropic Radiated Power (EIRP)

The default class of base station SHALL operate at a maximum of 4 W EIRP.

    - Duty Cycle

Base stations SHALL support 100% transmit duty cycle at rated power.

    - Monitoring

The BS or its operator SHALL periodically access a TV channel availability database derived from current FCC data and computational software to ensure that occupied channels by TV remain unused.

      - Manual Monitoring

      - Automatic Monitoring

Information will be sent and collated at the base station where mapping of the interference situation will be made based on the sensing information from the CPEs and according to their respective geographic coordinates.  Solutions to avoid interference will be deduced at the base station and measures will be signalled to the CPEs such as channel change or power reduction.

Sensing reliability is improved by the collaborative sensing from a number of well selected CPEs based on their geo-location relative to the presumed origin of the signal since all this information will be available at the base station.  The sensing scheme needs to be considered more as a component of a more complex distributed sensing system than an independent device that, on its own, needs to meet the sensing requirement.[This is redundant].

  - Customer Premises Equipment

    - Maximum Effective Isotropic Radiated Power

4 Watt maximum CPE transmit EIRP.

Accordingly, the maximum transmitted CPE EIRP will have to be limited (by capping the TPC range) to a value that is related to the channel separation between WRAN and TV operations.

- Duty Cycle

CPEs SHALL support 100% transmit duty cycle at rated power to support cases where maximum upstream throughput is required from a terminal at the edge of the coverage area.

Annex 1

Planning of Service Procedure Example [Need to also describe a simpler methodology for deployment also (involving a simpler propagation model, F(50,50) and separation distances).***]

- Acquire the necessary tools and information.

  - WRAN BS operating parameters

    - Antenna characteristics (OMNI-directional dipole is assumed)

    - Intended height of the center of radiation (AGL) (75 m is assumed)

    - Operating channel

  - Incumbent receiver characteristics

Table : Incumbent Receiver Characteristics

|  | Low VHF | High VHF | UHF |
| --- | --- | --- | --- |
| Lead in loss (dB) | 1 | 2 | 4 |
| Front to back ratio (dB) | 10 | 12 | 14 |
| Antenna gain (dBi) | 6.15 | 8.15 | 12.15 |

    - Receivers are outdoors at 9 m height.

    - Receiver antenna patterns use a cosine exponent equal to 4.  If the azimuth offset between the WRAN transmission and the main lobe of the receiver antenna azimuth pattern is less than 90 degrees, the pattern contribution returned is the maximum of either the cosine of the azimuth offset raised by the pattern exponent or the front-to-back ratio.  If the azimuth offset is greater than 90 degrees, the pattern contribution is equal to the front-to-back ratio.

  - Incumbent database

    - TV transmitter operating parameters are from the 12/20/06 FCC CDBS database.

    - Only co-channel and adjacent channel interference was considered (“taboo” channel relationships were not considered).

    - Only high power TV stations were considered (low power or “class A” TV stations were not considered).

  - Propagation analysis tool

    - 8-Vestigial Side-Band (8-VSB) Advanced Television Systems Committee modulation for 802.22 systems.

    - Desired to undesired ratios (D/U) from the Federal Communications Commission (FCC) Office of Engineering and Technology Bulletin 69.

    - 30 meter United States Geological Survey terrain data.

    - Population from 2000 Topographically Integrated Geographic Encoding and Referencing system census data.

    - Terrain Integrated Rough Earth Model propagation prediction model (TIREM).

    - Long Term Fading, Environmental Science Services Administration (ESSA) Technical Report ESSA Research Laboratories 79-Institute for Telecommunication Sciences 67.

    - Surface of the Earth Electrical Characteristics from International Telecommunication Union Recommendation 527-3.

    - 802.22 coverage simulations use 99.9 % time availability.

    - Digital television (DTV) interference analysis, desired and undesired signals use 90 % and 10 %, respectively for time availability.

    - Analog interference analysis desired and undesired signals use 50 % and 10 %, respectively for time availability.

    - Other assumptions regarding transmitter and receiver parameters for link budgets are from 802.22 document number 22-04-0002-10-0000.

- Identify the area of interest to deploy the WRAN service.

In this example, a location in Linden, TN was chosen (35.37°, -87.50°)

- Determine a useable channel by plotting the co- and adjacent channel incumbent service noise-limited contours in the area of interest.

Note: The figures above do not accurately represent final post-transition DTV service or associated service contours.

Current co- and adjacent channel DTV noise-limited contours were plotted in the Linden, TN area for channels 25 through 30.  In the figures above, the black circle is a 40 km radius circle with center at a site in Linden, TN of high terrain elevation.  The red contours are the co- and adjacent channel DTV noise-limited contours.  It can be seen in the figures above that channels 26 through 28 are unusable for the Linden, TN site.  It appears that channels 25, 29, and 30 are possible because the selected site would not be contained within either a co- or adjacent channel DTV noise-limited contour.

For this example channel 30 is selected.

- Analyze the coverage achieved by the WRAN system.

Note: The figure above is still being developed.  Coverage shown may not be accurate.

The 4 W transmission from the WRAN BS does a good job serving the 40 km circle with a -102 dBm signal or greater.  Hovever the interference caused to this transmission from the nearby co- and adjacent channel higher power DTV transmissions is significant.  The interference is represented by the gray color in the figure above.

- Analyze the interference potential into the planned WRAN system from the nearby DTV service.

Note: The figure above is still being developed.  The interference shown may not be accurate.

The co- and adjacent channel interference into the WRAN BS from the nearby DTV transmissions is once again depicted in the figure above.  In this figure the interference is color coded to indicate the particular station that is causing the particular interference.

References:

- G. Chouinard, “WRAN Reference Model”, IEEE 802.22-04-0002-14-0000-WRAN_Reference_Model.xls, May 2006.

- G. Chouinard, “OFDMA Parameters”, IEEE 802.22-06-0264-07-0000-ofdma_parameters.xls, June 2006, https://mentor.ieee.org/802.22/file/06/22-06-0264-07-0000-ofdma-parameters.xls

- K. Salehian, Y. Wu, G. Gagnon, "ATSC-DTV Receiver Performance in the Presence of Adjacent/Taboo Channels Interference, and Antenna Cross-Polarization Discrimination Tests", IEEE 57th Annual Broadcast Symposium, Washington DC, October 31 to November 2, 2007.

- US Federal Communications Commission (FCC) Rules and Regulations, Title 47 Code of Federal Regulations, Part 73 Radio Broadcast Services, Section 73.625(b) DTV Coverage of Principal Community and Antenna System – Determining Coverage, http://edocket.access.gpo.gov/cfr_2007/octqtr/47cfr73.625.htm.

- US Federal Communications Commission (FCC) Rules and Regulations, Title 47 Code of Federal Regulations, Part 73 Radio Broadcast Services, Section 73.684 Prediction of Coverage, http://edocket.access.gpo.gov/cfr_2007/octqtr/47cfr73.684.htm.

- Institute of Electrical and Electronics Engineers (IEEE) LAN/MAN Standards Committee 802 Working Group Draft Standard for Wireless Regional Area Networks (WRAN) Part 22: Cognitive Wireless RAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications: Policies and Procedures for Operation in the TV Bands, 2006.

National Association of Radio Telecommunications Engineers

Electro Magnetic Compatibility

Society of Broadcast Engineers