IEEE P802.22
Wireless RANs

| Draft Recommended Practice |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-06-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Winston Caldwell | FOX | 10201 W.Pico Blvd Los Angeles, CA 90064 | 310-369-4367 | Winston.caldwell@fox.com |
| Gerald Chouinard | Communications Research Center | 3701 Carling Avenue Ottawa, Canada K2H-8S2 | 613-998-2500 | Gerald.chouinard@crc.ca |

Abstract

This document will serve as the working document toward the draft 802.22.2 Recommended Practice.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Interference

# Interference into Incumbent Services

Recommendation 1-1-1

Do not cause harmful interference into the protected licensed services.

Recommendation 1-1-2

Do not allow the received power from a WRAN device transmission to exceed -105 dBm at the input of a DTV receiver on a UHF TV channel.

Recommendation 1-1-3

Avoid third-order intermodulation products from two signals present at the TV receiver.

Recommendation 1-1-4

Avoid using channels which would generate an intermod component considering the ‘2A-B’ intermodulation scenario.

Recommendation 1-1-5

Apply an additional 15 dB reduction to the maximum EIRP level of the TPC range if the CPE needs to use the channel because of the lack of other available channels.

# Interference from Incumbent Services

Recommendation 1-2-1

Accept and be capable of tolerating any and all interference that is caused by the licensed services.

# Interference with other Services

Recommendation 1-3-1

Do not willfully cause interference into other services.

Recommendation 1-3-2

Filter the emissions that are outside of the intended channel so that the out-of-band suppression is at least 87 dB.

# Coexistence

# Coexistence

# Coexistence Policy

# Deployment

This section is intended to be used as a guide by the owner of the WRAN service and the surveyor who selects the area of deployment for the WRAN system.

Recommendation 3-1

Plan the WRAN service carefully to avoid excessive future disruption.

Recommendation 3-2

Use protection margins in the service planning to avoid future unexpected situations.

  - Coverage and Interference Prediction Model

Recommendation 3-1-1

Use a more precise coverage and interference model than the FCC Part 73 coverage curves or the ITU-R Recommendation P.1546 for deployment planning of WRAN systems.

    - Characteristics

Recommendation 3-1-1-1

Use a coverage and interference prediction model for the planning of a WRAN service that incorporates at least the following:

- Point-to-point propagation model.

- Desired to undesired protection ratios (D/U) for the various channel relationships considered should be selected from a scientific reference that provides results from analysis examining the specific modulations used by both the desired and undesired signals.

- Transmitting and receive antenna pattern characteristics for the incumbent, BS, and CPEs.

- Height of the transmitting antenna Above Ground Level (AGL).

- 30 meter terrain data.

- Population from current census data.

- K-factor and other atmospheric effects on signal fading.

- Ground cover (wetlands, desert, tropical, etc)

- Ground clutter (trees, urban buildings, etc).

- Long term fading effects, such as those described in Environmental Science Services Administration (ESSA) Technical Report ESSA Research Laboratories 79-Institute for Telecommunication Sciences 67.

- Surface of the Earth electrical characteristics, such as those described in the Recommendation ITU-R P.527-3.

    - Statistics

Recommendation 3-1-2-1

Use the following statistical information when performing a coverage and interference prediction analysis:

- 802.22 coverage simulations should use 99.9 % time availability.

- Digital television (DTV) interference analysis should use 90 % and 10 % for time availability for desired and undesired signals, respectively and 50 % for location availability.

- Analog interference analysis desired and undesired signals use 50 % and 10 %, respectively for time availability and 50 % for location availability.

  - Selection of Deployment Location

Recommendation 3-2-1

Locate WRAN devices outside the keep-out distance that is beyond co- and adjacent channel protected contours of TV stations that is specified by the local regulations.

Recommendation 3-2-2

Collect specific data that would influence RF propagation, including the difference in height between transmitter and receiver, terrain, ground cover, and atmospheric characteristics for the deployment area of interest.

Recommendation 3-2-3

Use the data collected in Recommendation 3-2-2 as input into a RF propagation prediction tool with the characteristics listed in Recommedation 3-1-1-1.

Recommendation 3-2-4

Respect the greater keep-out distance between the one resulting from Recommendation 3-2-1 and the one resulting from Recommendation 3-2-3.

Recommendation 3-2-5

Locate a WRAN device at a 16 m minimum distance from the closest TV receiver to avoid causing taboo and third-order intermodulation interference while operating on second adjacent channel relationships and beyond inside the protected contour.

Recommendation 3-2-6

Avoid a 1 dB WRAN receiver desensitization as caused by a high power TV transmission by adhering to the minimum separation distances as provided in the following table:

Table 1:  Distance from a DTV transmit station

| DTV into WRAN | Desens. | CPE | BS |
| --- | --- | --- | --- |
| Co-channel, N | 1 dB | 406 km | TBD for 75m |
| Adjacent channels, N±1 | 1 dB | 91.2 km | TBD for 75 m |
| N±2 and beyond | 1 dB | 6.6 km | TBD for 75 m |

Note: ITU-R Rec. P.1546, 1 MW ERP and 300 m HAAT DTV station

In the case of the adjacent channel, the WRAN interference into DTV reception might be the constraining case.  The minimum distance needed to prevent DTV interference into the WRAN service may be less than the distance to the protected contour depending on the relative selectivity of the WRAN receiver.  Table 12 (missing table) also indicates that CPEs and BSs operating on N+/-2 and beyond that are located too close to a TV transmitter may suffer desensitization from the out-of-band emissions from the TV transmissions.  Again, the WRAN system planner may decide to reduce the extent of his WRAN coverage and allow his receivers to suffer more that the 1 dB desensitization.  The BS and CPEs could then be located closer than the minimum distances given in Table 12 (missing table).  It is likely, however, that considering the extent of the out-of-band rejection of the DTV transmitter in these alternate channels, the WRAN receivers will be saturated long before this desensitization due to the DTV out-of-band signal take place.  [some more calculations needed here.]

Recommendation 3-2-7

Keep the WRAN power level at the input of the TV receiver below the saturation level of the TV receiver.

Recommendation 3-2-8

Avoid a channel that is occupied by Part 74 operation, be located beyond a radius of 4 km from the Part 74 receiver, or reduce the maximum TPC limit according to the following equation:

Tapering = Path loss exponent * 10*log(actual distance/distance in the table)   (dB)

where:	path loss exponent = 3.0   [tbd]

# Installation

After the WRAN system planner completes the suggested deployment process described above in Section 7, the professional installer should follow the suggestions provided in this section for a proper installation.  This section also describes the characteristics of the WRAN system devices that should be installed.    The 802.22 standard was designed while assuming the WRAN system characteristics provided in this section.

# System

Recommendation 4-1-1

Comprise the WRAN system of a BS and CPEs.

Recommendation 4-1-2

Do not install equipment with physical damage or loose connectors.

Recommendation 4-1-3

Utilize appropriate weather stripping tape to any connector that might be exposed to the outdoor environment.

    - Cable Protection

Recommendation 4-1-1-1

Install transmitting equipment at least 10 m away from cable TV equipment.

Recommendation 4-1-1-2

Check nearby TV receivers to ensure that the installed WRAN transmission does not cause interference to the TV service.

Recommendation 4-1-1-3

Ensure that metallic objects or reflectors are not close to or in the path of the WRAN transmitting antenna.

# Base Station

Recommendation 4-2-1

Perform and inspect the installation of BSs by a trained, competent professional, such as e.g. a NARTE Certified EMC Engineer, an SBE Certified Professional Broadcast Engineer, or a Registered Professional Engineer.

Recommendation 4-2-2

Ensure that the BS does not transmit unless communication between the BS and the database is established.

Recommendation 4-2-3

Install the BS antennas in a fixed position, outdoor, and in the clear.

Recommendation 4-2-4

Install the BS antenna at a height that satisfies the local regulations.

# Transmit/Receive Antenna

Recommendation 4-2-1-1

Use an Omni-directional BS transmit/receive antenna with 8 dBi of elevation gain.

Recommendation 4-2-1-2

Orient the BS transmit/receive antenna orthogonal to the orientation of the nominal television receive antennas in the area in the polarization plane within a tolerance of 3 deg.

    - Sensing Antenna

Recommendation 4-2-2-1

Use a BS sensing antenna with a gain of 0 dBi or greater (where all losses between the antenna and the input to the receiver are included) in any azimuthal direction and polarization.

Recommendation 4-2-2-2

If the sensing antenna provides less gain than 0 dBi in any azimuthal direction or polarization, adjust the sensing threshold to be more sensitive to account for the dB loss in antenna gain.

Recommendation 4-2-2-3

Use  a BS sensing antenna that has similar gain in both horizontal and vertical polarizations.

# Customer Premises Equipment (CPE)

Recommendation 4-2-1

Perform and inspect the installation of CPEs by a trained, competent professional, such as e.g. a NARTE Certified EMC Engineer, an SBE Certified Professional Broadcast Engineer, or a Registered Professional Engineer.

Recommendation 4-2-2

Prevent CPE transmissions absent reliable verification of the Recommendations 4-1-1-1, 4-1-1-2, 4-1-1-3, and 4-2-1.

Recommendation 4-2-3

Permanently install the CPE sensing antenna outdoors, in the clear, on a fixed structure, and properly aligned.

Recommendation 4-2-4

Verify that the CPE sensing antenna remains in the location where it was installed.

Recommendation 4-2-5

Employ provisions to prevent the use of antenna equipment that could impair or defeat Recommendations 4-2-1, 4-2-2, 4-2-3, and 4-2-4.

# Transmit/Receive Antenna

Recommendation 4-3-1-1

Enforce a minimum separation distance of 10 m between the CPE transmit/receive antenna and DTV receiving antennas.

# Installation Height

Recommendation 4-3-1-1-1

Install the CPE transmit/receive antenna outdoors at a nominal height of 10 meters above ground level and co-located with the CPE sensing antenna.

      - Pattern

Recommendation 4-3-1-2-1

Do not allow

the gain differential between the transmit and receive patterns to exceed 0.5 dB.

Recommendation 4-3-1-2-2

Orient the CPE transmit/receive antenna toward the base station of the selected service provider.

Recommendation 4-3-1-2-3

Re-orient the CPE transmit/receive antenna toward the new service provider if a new service provider is selected

.

Recommendation 4-3-1-2-4

Verify that the CPE transmit/receive antenna is oriented properly.

Recommendation 4-3-1-2-5

Further adjust the CPE transmit/receive antenna to minimize the gain in the direction of an interfering source.

Recommendation 4-3-1-2-6

Orient the CPE transmit/receive antenna so that it and the TV receive antenna are looking away from each other.

Recommendation 4-3-1-2-7

Use CPE transmit/receive antennas that are polarized orthogonally to nearby DTV receive antennas.

Recommendation 4-3-1-2-8

Ensure that the CPE transmit/receive antenna is oriented in the polarization plane within a tolerance of 10 deg.

Recommendation 4-3-1-2-9

Mount the CPE transmit/receive antenna at the same height as the nearby TV receive antenna.

    - Sensing Antenna

# Installation Height

Recommendation 4-3-2-1-1

Install the CPE sensing antenna outdoors at a nominal height of 10 meters above ground level and co-located with the its transmit/receive antenna.

Recommendation 4-3-2-1-2

Give special attention to the installation of the outdoor antenna to make sure that the sensing capabilities of the CPE are not impacted by local obstructions.

# Pattern

Recommendation 4-3-2-2-1

Use a CPE sensing antenna with 0 dBi gain or greater (where all losses between the antenna and the input to the receiver are included) in any azimuthal direction and polarization.

Recommendation 4-3-2-2-2

Use a CPE sensing antenna that is able to receive equally on both horizontal and vertical polarizations.

# Operation

- System

# Continuous Monitoring

# Software

Devices must include features to ensure that only software that is approved with a device can be loaded into this device, and the software may not allow the user to operate the device with parameters outside those that were approved.  “Software” in this context includes the software that selects a device’s operating frequency, determines a device’s geographic location, identifies TV channels that are vacant, and accesses information in a related database.

# Distributed Sensing

Sensing reliability may be improved by the collaborative sensing from a number of well selected CPEs.  Information  on the geo-location of the CPE relative to the presumed origin of the signal will be available at the BS.  [We are hoping for text from A. Mody and R. Reddy.]

# Base Station

Different classes of BSs with different conditions of operation such as transmit power levels, etc., are supported where permitted by regulatory domains.

# Requests to the CPE

The BS SHALL be able to request local interference sensing and channel scanning from a CPE at any time.  When a new CPE attempts to register with a BS,, the BS shall verify that the local interference potential from the CPE would allow the CPE to access the network.

# Maximum Effective Isotropic Radiated Power (EIRP)

The default class of base station SHALL operate at a maximum of 4 W EIRP.

# Duty Cycle

Base stations SHALL support 100% transmit duty cycle at rated power.

# Monitoring

The BS or its operator SHALL periodically access a TV channel availability database derived from current FCC data and computational software to ensure that occupied channels by TV remain unused.

# Manual Monitoring

# Automatic Monitoring

Information will be sent and collated at the base station where mapping of the interference situation will be made based on the sensing information from the CPEs and according to their respective geographic coordinates.  Solutions to avoid interference will be deduced at the base station and measures will be signalled to the CPEs such as channel change or power reduction.

Sensing reliability is improved by the collaborative sensing from a number of well selected CPEs based on their geo-location relative to the presumed origin of the signal since all this information will be available at the base station.  The sensing scheme needs to be considered more as a component of a more complex distributed sensing system than an independent device that, on its own, needs to meet the sensing requirement.[This is redundant].

# Customer Premises Equipment

# Maximum Effective Isotropic Radiated Power

4 Watt maximum CPE transmit EIRP.

Accordingly, the maximum transmitted CPE EIRP will have to be limited (by capping the TPC range) to a value that is related to the channel separation between WRAN and TV operations.

# Monitoring

The BS or its operator SHALL periodically access a TV channel availability database derived from current FCC data and computational software to ensure that occupied channels by TV remain unused.

# Manual Monitoring

# Automatic Monitoring

Note that the information acquired from the local WRAN base stations through the CHO-UPD MAC message during initialization could be used to skip the channels where presence of WRAN or incumbent operation is known. However, since time is less critical at initialization, it may be useful that the CPE carries out the above detection and sensing process to confirm the local presence of WRAN or incumbent operation as indicated in the CHO-UPD MAC message.

Annex 1

Planning of Service Procedure Example [Need to also describe a simpler methodology for deployment also (involving a simpler propagation model, F(50,50) and separation distances).***]

- Acquire the necessary tools and information.

  - WRAN BS operating parameters

    - Antenna characteristics (OMNI-directional dipole is assumed)

    - Intended height of the center of radiation (AGL) (75 m is assumed)

    - Operating channel

  - Incumbent receiver characteristics

Table : Incumbent Receiver Characteristics

|  | Low VHF | High VHF | UHF |
| --- | --- | --- | --- |
| Lead in loss (dB) | 1 | 2 | 4 |
| Front to back ratio (dB) | 10 | 12 | 14 |
| Antenna gain (dBi) | 6.15 | 8.15 | 12.15 |

    - Receivers are outdoors at 9 m height.

    - Receiver antenna patterns use a cosine exponent equal to 4.  If the azimuth offset between the WRAN transmission and the main lobe of the receiver antenna azimuth pattern is less than 90 degrees, the pattern contribution returned is the maximum of either the cosine of the azimuth offset raised by the pattern exponent or the front-to-back ratio.  If the azimuth offset is greater than 90 degrees, the pattern contribution is equal to the front-to-back ratio.

  - Incumbent database

    - TV transmitter operating parameters are from the 12/20/06 FCC CDBS database.

    - Only co-channel and adjacent channel interference was considered (“taboo” channel relationships were not considered).

    - Only high power TV stations were considered (low power or “class A” TV stations were not considered).

  - Propagation analysis tool

    - 8-Vestigial Side-Band (8-VSB) Advanced Television Systems Committee modulation for 802.22 systems.

    - Desired to undesired ratios (D/U) from the Federal Communications Commission (FCC) Office of Engineering and Technology Bulletin 69.

    - 30 meter United States Geological Survey terrain data.

    - Population from 2000 Topographically Integrated Geographic Encoding and Referencing system census data.

    - Terrain Integrated Rough Earth Model propagation prediction model (TIREM).

    - Long Term Fading, Environmental Science Services Administration (ESSA) Technical Report ESSA Research Laboratories 79-Institute for Telecommunication Sciences 67.

    - Surface of the Earth Electrical Characteristics from International Telecommunication Union Recommendation 527-3.

    - 802.22 coverage simulations use 99.9 % time availability.

    - Digital television (DTV) interference analysis, desired and undesired signals use 90 % and 10 %, respectively for time availability.

    - Analog interference analysis desired and undesired signals use 50 % and 10 %, respectively for time availability.

    - Other assumptions regarding transmitter and receiver parameters for link budgets are from 802.22 document number 22-04-0002-10-0000.

- Identify the area of interest to deploy the WRAN service.

In this example, a location in Linden, TN was chosen (35.37°, -87.50°)

- Determine a useable channel by plotting the co- and adjacent channel incumbent service noise-limited contours in the area of interest.

Note: The figures above do not accurately represent final post-transition DTV service or associated service contours.

Current co- and adjacent channel DTV noise-limited contours were plotted in the Linden, TN area for channels 25 through 30.  In the figures above, the black circle is a 40 km radius circle with center at a site in Linden, TN of high terrain elevation.  The red contours are the co- and adjacent channel DTV noise-limited contours.  It can be seen in the figures above that channels 26 through 28 are unusable for the Linden, TN site.  It appears that channels 25, 29, and 30 are possible because the selected site would not be contained within either a co- or adjacent channel DTV noise-limited contour.

For this example channel 30 is selected.

- Analyze the coverage achieved by the WRAN system.

Note: The figure above is still being developed.  Coverage shown may not be accurate.

The 4 W transmission from the WRAN BS does a good job serving the 40 km circle with a -102 dBm signal or greater.  Hovever the interference caused to this transmission from the nearby co- and adjacent channel higher power DTV transmissions is significant.  The interference is represented by the gray color in the figure above.

- Analyze the interference potential into the planned WRAN system from the nearby DTV service.

Note: The figure above is still being developed.  The interference shown may not be accurate.

The co- and adjacent channel interference into the WRAN BS from the nearby DTV transmissions is once again depicted in the figure above.  In this figure the interference is color coded to indicate the particular station that is causing the particular interference.

References:

- G. Chouinard, “WRAN Reference Model”, IEEE 802.22-04-0002-14-0000-WRAN_Reference_Model.xls, May 2006.

- G. Chouinard, “OFDMA Parameters”, IEEE 802.22-06-0264-07-0000-ofdma_parameters.xls, June 2006, https://mentor.ieee.org/802.22/file/06/22-06-0264-07-0000-ofdma-parameters.xls

- K. Salehian, Y. Wu, G. Gagnon, "ATSC-DTV Receiver Performance in the Presence of Adjacent/Taboo Channels Interference, and Antenna Cross-Polarization Discrimination Tests", IEEE 57th Annual Broadcast Symposium, Washington DC, October 31 to November 2, 2007.

- US Federal Communications Commission (FCC) Rules and Regulations, Title 47 Code of Federal Regulations, Part 73 Radio Broadcast Services, Section 73.625(b) DTV Coverage of Principal Community and Antenna System – Determining Coverage, http://edocket.access.gpo.gov/cfr_2007/octqtr/47cfr73.625.htm.

- US Federal Communications Commission (FCC) Rules and Regulations, Title 47 Code of Federal Regulations, Part 73 Radio Broadcast Services, Section 73.684 Prediction of Coverage, http://edocket.access.gpo.gov/cfr_2007/octqtr/47cfr73.684.htm.

- Institute of Electrical and Electronics Engineers (IEEE) LAN/MAN Standards Committee 802 Working Group Draft Standard for Wireless Regional Area Networks (WRAN) Part 22: Cognitive Wireless RAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications: Policies and Procedures for Operation in the TV Bands, 2006.

National Association of Radio Telecommunications Engineers

Electro Magnetic Compatibility

Society of Broadcast Engineers

National Association of Radio Telecommunications Engineers

Electro Magnetic Compatibility

Society of Broadcast Engineers

National Association of Radio Telecommunications Engineers

Electro Magnetic Compatibility

Society of Broadcast Engineers