IEEE P802.22
Wireless RANs

| Draft Recommended Practice |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-11-17 |  |  |  |  |
| NameAuthor(s): | Company | Address | Phone | email |
| Winston Caldwell | FOX | 10201 W.Pico Blvd Los Angeles, CA 90064 | 310-369-4367 | Winston.caldwell@fox.com |
| Gerald Chouinard | Communications Research Center | 3701 Carling Avenue Ottawa, Canada K2H-8S2 | 613-998-2500 | Gerald.chouinard@crc.ca |

Abstract

This document will serve as the working document toward the draft 802.22.2 Recommended Practice.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- Deployment

This section is intended to be used as a guide by the owner of the WRAN service and the individual responsible for selecting the service area of the WRAN system.

  - Coverage and Interference Prediction Model

Recommendation 1-1-1

Use a more precise coverage and interference model than the FCC Part 73 coverage curves or the ITU-R Recommendation P.1546 for deployment planning of WRAN systems.

    - Characteristics

Recommendation 1-1-1-1

Use a coverage and interference prediction model for the planning of a WRAN service that incorporates at least the following:

- Point-to-point propagation model.

- Desired to undesired protection ratios (D/U) for the various channel relationships considered should be selected from a scientific reference that provides results from analysis examining the specific modulations used by both the desired and undesired signals.

- Transmitting and receive antenna pattern characteristics for the incumbent, BS, and CPEs.

- Height of the transmitting antenna Above Ground Level (AGL).

- 30 meter terrain data.

- Population from current census data.

- K-factor and other atmospheric effects on signal fading.

- Ground cover (wetlands, desert, tropical, etc)

- Ground clutter (trees, urban buildings, etc).

- Long term fading effects, such as those described in Environmental Science Services Administration (ESSA) Technical Report ESSA Research Laboratories 79-Institute for Telecommunication Sciences 67.

- Surface of the Earth electrical characteristics, such as those described in the Recommendation ITU-R P.527-3.

    - Statistics

Recommendation 1-1-2-1

Use the following statistical information when performing a coverage and interference prediction analysis:

- 802.22 coverage simulations should use 90 % and 10 % for time availability for desired and undesired signals, respectively and 50 % for location availability.

- Digital television (DTV) interference analysis should use 90 % and 10 % for time availability for desired and undesired signals, respectively and 50 % for location availability.

- Analog interference analysis desired and undesired signals use 50 % and 10 %, respectively for time availability and 50 % for location availability.

  - Selection of Deployment Location

    - Keep-Out Distance Considerations

Recommendation 1-2-1

Locate WRAN devices outside the keep-out distance that is beyond co- and adjacent channel protected contours of TV stations that is specified by the local regulations.

Recommendation 1-2-2

Locate WRAN devices outside the regulatory required keep-out distances from translator receive sites, cable headends, fixed broadcast auxiliary service links, Private Land Moblie Radio Services/Commercial Mobile Radio Services, offshore radiotelephone services, Part 74 devices including wireless microphones, border areas, and radio astronomy services that are registered as protected services in the database service.

Recommendation 1-2-3

Collect specific data that would influence RF propagation, including the difference in height between transmitter and receiver, terrain, ground cover, and atmospheric characteristics for the deployment area of interest.

Recommendation 1-2-4

Use the data collected in Recommendation 1-2-3 as input into an RF propagation prediction tool with the characteristics listed in Recommedation 1-1-1-1 to determine the necessary keep-out distances to avoid causing harmful interference.

Recommendation 1-2-5

Respect the greater keep-out distances between those required from Recommendation 1-2-1, those required from Recommendation 1-2-2, and those resulting from Recommendation 1-2-4.

Recommendation 1-2-6

Locate a WRAN device’s transmit antenna at a 16 m minimum distance from the closest TV receiver to avoid causing taboo and third-order intermodulation interference while operating on second adjacent channel relationships and beyond inside the protected contour.

Recommendation 1-2-7

Avoid 1 dB of WRAN receiver desensitization as caused by a high power TV transmission by adhering to the minimum separation distances as provided in the following table:

Table 1:  Distance from a DTV transmit station

| DTV into WRAN | Desens. | CPE | BS |
| --- | --- | --- | --- |
| Co-channel, N | 1 dB | 406 km | TBD for 75m |
| Adjacent channels, N±1 | 1 dB | 91.2 km | TBD for 75 m |
| N±2 and beyond | 1 dB | 6.6 km | TBD for 75 m |

Note: ITU-R Rec. P.1546, 1 MW ERP and 300 m HAAT DTV station

Recommendation 1-2-8

Maintain the necessary separation from high power TV transmitters in order to avoid saturation of the WRAN receiver.

Recommendation 1-2-9

Avoid a channel that is occupied by Part 74 operation, be located beyond a radius of 4 km from the Part 74 receiver, or reduce the maximum TPC limit according to the following equation:

Tapering = Path loss exponent * 10*log(actual distance/[distance in the table])   (dB)

where:	path loss exponent = 3.0   [tbd]

    - Interference Considerations

      - Interference into Incumbent Services

Recommendation 1-2-2-1-1

Do not cause harmful interference into the protected licensed services.

Recommendation 1-2-2-1-2

Do not allow the received power from a WRAN device transmission for the channel relationships provided in the following table to a protected DTV transmission to exceed the values provided in the table as measured at the input of a DTV receiver.

Table 2: Received Power Limit from a WRAN Device to Protect the DTV Service from Interference

| Channel Relationship | Received Power Limit (dBm) |
| --- | --- |
| N | -107 |
| N+/-1 | -51 |
| N+/-2 | -40 |
| N+/-3 | -36 |
| N+/-4 | -32 |
| N+/-5 | -28 |
| N+/-6 | -27 |
| N+/-7 | -27 |
| N+/-8 | -27 |
| N+/-9 | -27 |
| N+/-10 | -27 |
| N+/-11 | -27 |
| N+/-12 | -27 |
| N+/-13 | -27 |
| N+/-14 | -34 |
| N+/-15 | -34 |

Note: the received power limit is measured at the input of the protected DTV receiver in 6 MHz.

[Instead of a received power limit, use a separation distance.  Make the decision a yes/no for operating inside the DTV contour or not. This threshold has been suggested to be, if over 20 m for each channel relationship, recommend not operating inside the contour].

Recommendation 1-2-2-1-3

Avoid using channels which would generate an intermod component considering the ‘2A-B’ intermodulation scenario equivalent to –107 dBm in a channel for which the DTV receiver is inside its protected contour. [Convert the received power limit recommendation into a 23 dB D/U].

Recommendation 1-2-2-1-4

Apply an additional 15 dB reduction to the maximum EIRP level of the TPC range if the CPE needs to use a channel that would cause the ‘2A-B’ intermodulation scenario because of the lack of other available channels.

Recommendation 1-2-2-1-5

Prevent the WRAN power level at the input of the TV receiver from exceeding the saturation level of the TV receiver.

      - Interference from Incumbent Services

Recommendation 1-2-2-2-1

Plan the deployment area to be capable of operating at QPSK modulation tolerating an interference field strength from licensed services that is 8 dB lower than the intended received signal level at QPSK modulation.

      - Coexistence with other License-Exempt Services

Recommendation 1-2-2-3-1

Avoid causing harmful interference into other services as much as possible

Recommendation 1-2-2-3-2

Use the means provided in the standard for self-coexistence to coexist as much as possible with other license-exempt systems.

Recommendation 1-2-2-3-3

Deploy a sufficient number of CPEs about the outer edge of the coverage area so to increase the probability of successful coexistence beacon protocol transmissions.

Unreliable CBP burst transmissions across overlapping WRAN cells on the same TV channel would not allow proper coexistence and would result in either interference or having to switch to different channels for operation.

Recommendation 1-2-2-3-4

Avoid BS interference into neighboring BSs by avoiding line of sight (LOS) between their BSs.

Interference between LOS BSs operating on the same TV channel will create upstream/downstream collision which would need to be avoided by either synchronizing the upstream/downstream split, reducing the upstream/downstream flexibility, or allocating different frames for each BS resulting in less capacity.

- Installation

After the WRAN system planner completes the suggested deployment process described above in Section 7, the professional installer should follow the suggestions provided in this section for a proper installation.  This section also describes the characteristics of the WRAN system devices that should be installed.    The 802.22 standard was designed while assuming the WRAN system characteristics provided in this section.

  - System

Recommendation 2-1-1

Only install certified 802.22 equipment.

Recommendation 2-1-2

Comprise the WRAN system of a BS and CPEs.

Recommendation 2-1-3

Do not install equipment with physical damage or loose connectors.

Recommendation 2-1-4

Utilize appropriate weather stripping tape to any connector that might be exposed to the outdoor environment.

    - Cable Protection

Recommendation 2-1-1-1

Install transmitting equipment at least 10 m away from cable TV equipment.

Recommendation 2-1-1-2

Check nearby TV receivers to ensure that the installed WRAN transmission does not cause interference to the TV service.

Recommendation 2-1-1-3

Ensure that metallic objects or reflectors are not close to or in the path of the WRAN transmitting antenna.

  - Base Station

Recommendation 2-2-1

Perform and inspect the installation of BSs by a trained, competent professional, such as e.g. a NARTE Certified EMC Engineer, an SBE Certified Professional Broadcast Engineer, or a Registered Professional Engineer.

Recommendation 2-2-2

Ensure that the BS does not transmit unless it has established a communication connection with the database service and has received channel availability information from the database service.

Recommendation 2-2-3

Configure the BS with the network address of the database service.

Recommendation 2-2-4

Configure the BS with the network address of the root CA that issues the certificate to the database service.

Recommendation 2-2-5

Install the BS antennas in a fixed position, outdoor, and in the clear.

Recommendation 2-2-6

Install the BS antennas at a height that satisfies the local regulations.

    - Transmit/Receive Antenna

Recommendation 2-2-1-1

Use a BS transmit/receive antenna with at least 8 dBi of gain contributed from the vertical pattern.

Recommendation 2-2-1-2

Orient the BS transmit/receive antenna orthogonal to the orientation of the nominal television receive antennas in the area in the polarization plane within a tolerance of 3 deg.

    - Sensing Antenna

Recommendation 2-2-2-1

Use a BS sensing antenna with a gain of 0 dBi or greater (where all losses between the antenna and the input to the receiver are included) in any azimuthal direction and polarization.

Recommendation 2-2-2-2

If the sensing antenna provides less gain than 0 dBi in any azimuthal direction or polarization, adjust the sensing threshold to be more sensitive to account for the dB loss in antenna gain.

Recommendation 2-2-2-3

Use  a BS sensing antenna that has similar gain in both horizontal and vertical polarizations.

  - Customer Premises Equipment (CPE)

Recommendation 2-3-1

Perform and inspect the installation of CPEs by a trained, competent professional, such as e.g. a NARTE Certified EMC Engineer, an SBE Certified Professional Broadcast Engineer, or a Registered Professional Engineer.

Recommendation 2-3-2

Prevent CPE transmissions absent reliable verification of the Recommendations 2-1-1-1, 2-1-1-2, 2-1-1-3, and 2-3-1.

Recommendation 2-3-3

Permanently install the CPE sensing antenna outdoors, in the clear, on a fixed structure, and properly aligned.

Recommendation 2-3-4

Verify that the CPE sensing antenna remains in the location where it was installed.

Recommendation 2-3-5

Employ provisions to prevent the use of antenna equipment that could impair or defeat Recommendations 2-3-1, 2-3-2, 2-3-3, and 2-3-4.

    - Transmit/Receive Antenna

Recommendation 2-3-1-1

Enforce a minimum separation distance of 10 m between the CPE transmit/receive antenna and DTV receiving antennas.

Recommendation 2-3-1-2

Orient the CPE transmit/receive antenna so that it and the TV receive antenna are looking away from each other.

Recommendation 2-3-1-3

Install the CPE transmit/receive antenna so that the TV receive antenna is not in the path between the CPE transmit/receive antenna and the BS.

Recommendation 2-3-1-4

Minimize the loss, degree of re-radiation, and VSWR between the CPE transmit/receive antenna and the CPE receiver due to cable leakage and impedance mismatch.

      - Installation Height

Recommendation 2-3-1-1-1

Install the CPE transmit/receive antenna outdoors at a nominal height of 10 meters above ground level and co-located with the CPE sensing antenna.

      - Pattern

Recommendation 2-3-1-2-1

Do not allow the gain differential between the transmit and receive patterns to exceed 0.5 dB.

Recommendation 2-3-1-2-2

Orient the CPE transmit/receive antenna toward the base station of the selected service provider.

Recommendation 2-3-1-2-3

Re-orient the CPE transmit/receive antenna toward the new service provider if a new service provider is selected.

Recommendation 2-3-1-2-4

Verify that the CPE transmit/receive antenna is oriented properly.

Recommendation 2-3-1-2-5

Further adjust the CPE transmit/receive antenna to minimize the gain in the direction of an interfering source.

Recommendation 2-3-1-2-6

Use CPE transmit/receive antennas that are polarized orthogonally to nearby DTV receive antennas.

Recommendation 2-3-1-2-7

Ensure that the CPE transmit/receive antenna is oriented in the polarization plane within a tolerance of 10 deg.

Recommendation 2-3-1-2-8

Mount the CPE transmit/receive antenna at the same height as the nearby TV receive antenna.

    - Sensing Antenna

      - Installation Height

Recommendation 2-3-2-1-1

Install the CPE sensing antenna outdoors at a nominal height of 10 meters above ground level and co-located with the its transmit/receive antenna.

Recommendation 2-3-2-1-2

Give special attention to the installation of the outdoor antenna to make sure that the sensing capabilities of the CPE are not impacted by local obstructions.

      - Pattern

Recommendation 2-3-2-2-1

Use a CPE sensing antenna with 0 dBi gain or greater (where all losses between the antenna and the input to the receiver are included) in any azimuthal direction and polarization.

Recommendation 2-3-2-2-2

Use a CPE sensing antenna that is able to receive equally on both horizontal and vertical polarizations.

Annex 1

Planning of Service Procedure Example [Need to also describe a simpler methodology for deployment also (involving a simpler propagation model, F(50,50) and separation distances).***]

- Acquire the necessary tools and information.

  - WRAN BS operating parameters

    - Antenna characteristics (OMNI-directional dipole is assumed)

    - Intended height of the center of radiation (AGL) (75 m is assumed)

    - Operating channel

  - Incumbent receiver characteristics

Table : Incumbent Receiver Characteristics

|  | Low VHF | High VHF | UHF |
| --- | --- | --- | --- |
| Lead in loss (dB) | 1 | 2 | 4 |
| Front to back ratio (dB) | 10 | 12 | 14 |
| Antenna gain (dBi) | 6.15 | 8.15 | 12.15 |

    - Receivers are outdoors at 9 m height.

    - Receiver antenna patterns use a cosine exponent equal to 4.  If the azimuth offset between the WRAN transmission and the main lobe of the receiver antenna azimuth pattern is less than 90 degrees, the pattern contribution returned is the maximum of either the cosine of the azimuth offset raised by the pattern exponent or the front-to-back ratio.  If the azimuth offset is greater than 90 degrees, the pattern contribution is equal to the front-to-back ratio.

  - Incumbent database

    - TV transmitter operating parameters are from the 12/20/06 FCC CDBS database.

    - Only co-channel and adjacent channel interference was considered (“taboo” channel relationships were not considered).

    - Only high power TV stations were considered (low power or “class A” TV stations were not considered).

  - Propagation analysis tool

    - 8-Vestigial Side-Band (8-VSB) Advanced Television Systems Committee modulation for 802.22 systems.

    - Desired to undesired ratios (D/U) from the Federal Communications Commission (FCC) Office of Engineering and Technology Bulletin 69.

    - 30 meter United States Geological Survey terrain data.

    - Population from 2000 Topographically Integrated Geographic Encoding and Referencing system census data.

    - Terrain Integrated Rough Earth Model propagation prediction model (TIREM).

    - Long Term Fading, Environmental Science Services Administration (ESSA) Technical Report ESSA Research Laboratories 79-Institute for Telecommunication Sciences 67.

    - Surface of the Earth Electrical Characteristics from International Telecommunication Union Recommendation 527-3.

    - 802.22 coverage simulations use 99.9 % time availability.

    - Digital television (DTV) interference analysis, desired and undesired signals use 90 % and 10 %, respectively for time availability.

    - Analog interference analysis desired and undesired signals use 50 % and 10 %, respectively for time availability.

    - Other assumptions regarding transmitter and receiver parameters for link budgets are from 802.22 document number 22-04-0002-10-0000.

- Identify the area of interest to deploy the WRAN service.

In this example, a location in Linden, TN was chosen (35.37°, -87.50°)

- Determine a useable channel by plotting the co- and adjacent channel incumbent service noise-limited contours in the area of interest.

Note: The figures above do not accurately represent final post-transition DTV service or associated service contours.

Current co- and adjacent channel DTV noise-limited contours were plotted in the Linden, TN area for channels 25 through 30.  In the figures above, the black circle is a 40 km radius circle with center at a site in Linden, TN of high terrain elevation.  The red contours are the co- and adjacent channel DTV noise-limited contours.  It can be seen in the figures above that channels 26 through 28 are unusable for the Linden, TN site.  It appears that channels 25, 29, and 30 are possible because the selected site would not be contained within either a co- or adjacent channel DTV noise-limited contour.

For this example channel 30 is selected.

- Analyze the coverage achieved by the WRAN system.

Note: The figure above is still being developed.  Coverage shown may not be accurate.

The 4 W transmission from the WRAN BS does a good job serving the 40 km circle with a -102 dBm signal or greater.  Hovever the interference caused to this transmission from the nearby co- and adjacent channel higher power DTV transmissions is significant.  The interference is represented by the gray color in the figure above.

- Analyze the interference potential into the planned WRAN system from the nearby DTV service.

Note: The figure above is still being developed.  The interference shown may not be accurate.

The co- and adjacent channel interference into the WRAN BS from the nearby DTV transmissions is once again depicted in the figure above.  In this figure the interference is color coded to indicate the particular station that is causing the particular interference.

References:

- G. Chouinard, “WRAN Reference Model”, IEEE 802.22-04-0002-14-0000-WRAN_Reference_Model.xls, May 2006.

- G. Chouinard, “OFDMA Parameters”, IEEE 802.22-06-0264-07-0000-ofdma_parameters.xls, June 2006, https://mentor.ieee.org/802.22/file/06/22-06-0264-07-0000-ofdma-parameters.xls

- K. Salehian, Y. Wu, G. Gagnon, "ATSC-DTV Receiver Performance in the Presence of Adjacent/Taboo Channels Interference, and Antenna Cross-Polarization Discrimination Tests", IEEE 57th Annual Broadcast Symposium, Washington DC, October 31 to November 2, 2007.

- US Federal Communications Commission (FCC) Rules and Regulations, Title 47 Code of Federal Regulations, Part 73 Radio Broadcast Services, Section 73.625(b) DTV Coverage of Principal Community and Antenna System – Determining Coverage, http://edocket.access.gpo.gov/cfr_2007/octqtr/47cfr73.625.htm.

- US Federal Communications Commission (FCC) Rules and Regulations, Title 47 Code of Federal Regulations, Part 73 Radio Broadcast Services, Section 73.684 Prediction of Coverage, http://edocket.access.gpo.gov/cfr_2007/octqtr/47cfr73.684.htm.

- Institute of Electrical and Electronics Engineers (IEEE) LAN/MAN Standards Committee 802 Working Group Draft Standard for Wireless Regional Area Networks (WRAN) Part 22: Cognitive Wireless RAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications: Policies and Procedures for Operation in the TV Bands, 2006.

National Association of Radio Telecommunications Engineers

Electro Magnetic Compatibility

Society of Broadcast Engineers

National Association of Radio Telecommunications Engineers

Electro Magnetic Compatibility

Society of Broadcast Engineers