IEEE P802.22 Wireless RANs

| Contribution for IEEE 802.22 WRAN Systems: Interference Detection with Preamble |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-11-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Linjun Lv | Huawei Technologies | Shenzhen, China | 86-755-28973119 | lvlinjun@huawei.com |
| Zhou Wu | Huawei Technologies | Shenzhen, China | 86-755-28979499 | wuzhou@huawei.com |
| Mingwei Jie | Huawei Technologies | Shenzhen, China | 86-755-28972660 | jiemingwei@hauwei.com |
| Soo-Young Chang | Huawei Technologies | Davis, CA, U.S. | 1-916 278 6568 | sychang@ecs.csus.edu |
| Jianwei Zhang | Huawei Technologies | Shanghai, China | 86-21-68644808 | zhangjianwei@huawei.com |
| Lai Qian | Huawei Technologies | Shenzhen, China | 86-755-28973118 | qlai@huawei.com |
| Jianhuan Wen | Huawei Technologies | Shenzhen, China | 86-755-28973121 | wenjh@huawei.com |

Abstract

In this proposal a method for incumbent signal detection that does not need to interrupt WRAN services is introduced. With this method co-channel signals of other incumbent user systems can be detected without scheduling and inserting quiet periods and channel estimation. Some simulation results are suggested.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Document Policy and Procedures: The contributor is familiar with the IEEE 802 Document Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of document(s), including document applications, provided the IEEE receives assurance from the document holder or applicant with respect to documents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of document information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if documented technology (or technology under document application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Document Committee Administrator at <patcom@ieee.org>.

Contents

1. Reference	5

2. Introduction	5

3. Technology of Interference Detection with Preamble	5

3.1 Principle of the Algorithm	6

3.2 Process of the Algorithm	8

3.3 Performance Simulation	10

3.4 More Discussion on the Algorithm	17

List of Figures

Figure 1	Interference detection in OFDM system	5

Figure 2	Correlation of PNi and PN	7

Figure 3	distribution of receive signal energy with/without interference	9

Figure 4	Detection probability and false alarm probability with INR = 0dB	10

Figure 5	Detection probability and false alarm probability with INR = 5dB	11

Figure 6	Detection probability  VS INR  using preamble of superframe	12

Figure 7	Detection probability and false alarm probability with INR = 0dB	13

Figure 8	Detection probability and false alarm probability with INR = 5dB	14

Figure 9	Detection probability  VS INR using preamble of frame	14

Figure 10	Detection probability and false alarm probability with INR = 0dB	15

Figure 11	Detection probability and false alarm probability with INR = 5dB	16

Figure 12	Detection probability  VS INR using preamble of US burst	17

List of Tables

None

- Reference

[1]	IEEE 802.22 Working Group, “Draft Standard for Wireless Regional Area Networks Part22: Cognitive Wireless RAN Medium Access Control (MAC) and Physical Layer (PHY) specifications: Policies and procedures for operation in the TV Bands”, IEEE 802.22-06/0067D0, May 2006.

[2]     M. D. Duncan, and A. S. Robert, “Acquisition of spread spectrum signals by an adaptive array”, IEEE Trans. On acoustics, speech, and signal processing, vol. 37, No. 8, Aug. 1989.

[3]      D. Landstrom, S. K. Wilson, J. – J. Van de Beek, Per Odling, and P. O. Borjesson, “Synchronization for a DVB-T receiver in presence of co-channel interference”, PIMRC, IEEE, vol. 5, pp. 2307-2311, Sept. 2002.

[4]      ATSC A/74, “ATSC Recommended Practice: Receiver Performance Guidelines”.

- Introduction

In this proposal we introduce a method of incumbent signal detection that does not need to interrupt WRAN services. With this method we can detect co-channel signals of other incumbent user systems without scheduling and inserting quiet periods and channel estimation.

As shown in Figure 1, in the WRAN system which is based on OFDM technology, Xk,i is the symbol transmitted at time i and on the kth subcarrier; Hk,i is the channel response at time i and on the kth subcarrier; Yk,i is the received symbol at time i and on the kth subcarrier; Ik,i is the interference signal at time i and on the kth subcarrier.

- Interference detection in OFDM system

When the interference exists, the received signal in frequency domain can be depicted as:

Yk,i＝Xk,iHk,i＋Ik,i＋nk,i                                                                    (1)

Without considering the interference, the above equation can be modified as:

Yk,i＝Xk,iHk,i＋nk,i                                                                           (2)

In most WRAN scenarios, the schemes proposed herein can be assumed to work in static or quasi-static channels. Within the coherence time and the coherence bandwidth, Hk,ican be regarded as a constant. Then Hk,ican be noted simply as H.

- Technology of Interference Detection with Preamble

To realize synchronization and channel estimation, in WRAN frame structure, preambles are inserted at beginning of the frame and super frame. At the beginning of US burst in the upload direction, preambles are also inserted. Preamble is composed of pseudo random series (PN), which can be generated through a series of linear shift registers. The PN comes from a primitive polynomial, so it has characteristics of an m sequence which can be used for interference detection. More details on this can be found in reference [1].

  - Principle of the Algorithm

On the kth sub-carrier at time slot i, the received symbol can be expressed as:

Yk,i＝Xk,iHk,i＋Ik,i＋nk,i                         (3)

When there is no interference, Ik,i can be set to 0.

At any time slot i, signals in frequency domain, Xk,i,Ik,i and nk,i (k=0,1,…, Msubcarrier, Msubcarrier is the number of sub-carriers in a TV sub-band of 6MHz) are irrelevant. At receiver when i=1,2, Xk,i is the first and second received OFDM symbol called preamble which is a PN modulated on frequency domain.(For downlink frame, the first symbol is composed of several short training sequences (PN) PFrame,ST(k) while the second  symbol is composed of two long training sequencesPFrame,LT(k); for downlink super frame, the first symbol is composed of several short training sequences (PN) PST(k) while the second  symbol is composed of two long training sequencesP(k); for uplink, the first symbol is composed of several short training sequences (PN) PBurst(k).Details of these sequence formats can also be seen in Section 8.3.1 in Reference [1].) The preamble has a characteristic of approximately zero cyclic self-correlation with no-zero shift and no-zero cyclic self-correlation with zero shift.

In addition, according to requirement of fractional bandwidth detection, m PN series PNm (m＝1,2,…,6) are constructed in frequency domain, each of which represent a PN inserted in one of the six 1MHz bandwidths. Construction of the PN is:PNm: except for sub-carriers in the mth MHz, set points on other sub-carriers to zero.

After then, perform shift correlation operations with each PNm and received PN. It can be seen that result of cyclic correlation mentioned in the above is nearly 0 with no-zero shift and take the maximum value when shift is zero (value of the maximum actually equals to energy of PNm).

On the other hand, PNm has the stochastic property, for external interference, assuming Ik,i and nk,i are stochastic signals independent to each other, for example Gaussian stochastic signals, then, according to stochastic theories, expectation of correlation of PNm and Ik,i or PNm and nk,iare all 0.

Therefore, when performing shift correlation operations, and when received signal is aligned with PNm, maximum correlation result can be attained, and the value equals to production of channel coefficient Hk,i  and maximum of the self-correlation of sequence PNm, this can be expressed by the following formula:

kYk,iXk,i=kHiXk,iXk,i＋kIk,iXk,i＋knk,iXk,i=kHiXk,iXk,i                      (4)

The above calculation is based on the assumption thatXk,i,Ik,i and nk,i are independent and can be seen in the simulation curve of Figure 2. The result of shift correlation of PNm and PN is close to impulse sequence, this characteristic can be used for detection of fractional bandwidth interference in a WRAN sub-band (its bandwidth can be 6MHz, 7MHz and 8MHz).

- Correlation of PNi and PN

(from top to bottom show PN1～PN6 correlation with PN)

On the other hand, energy of signal Yk,i=Xk,iHk,i＋Ik,i＋nk,i can be calculated as below, it is assumed here that Xk,i,Ik,i and nk,iare independent.

kYk,iYk,i=Hi2kXk,iXk,i+kIk,iIk,i+knk,ink,i        (5)

Since PN is a known sequence, so kXk,iXk,i is also known. Therefore, according to formula (1) Hi can be attained. Then combine formula (5) and (Xk,i,Xk,i) which already known, formula (6) among (5) can be determined.

kIk,iIk,i+knk,ink,i=kYk,iYk,i−Hi2kXk,iXk,i                      (6)

Formula (6) denotes power of sum of interference and noise.

In process of detection, kYk,iXk,i is attained through shift correlation of two sequences: {Yk,i|i denotes transmitting time of preamble, k=0, 1, 2….}, ｛Xk,i |i denotes transmitting time of preamble, k=0,1,2….｝. Let Zndenote the shift correlation sequence. Then

Zn=kYk+n,iXk,i,n=0,1,2......                                                      (7)

Operation of formula (7) is performed in the frequency domain. Considering the large amount of computation for correlation operations, we can perform IFFT to both side of formula (7) first, and then we can get the representation in the time domain like the following formula (8):

zn=yn,ixn,i,n=0,1,2......                                          (8)

In the above formulazn, yn,i andxn,iare respectively corresponding to time domain representations of Zn, Yn,i and Xn,i.

Therefore, we can firstly calculate the conjugated product of signals in the time domain and then perform FFT operations to get the conjugated shift correlation sequences in the frequency domain.

  - Process of the Algorithm

According to fundamental principles mentioned in the above section, the scheme of interference detection can be described as following:

After receiving preamble, the receiver can detect interference based on preamble as the flow below:

1) assuming received signal sequence in time domain is｛yn,i| where i denotes transmitting time, n=0,1,2,…},herein, length of the sequence equals to FFT size of OFDM symbol.

2) calculate six product sequences:zm(n)=yn,iqm(n),n=0,1,2,...

where qi(n)=pi(n), and pi(n)=IFFTPNi(k), i.e. qi(n)  is the time domain representation of PNi.

3) perform FFT to zm(n), then find maximum of each sequence:

zm= arg max zm(n)

4) calculate channel parameters Hm according to formula (4)

Hm=zmTPNm, where TPNm is energy sum of transmitted sequence in unit bandwidth calculated beforehand, i.e. TPNm=kXk,iXk,i, and the sum comes from all sub-carriers of the mth 1MHz bandwidth of the PN.

5) substitute Hm to formula (6), sum of interference and noise energies of each fractional bandwidth can be attained as:

Tm=Ym−Hm2TPNmMsubcarrier6,

wherein, Ym is the energy sum of received sequence, according the Parseval’s theorem we know:

Ym=kYk,iYk,i

6) comparing Tm with a threshold T, judgment can be confirmed that if there is interference on the mth 1MHzof current sub-band:

if Tm>T, judgment is that is interference on mth 1MHz of current sub-band;

if Tm<T, judgment is that is not interference on mth 1MHz of current sub-band;

In realization, making certain the threshold T is very important. According to some analyses, it is shown that T is mostly related to some transcendental information. That is, we choose T according to some transcendental information, such as characteristics of interference and noise. Ultimately, the threshold has little relation to power of current system signals. Thus we can choose threshold according to noise power and sensitivity of detection requirements. The following analyse how to choose the threshold.

When there is no interference, equation (6) can be transformed to

knk,i×nk,i=knk,i2=σn2⋅knk,iσn22                            (9)

Equation (9) submits to Chi-square distribution with degree of freedom of 341 (the number of sub-carrier on a sub-band).

When the interference exists, based on the assumption that Ik,i and nk,i are independent, we can get

kIk,i+nk,i2k(Ik,i×Ik,i+Ik,i×nk,i+nk,i×Ik,i+nk,i×nk,i)kIk,i2+knk,i2                       (10)

Ik,i+nk,iis the sum of two Gauss stochastic variable, so it also submit to Gauss distribution, and mean is zero, variance is σn2+σI2. So equation (10) also submits to Chi-square distribution with degree of freedom of 341. When the freedom becomes bigger, the distribution of (9) and (10) is spiculate. The energy distribution curve of (9) and (10) can be disjoined, so this characteristic can be used in interference detection. Figure 3 illustrates this point. In the figure, the blue curve show the distribution of no interference, and the red, magenta, green curve show the distribution of INR=-5dB, 0dB, 5dB respective.

- distribution of receive signal energy with/without interference

In practical condition, the power of noise and interference based on formula (6) may be bigger than practical value because of the channel condition. So the threshold will be affected by channel. We can choose a bigger threshold than it from Figure 3.

  - Performance Simulation

Simulations to evaluate the performance of the schemes using preamble of download frame and super frame and upload burst have been carried out. Wherein, interferences added to these signals are narrow band (200 KHz) interferences representing signals of FCC Part 74 devices such as wireless microphone systems.

(1) Simulation of interference detection using preamble of superframe

Firstly, verifying relationships between threshold and probability of detection and probability of false alarm, wherein all noises are additive white Gaussian signals with mean of zero and variance of 0.2.

- Detection probability and false alarm probability with INR = 0dB

200 experiments in one point

- Detection probability and false alarm probability with INR = 5dB

200 experiments in one point

Secondly, choose better threshold verifying detection probability with different INR.

- Detection probability  VS INR  using preamble of superframe

It can be found that result of simulation is very good.

(2) Simulations of interference detection using preamble of frame

Firstly, verifying relationships between threshold and probability of detection and probability and false alarm, wherein all noises are additive white Gaussian signals with mean of zero and variance of 0.2.

- Detection probability and false alarm probability with INR = 0dB

200 experiments in one point

- Detection probability and false alarm probability with INR = 5dB

200 experiments in one point

- Detection probability  VS INR using preamble of frame

(3) Simulation of interference detection using preamble of US burst

- Detection probability and false alarm probability with INR = 0dB

200 experiments in one point

- Detection probability and false alarm probability with INR = 5dB

200 experiments in one point

- Detection probability  VS INR using preamble of US burst

  - More Discussion on the Algorithm

Considering of that are null sub-carriers (with no traffic data modulated or with data modulated to zero) in preambles of downstream frames and super frames, energy of the interference or noise can be evaluated by calculating energy of the energy on these null sub-carriers. Accuracy of this method could be better than the algorithm mentioned above. Combining these null sub-carriers, with a little modification, we can get the new interference detection algorithm.

For short training sequence, the null sub-carriers have sequence numbers that can not be divided by 4. For long training sequence, the null sub-carriers have sequence numbers that can not be divided by 2.

Details of the modification are listed as follow:

1) Calculating data energy on null sub-carriers directly

Assuming received signal sequence in time domain is ｛yn,i| i denote transmitting time slot of the preamble, n=1,2,3,…},herein, length of the received sequence is size of FFT.

First, performing FFT, representation of the received signal in frequency domain can be attained, that is {Yk,i|i denote transmitting time slot of the preamble, where k=1,2,3,…}, then calculating corresponding energy of null sub-carriers on each 1MHz bandwidth.

For short training sequence, energy can be derived from the following formula:

Tm1=(kIk,iYk,i)(Msubcarrier8)             (11)

where Yk,i is derived from time domain signal yn,i by FFT, Msubcarrier is sample number of the whole 6MHz sub-band, k is the sequence number of a sub-carrier on the mth 1MHz bandwidth and k can not be divided by 4.

For long training sequence, energy can be derived from the following formula:

Tm1=(kIk,iYk,i)(Msubcarrier12)                   (12)

2) For interference and noise energy on none null sub-carriers, it can be derived as the algorithm mentioned in Sections 3.1～3.23.

(a) For interference detection with short training sequence, a different procedure from what is described in Section  3.2 should be applied:

Sequence yn,i derived in the first step of Section 3.2 should be truncated, and only 1/4 numbers in the front of the sequence is needed. The following FFT size should then be changed from Msubcarrier to Msubcarrier/4, and also,  Msubcarrier in the formula above should be changed to Msubcarrier/4. Wherein, interference and noise energy on the non-null sub-carriers is:

Tm2=(Ym−|Hm|2TPNm)(Msubcarrier24)          (13)

(b) For interference detection with long training sequence, a different procedure from what is described in Section 3.2 should be applied:

Sequence yn,i derived in the first step of Section 3.2 should be truncated, and only 1/2 numbers in the front of the sequence is needed. The following FFT size should then be changed form Msubcarrier to Msubcarrier/2, and also,  Msubcarrier in the formula above should be changed to Msubcarrier/2. Wherein, interference and noise energy on the non-null sub-carriers is:

Tm2=(Ym−|Hm|2TPNm)(Msubcarrier12)              (14)

3) Calculating the sum of energies obtained in the first two steps mentioned above.

Energy on the mth 1MHz bandwidth can be obtained by calculating sum of the energies in steps (1) and (2).

For relatively short training sequences, power of interference and noise can be calculated by the formula as below:

Tm=(3Tm1+Tm2)4                    (15)

For relatively long training sequences, power of interference and noise can be calculated by the formula as below:

Tm=(Tm1+Tm2)2                (16)

The last step is judgment of existence of interference:

when Tm>Tthreshold, there is interference existing on the mth 1MHz bandwidth of the sub-band;

when Tm<Tthreshold, there is not interference existing on the mth 1MHz bandwidth of the sub-band.