IEEE P802.22 Wireless RANs

| Synchronization Decoding for Beacon Devices for the 802.22.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-1-5 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Linjun Lv | Huawei Technologies | Shenzhen, China | 86-755-28973119 | lvlinjun@huawei.com |
| Zhou Wu | Huawei Technologies | Shenzhen, China | 86-755-28979499 | wuzhou@huawei.com |
| Mingwei Jie | Huawei Technologies | Shenzhen, China | 86-755-28972660 | jiemingwei@hauwei.com |
| Soo-Young Chang | Huawei Technologies | Davis, CA, U.S. | 1-916 278 6568 | sychang@ecs.csus.edu |
| Jianwei Zhang | Huawei Technologies | Shanghai, China | 86-21-68644808 | zhangjianwei@huawei.com |

Abstract

The solutions in this document are aimed to meet functional requirements of an IEEE 802.22 based WRAN system. In this contribution, a synchronization decoding scheme for beacon devices is proposed.

Our scheme proposed in this contribution can increase synchronization burst robustness, achieve higher decoding probability, and avoid resource waste because an SPD receives the whole superframe while it does not need to synchronize with the PPD.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Document Policy and Procedures: The contributor is familiar with the IEEE 802 Document Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of document(s), including document applications, provided the IEEE receives assurance from the document holder or applicant with respect to documents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of document information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if documented technology (or technology under document application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Document Committee Administrator at <patcom@ieee.org>.

Contents

1. Reference	5

2. Introduction	5

2.1 The Drawbacks of the Related Works	5

3. Synchronization burst decoding for beacon device	6

4. Conclusion	8

List of Figures

Figure 1	Superframe format	5

Figure 2	The performance of synchronization burst in AWGN channel	6

Figure 3	The transmitter diagram of our proposal	6

Figure 4	The performance comparison of before and after Walsh code spread	7

Figure 5	Format of the sync field	7

Figure 6	The number of bit <threshold	8

List of Tables

- Reference

[1]	Motorola “Part 22.1: Enhanced Protection for Low Power Licensed devices operating in TV Broadcast Bands”.

- Introduction

The Motorola proposed MAC and PHY specification for the IEEE 802.22.1 standard for protection of low power licensed devices operating in the TV spectrum. In this specification, Motorola presented a superframe format, which could be expressed:

BURSTS

Sync

N

Sync

N-1

Sync

1

Sync

0

BEACON PSDU

…

Sync

N

ANP

…

Frame period

BEACON

Rx period

- Superframe format

The superframe structure consists of a plurality of synchronization bursts, followed by a beacon frame. Under control of an upper layer, a receive period, and an acknowledgement/non-acknowledgement period (ANP) may also be included.  This format repeats without interruption while the protecting device is in operation.

The synchronization bursts, each of which consists of a synchronization word followed by a decrementing index value, enable a receiver asynchronously sampling the beacon channel to quickly determine when the beacon will be sent.

Primary protecting device (PPD) sent a serial of synchronization burst to secondary protecting device (SPD). SPD synchronized PPD through these burst. If an SPD wanted to sent data to the PPD, it sends an RTS burst to the PPD during the receive period of the superframe. Therefore synchronization bursts are so important for SPD to ensure that their sending RTS can be received by PPD accurately.

  - The Drawbacks of the Related Works

Just mentioned above, synchronization bursts are important. So SPD must decode synchronization bursts correctly. However in Motorola specification synchronization bursts aren’t encoded and modulated. That is to say there is not any protection proposal for them. Once channel becomes worse, SPD can not decode synchronization burst correctly.

The figure 1 shows the performance of synchronization burst in AWGN channel. From figure we can find that when channel condition is not good, BER is bad. So if SPD decodes synchronization burst inaccurately, they would lose synchronization with PPD. Thus they can not send their data to PPD when they desire.

To deal with this problem, we propose a synchronization burst decoding proposal for beacon device, which can increase synchronization burst robustness and decoding probability correctly.

- The performance of synchronization burst in AWGN channel

- Synchronization burst decoding for beacon device

Our proposal has two steps:

Step One: we proposal 8 bits Walsh code to spread synchronization index.

Synchronization index

Walsh Spread

BPSK Modulation

AWGN Channel

- The transmitter diagram of our proposal

Figure 3 shows the transmitter diagram of our proposal. The reason for our selecting Walsh code is that Walsh code has good orthogonal character, which can increase synchronization burst robustness.

Walsh code can be generated by n×n Hadamard matrix.  Hadamard matrix has the following characters:

H20=H1=1,H2r=H2r−1H2r−1H2r−1−H2r−1,r=1,2,…                                        (1)

For example: whenr=1,

H21=H2=H1H1H1−H1=111−1

Whenr=2,

H22=H4=H2H2H2−H2=11111−11−111−1−11−1−11

Walsh code can be generated with the following equation:

W2rn=[H2r]n+1                                     						(2)

The orthogonal character of Walsh code can be expressed:

i=02r−1W2rnW2rm=2r,当m=n时0，当m≠n时                                			(3)

- The performance comparison of before and after Walsh code spread

Figure 4 shows the performance comparison of before and after Walsh code spread. From figure we can find that after Walsh code spread, the performance is better than before and increase 4.5dB. This proves the robustness of synchronize burst index is improved after Walsh code spread.

Step two: we set a BER thresholdε and after SPD received a serial of synchronization bursts, if BER <thresholdε, SPD considered these synchronization bursts were decoded correctly. Whereas if BER >thresholdε, SPD considered these synchronization bursts were decoded inaccurately and they would abort these synchronization burst and wait for new superframe coming. According to the performance curve in figure 4, we set BER with SNR=-1.5dB as threshold.

The sync field shall have the value shown in figure 5.

Bits:  0

1

2

1

3

1

4

0

5

1

6

0

7

1

8

1

9

0

10

0

11

1

12

0

13

0

14

0

1

1

- Format of the sync field

The synchronization header is composed of two fields, the sync field and the index field. The sync field is 15 bits long, and is used by the transceiver to obtain chip and symbol synchronization with incoming data. The index field is 9 bits in length, and is used to obtain frame synchronization with an incoming beacon. Thus the sync burst is 15+9=24 bits long. In a superframe there contains 382 sync bursts. Next we will look for how many sync bursts can satisfy threshold approximately.

- The number of bit <threshold

Figure 6 shows the number of bits<threshold within 30 superframes. From figure we can find average number of bits < threshold is 1717 bits within 30 superframes. That is to say that BER can satisfy threshold by 1717/24=72 sync bursts.

So based on this observation we propose that after SPD receives 72 sync bursts, if BER>threshold, SPD can consider it doesn’t synchronize with PPD. Then it stops receiving spare data in this superframe. If BER<threshold, SPD can consider it synchronize with PPD and continue to receive data until this superframe is over. This method can increase SPD and PPD synchronization speed and avoid resource waste because SPD receives the whole superframe but finally it doesn’t synchronize with PPD.

- Conclusion

In this document, a synchronization burst decoding scheme for beacon device is proposed.  This scheme contains two steps. The first step is that using Walsh code spread sync burst index to increase the robustness. Next the second step we select a thresholdε according to the performance curve of Walsh code. Then we find the number of sync bursts<this threshold. We set the number of sync bursts as a basis. When SPD receive these number of sync bursts, if BER>threshold, SPD consider it doesn’t synchronize with PPD. Then it stops receiving spare data in this superframe. If BER<threshold, SPD can consider it synchronize with PPD and continue to receive data until this superframe is over. This method increase SPD and PPD synchronization speed and avoid resource waste because SPD receives the whole superframe but finally it doesn’t synchronize with PPD.