IEEE P802.22
Wireless RANs

| Proposed text changes to Section A.4 Inter-BS Dynamic Resource Sharing |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-01-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Baowei Ji | Samsung Telecom. America, LP | 1301 E. Lookout Dr. Richardson, TX 75082 | +1-972-761-7167 | Baowei.ji@samsung.com |
| Tae-In Hyon | Samsung Advanced Institute of Technology | Korea | +82-31-280-9503 | Taein.hyon@samsung.com |
| Sang-Jo Yoo | Inha University | Korea | +82-32-860-8304 | sjyoo@inha.ac.kr |

Abstract

This document provides normative text related to Inter-BS Dynamic Resource Sharing for IEEE 802.22 WRAN self coexistence, including both spectrum etiquette and dynamic resource renting and offering, which are optional features in the MAC.

This contribution proposes to include this normative text in the draft.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

A.4 Inter-BS Dynamic Resource Sharing

Finding unoccupied channels to operate in a WRAN system requires intensive computation. Spectrum resource sharing among neighboring WRAN saves computation via cooperation among WRAN and facilitates the opportunistic usage of TV bands.

As shown in Figure 1, four schemes are included for supporting dynamic resource sharing among neighboring cells. When an event for dynamic frequency selection is triggered either internal the cell or from neighboring cells, the cell first tries to address the issue locally following the procedure of spectrum etiquette. If the issue cannot be addressed locally, it proceeds with the dynamic resource renting/offering procedure. If the issue still cannot be addressed, it contends with its neighboring cells for dynamic channel sharing.

Such resource sharing is facilitated by the use of the CBP protocol. CBP allows 802.22 stations (BS and CPEs) to transmit beacons that provide its recipients information to achieve better self-coexistence and resource sharing among overlapping 802.22 cells. These beacons are intended for inter-cell communication and carry specific information about a CPEs association and downstream/upstream bandwidth allocations with the BS, while keeping interference levels to a minimum.

Figure 1. Inter-BS dynamic resource sharing.

A.4.1 Spectrum Etiquette

The channel selection at a cell should obey the spectrum etiquette rule so that the chosen channel does not interfere, or interferes with a minimum number of channels to be used by its neighbour cells. Without cooperation, the frequency selection in a cell may prohibit neighbour cells from normal function. For example, the available channel sets at BS1 and BS2 are {1, 3}, and {1, 2, 3} respectively. If BS2 decides to use {1, 3}, BS1 would have no channel to use. The cooperation is also necessary for load balancing. Say BS1 is heavy loaded while BS2 is not. BS2 could use {2}, then BS1 can use {1, 3}.

The channel-selection decision of each cell should follow the flowchart in Figure 2. The terms of central cell and neighbour cells are relative concept. Beside the candidate and active sets, Fpool and Flocal are defined and used for describing the principles of spectrum etiquette.

- Fcandidate set, CellID := the frequencies that do not interfere incumbent uses.

- Factive set, CellID := the frequencies that the cell has picked up.

- Fpool := the frequencies that are usable in the central cell and are not used by neighbour cells := Fcandidate set, CellID \ (union of the candidate sets of all its neighbour cells)

- Flocal := Fpool \ {union of the active sets of all its neighbour cells}

Note that:

- symbols U, ∩, and \ are set operation of union, intersection, and exclude, respectively.

- CellID is cell ID. Note that Fpool and Flocal are local information, which are not shared with neighbour cells. That is why they do not need CellID.

The procedure of WRAN spectrum etiquette is as follows.

- The central cell decides its Fcandidate set, and Fpool.

- The central cell picks up frequencies from the Fpool according to the following etiquette principles.

  - Try to use the frequencies that cannot be used by neighbor cells at all. In other words, use first the frequencies in Flocal

  - If the central cell has got enough frequencies, go to Step 3. Otherwise, try to pick up frequencies from the rest of Fpool with the consideration of avoid using those that will affect most of its neighbour cells. For example, Use first the frequencies that are not shared by more than one neighbour cells, then other frequencies that may affect more and more neighbour cells.

  - If the central cell has got enough frequencies, go to Step 3. Otherwise, it should resorts to a contention scheme for sharing a channel with its neighbor.

- Update neighbour cells of its Fcandidate set, and Factive set. Go back to Step 1.

Figure 2 The flow chart of channel selection and update with neighbors.

- RS-SEM message format

| Syntax | Size | Notes |
| --- | --- | --- |
| RS-SEM_Message_Format() { |  |  |
| Management Message Type = 60 | 8 bits |  |
| BS ID | 48 bits | Unique identifier for this BS. |
| Channel Number (1st active channel) | 8 bits |  |
| Channel Number (2nd active channel) | 8 bits |  |
| Channel Number (3rd active channel) | 8 bits |  |
| Channel Number (1st candidate channel) | 8 bits |  |
| Channel Number (2nd candidate channel) | 8 bits |  |
| Channel Number (3rd candidate channel) | 8 bits |  |
| Channel Number (4th candidate channel) | 8 bits |  |
| Channel Number (5th candidate channel) | 8 bits |  |
| } |  |  |

Add the following timer to Table 237.

- Table 237 —General parameter setting

| Entity | Name | Time reference | Minimum value | Default value | Maximum value |
| --- | --- | --- | --- | --- | --- |
| BS | Interval for spectrum etiquette (Tse) | Time between transmission of the broadcast message of the active and candidate channel sets for the purpose of dynamic resource sharing. |  |  | 60 s |

A.4.1.1 Illustration of the procedure of spectrum etiquette:

Spectrum etiquette operates no matter sectorization is used or not. The procedure is described above based on non-sectorization cells. To completely describe it, the following example is given for the case of sectorization, i.e., six sectors per cell, each sector with three neighboring sectors.

The central sector of Figure 3 has three neighbor sectors (N1, N2, and N3). Neighbor sector N1 has candidate channel set {1, 2, and 3}, with channel 2 being used. The similar messages for N2 and N3 are shown in the table. The pool of available channels (i.e., Fpool) of the central sector are {1, 3, 4, 6, 7}. If the central sector needs only one channel, it would pick up channel 7 which is the best candidate according to Spectrum Etiquette principles because that channel is not usable for any neighbors. If the central sector needs one more channel, it would pick up one randomly from channel set {3, 4, 6}. In the example, channel 4 is chosen. Finally, if totally three channels are needed by the central sector, it would pick up the third channel randomly from the set {3, 6}, and finishes the channel selection procedure.

Figure 3 The illustration of spectrum etiquette operation.

A.4.2 Overall Spectrum Renting/Offering

And inter-BS dynamic resource sharing is especially useful when a BS does not acquire the enough resources by itself and in the other hands a neighbor BS has enough resources to share with its neighbor BSs. WRAN systems may log the channel transactions and the settlement will be made afterward. Or, we may ask the WRAN systems to be supportive of each other and share the resources they have acquired for free.

Offerors can share active channel sets with potential renters. Offerors can broadcast their available amount of resources per each active channel.

Renters decide the amount of required resources and the required rental time from the neighbor offeror’s broadcasts.  And inform them to the selected neighbor offerors.

The channel selection at the renting base station should obey the spectrum etiquette rule. If the current channel utilization of the active channels is lower than the predetermined threshold that is maintained by network operator, the offeror BS can share the resource with other BSs.

If the current channel utilization of the active channels is higher than the predetermined threshold that is maintained by network operator and the BS expects that user’s traffic demands exceed the current available capability including candidate or backup channels, then the BS can initiate renting procedure from other offeror BSs.

Figure 2 - Scenario for the Dynamic Resource Sharing.

Figure 3- Spectrum Renting/Offering Procedure

Figure 4- The Whole Procedure

A.4.2.1  Dynamic Resource Offering procedure

In offering, the offeror initiates offering procedure to share its resource by broadcasting the resource advertisement message. The message contains the channel set information such as offering channel set, each offering channel’s available resource amount. The renter in need of resource makes a renting request on a specific channel in the offeror’s active channels. The renting request includes the amount of required resources and renting time. Finally, the offeror finishes the procedure by sending acknowledgement to the renter BS.

On sharing resource with renters, Collecting and Returning procedure can be implemented.

* Returning Procedure: when the renters don’t need some of all of borrowed resource from offeror anymore, it can return the borrowed resources to offeror.

* Collecting Procedure: when the offeror needs more resources it can collect (withdraw) lending resources from renter. Collecting should be agreed by the renter.

The procedure in detail:

1). 	The offeror initiates offering to share its resource by broadcasting the Resource Advertisement using SCH (add RS field to SCH, RS=1)

2). 	When the offeror BS receives Rent Request which contains information about desired resource and the renting time through CBP packet (in self-coexistence), it sends Resource Allocation (RA-RSP) message in self-coexistence of the renter.

Resource Allocation message includes Resource Allocation bit flag and information of the channel set which is possible for sharing.

If offeror can support the channel requested by the renter, Resource Allocation bit flag is 1, else Resource Allocation bit flag is 0.

3). 	The offeror shares the resources with the renter BS by receiving acknowledgement.

4) 	The renter may return the borrowed resources before finishing the original rental time.

5) 	The offeror may collect the resource before finishing the rental time.

Collecting

Returning

Figure 5– The Flow Chart of Offeror Procedure

A.4.2.2  Dynamic resource renting procedure

When a base station needs more resources and it is not able to get the desired resources form the current active and candidate channels, it can start dynamic resource renting procedure. The renter BS will scan neighbor BS’s active channel and wait to hear Resource Advertisement message from the neighbor BS’s SCH. If the renter BS finds out the offeror that can support the renter BS’s demands, then it sends Rent Request with the desired amount of resources and rental time. Upon reception of successful Resource Allocation message from the offeror, it replies with ACK.

The renter BS may actively initiate resource returning procedure when it does not need some of all of the borrowed resources. When the renter BS receives the resource collection request from the offeror BS, it should let the offeror know that the collection request is accepted or rejected by the renter BS.

The procedure in detail:

1). 	When the renter BS hears the Resource Advertisement message through the SCH (RS=1), the renter BS tries contention for Rent Request (RR-REQ) in self-coexistence of the offeror. Then renter BS sends Rent Request message which contains information about desired resources and the renting time through the CBP packet (in self-coexistence).

2). 	Upon reception of the Resource Allocation (RA-RSP) message including Resource Allocation bit flag and information of the channel set which is possible for sharing, the renter BS confirms Resource Allocation bit flag.

3)	If Resource Allocation bit flag is 1, the renter BS sends the RA-ACK in self-coexistence of the offeror. Else if Resource Allocation bit flag is 0, the renter BS adjusts Resource to request and retries to sending the RR

4). 	If the renter BS does not receive the RA message until timeout, it will retry resource sharing to another BS.

5). 	The renter BS may return the borrowed resource before finishing the rental time.

The renter BS sends Returning Request (RE-REQ) message including information of channel to return in self-coexistence of the offeror.

6). 	Offeror may collect the resource before finishing the rental time.

If the offeror BS requests Resource Collection (RC-REQ), the renter BS decides acceptance or rejection, then it sends RC response.

▪ RC response bit flag = 1 : tt indicates “Accept”.

▪ RC response bit flag = 0 : tt indicates “ Reject”.

Returning

Collecting

Figure 6– The Flow Chart of Renter Procedure

Message Definitions

- SCH RS : Resource Sharing field in Superframe Control Header.

If RS is 1, it Indicates the resource advertisement.

- RR-REQ : Rent Request Management message.

- RA-RSP : Resource Allocation Management message.

  - Includes Resource Allocation bit flag and information of the channel set (Active Set or Candidate/Backup Set) which is possible for sharing.

  - Allocation bit flag : indicates whether offeror BS supplies the channel requested by Renter or not.

0 =dissatisfaction , 1=satisfaction

- RA-ACK : Resource Allocation Acknowledge Management message.

- RC-REQ : Resource Collection Request Management message.

- RC-RSP : Resource Collection Response Management message.

  - Includes RC response bit flag : indicates Accept or  Reject

0 = Reject, 1= Accept

- RC-ACK : Resource Collection Response Acknowledge Management message.

- RE-REQ : Returning Request Management message.

- RE-RSP : Returning Response Management message.

- RE-ACK : Returning Response Acknowledge Management message.

Message Transmission

Figure 7– message transmission position

- —Superframe control header format

| Syntax | Size | Notes |
| --- | --- | --- |
| Superframe_Control_Header_Format() { |  | 1 OFDM symbol long and transmitted with well-known modulation/coding (e.g., QPSK rate ½) |
| ST = 0 | 6 bits | System Type Indicates the type of the system using this band. |
| CT | 1 bits | Content Type Indicates the type of the content following the transmission of the SCH. Superframe = 0 CBP Beacon = 1 |
| RS | 1 bits | Resource Sharing field Indicates the resource advertisement resource advertisement =1 normal =0 |
| Superframe Number | 8 bits | Positive integer that represents the superframe number (modulo 255). This field shall be incremented by 1 upon every new superframe. |
| FS | 4 bits | Frames per Superframe Indicates the number of frames within a superframe. Frames within a superframe have a fixed size which shall not change across superframes. |
| FDC | 8 bits | Frame Duration Code |
| TTQP | 16 bits | Time To Quiet Period The amount of time it will take for the next scheduled quiet period. This serves to synchronize the quiet period of overlapping BSs., the TTQP is divided into two subfields: Time Scale and Time. The Time Scale subfield defines the scale for the Time subfield. The Time subfield consists of a 15 bit unsigned integer number. |
| DQP | 16 bits | Duration of Quiet Period The estimated duration of the next scheduled quiet period. This is specified similar to the TTQP field. |
| PP | 1 bit | Preamble Present Indicates whether the preamble of the following frame is present or not. For example, a frame preamble may not be needed when the cell is operating in a single physical (i.e., TV) channel |
| Tx ID | 48 bits | Address that uniquely identifies the transmitter of the SCH (CPE or BS) |
| CN | 8 bits | Channel Number Indicates the starting physical (i.e., TV) channel number used by the BS. |
| NC | 2 bits | Number of Channels In case channel bonding is used, this field indicates the number of additional consecutive physical (i.e., TV) channels used by the BS. In the basic mode, NC = 2 (i.e., two additional TV channels). This translates into a total of 3 physical channels being bonded. |
| AW Present | 1 bit | Alert Window (AW) present Determines whether the AW is present in the superframe structure as part of the first frame. 0 = AW not present 1 = AW present |
| BFB | 16 bits | Bonded Frame Bitmap In case the BS is operating under bonding mode but single channel CPEs have to be supported, this field indicates frame-by-frame in the superframe which ones are bonded and which ones are single channel Bit = 0: the corresponding frame is bonded Bit = 1: the corresponding frame per TV channel |
| GIF | 1 bit | Guard Interval Factor Specifies the GIF used by the PHY in the frame transmissions of this superframe. Pre-determined values are: 4 = Default mode used for superframe transmission |
| Length | 8 bits | The length of the information following the SCH |
| IEs | Variable | Optional Information Elements which could be transmitted with the SCH. They are: MAC version Current transmit power Part 74 acknowledgement Location configuration information |
| HCS | 8 bits | Header Check Sequence |
| } |  |  |

- —RR-REQ message format

| Syntax | Size | Notes |
| --- | --- | --- |
| RR-REQ_Message_Format() { |  |  |
| Management Message Type = 61 | 8 bits |  |
| BS ID | 48 bits | Unique identifier for this Renter BS. Offeror BS can distinguish message from different collocated BSs. |
| Number of Channels | 8 bits | The number of channels which Renter BS wants to share |
| Renting Time | 8 bits | The time that Renter BS want to borrow resource. |
| } |  |  |

- —RA-RSP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| RA-RSP_Message_Format() { |  |  |
| Management Message Type =62 | 8 bits |  |
| BS ID | 48 bits | Offeror Base Station ID |
| Allocation bit flag | 1 bits | Indicates whether offeror BS supplies the channel requested by Renter or not. 0 =dissatisfaction 1=satisfaction |
| Number of Channels | 8 bits | The number of channel which is possible for sharing. |
| } |  |  |

- —RA-ACK message format

| Syntax | Size | Notes |
| --- | --- | --- |
| RA-ACK_Message_Format() { |  |  |
| Management Message Type = 63 | 8 bits |  |
| BS ID | 48 bits | Renter Base Station ID |
| Confirmation Code | 8 bits | Table 82 |
| } |  |  |

- —RC-REQ message format

| Syntax | Size | Notes |
| --- | --- | --- |
| RC-REQ_Message_Format() { |  |  |
| Management Message Type = 64 | 8 bits |  |
| BS ID | 48 bits | Offeror Base Station ID |
| Number of Channels | 8 bits | The number of channel that offeror BS wants to collect. |
| } |  |  |

- —RC-RSP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| RC-RSP_Message_Format() { |  |  |
| Management Message Type = 65 | 8 bits |  |
| BS ID | 48 bits | Renter Base Station ID |
| RCR bit flag | 1 bits | Resource Collection Response bit flag Indicates Accept or Reject 0 = Reject 1= Accept |
| } |  |  |

- —RC-ACK message format

| Syntax | Size | Notes |
| --- | --- | --- |
| RC-ACK_Message_Format() { |  |  |
| Management Message Type =66 | 8 bits |  |
| BS ID | 48 bits | Offeror Base Station ID |
| Confirmation Code | 8 bits | Table 82 |
| } |  |  |

- —RE-REQ message format

| Syntax | Size | Notes |
| --- | --- | --- |
| RE-REQ_Message_Format() { |  |  |
| Management Message Type = 67 | 8 bits |  |
| BS ID | 48 bits | Renter Base Station ID |
| Number of Channels | 8 bits | The number of channel that Renter BS wants to return. |
| } |  |  |

- —RE-RSP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| RE-RSP_Message_Format() { |  |  |
| Management Message Type = 68 | 8 bits |  |
| BS ID | 48 bits | Offeror Base Station ID |
| Confirmation Code | 8 bits | Table 82 |
| } |  |  |

- —RE-ACK message format

| Syntax | Size | Notes |
| --- | --- | --- |
| RE-ACK_Message_Format() { |  |  |
| Management Message Type = 69 | 8 bits |  |
| BS ID | 48 bits | Renter Base Station ID |
| Confirmation Code | 8 bits | Table 82 |
| } |  |  |

- Table 82—Confirmation Code (CC) values

| CC | Status |
| --- | --- |
| 0 | OK/success |
| 1 | reject-other |
| 2 | reject-unrecognized-configuration-setting |
| 3 | reject-temporary / reject-resource |
| 4 | reject-permanent / reject-admin |
| 5 | reject-not-owner |
| 6 | reject-service-flow-not-found |
| 7 | reject-service-flow-exists |
| 8 | reject-required-parameter-not-present |
| 9 | reject-header-suppression |
| 10 | reject-unknown-transaction-id |
| 11 | reject-authentication-failure |
| 12 | reject-add-aborted |
| 13 | reject-exceed-dynamic-service-limit |
| 14 | reject-not-authorized-for-the-request-SAID |
| 15 | reject-fail-to-establish-the-requested-SA |
| 16 | reject-not-supported-parameter |
| 17 | reject-not-supported-parameter-value |