IEEE P802.22
Wireless RANs

| Modified CAZAC Sequences Based Low PAPR Preambles |  |  |  |  |
| --- | --- | --- | --- | --- |
| Last Revised - Date: 2007-01-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Wai Ho Mow | HKUST | Hong Kong, China | 852-2358-7070 | eewhmow@ece.ust.hk |
| Vincent K. N. Lau | HKUST | Hong Kong, China | 852-2358-7066 | eeknlau@ece.ust.hk |
| Roger S. Cheng | HKUST | Hong Kong, China | 852-2358-7072 | eecheng@ece.ust.hk |
| Ross D. Murch | HKUST | Hong Kong, China | 852-2358-7044 | eermurch@ece.ust.hk |
| Khaled Ben Letaief | HKUST | Hong Kong, China | 852-2358-7064 | eekhaled@ece.ust.hk |
| Linjun Lu | Huawei Technologies | Shenzhen, China | 0086-755-28973119 | lvlinjun@huawei.com |
| Soo-Young Chang | Huawei Technologies | Davis, CA, U.S. | 1-916 278 6568 | sychang@ecs.csus.edu |
| Jianwei Zhang | Huawei Technologies | Shenzhen, China | 86-21-68644808 | zhangjianwei@huawei.com |
| Lai Qian | Huawei Technologies | Shenzhen, China | 86-755-28973118 | qlai@huawei.com |
| Jianhuan Wen | Huawei Technologies | Shenzhen, China | 86-755-28973121 | wenjh@huawei.com |

Abstract

In this document, modified Constant Amplitude Zero Autocorrelation (CAZAC) sequences are proposed to specify preambles in the frequency domain. The proposed preambles have very low peak-to-average-power ratio (PAPR) of at most 1.93dB, and at most 2.55 dB when extending to a set of 114 sequences. By design, similar CAZAC-like sequences can be used as channel sounding sequences such that the implementation cost of the preamble generator can be much reduced by sharing the common lookup table and computations.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

| Co-Author(s): |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | Company | Address | Phone | email |
| Jianhua Sun | HKUST | Hong Kong, China | 852-2358-7086 | sunjh@ece.ust.hk |
| Edward K. S. Au | HKUST | Hong Kong, China | 852-2358-7086 | eeedward@ece.ust.hk |
| Zhou Wu | Huawei Technologies | Shenzhen, China | 86-755-28979499 | wuzhou@huawei.com |
| Jun Rong | Huawei Technologies | Shenzhen, China | 86-755-28979499 | rongjun@huawei.com |
| Jian Jiao | Huawei Technologies | Beijing, China | 86-10-82882751 | Jiao_jian@huawei.com |
| Meiwei Jie | Huawei Technologies | Shenzhen, China | 86-755-28972660 | jiemingwei@huawei.com |

Modified CACAZ Sequences Based Low PAPR Preambles

1.  Introduction

In the current IEEE802.22 draft v0.2 [1], preambles are formed by QPSK symbols with I and Q components generated by two binary PN sequences, respectively. However, the frame and superframe preambles currently specified have high peak-to-average-power ratios (PAPR) (> 7 dB). High-PAPR preambles may be clipped by the power amplifier, resulting in lower synchronization and channel estimation accuracy and hence degraded detection performance. The PAPR of preambles should be minimized as much as possible so as to allow improved performance by boosting up the transmission power of preambles relative to that of the data signals. This is important especially when some effective methods (e.g. clipping, coding and companding) for reducing the PAPR of the data modulation signals may be applied. Furthermore, to reduce the adverse effect of adjacent cell interference on the synchronization and channel estimation accuracy, a set of low-PAPR preambles with low corss-correlation energy may be used.

This constribution proposes modified Costant Amplitude Zero Autocorrelation (CAZAC) sequences with smallest known PAPR values for a given length, to the best of our knowledge, to specify preambles in the frequency domain. The proposed preambles have a PAPR value of at most 1.93dB (and at most 2.55 dB when extending to a set of 114 sequences). By design, similar CAZAC-like sequences can be used as channel sounding sequences (e.g. GCL sounding sequences specified in the current draft) such that the implementation cost of the preamble generator can be much reduced by sharing the common lookup table and computations.

2.  Preamle Design

The preambles proposed in this contribution are based on a very general construction of M-phase CAZAC sequences (in the M-PSK format), i.e. Mow’s unified perfect roots-of-unity sequences (PRUS) [2], which include the well-known Frank, Chu and GCL sequences. It was proved by an exhaustive search that the unified PRUS construction includes all M-phase CAZAC sequences with M £ 15, sequence length L £ 20 and LM £ 1111. It was conjectured that no more unknown M-phase CAZAC sequences exist [3].  In the unified PRUS construction, an M-phase CAZAC sequence sCAZAC of length L = sm2 can be expressed as

sCAZAC(km+l)=expi2πmc(s)α(l)k2+β(l)k+γ(l)sm∀l∈Zm and ∀k∈Zsm,wherec(s)=1/2     if mod(s,2)=0		(1)

A preamble specified by the frequency-domain sequence of length Lpreamble can be obtained by modifying a properly selected sCAZAC , whose parameters s, m, α(l), β(l),  (l) are optimized to give a low PAPR value, where the required value Lpreamble is dependent on the FFT size, the number of (left and right) guard carriers and the decimation factor. Here the phase alphabet size M of the modified CAZAC sequence is restricted to a certain power of 2, say, M  = 2n, so as to allow the straightforward generation of the sequence by storing nLpreamble bits, at most.

A more memory-efficient implementation is to generate the integer indices based on Equation (1) and perform table lookup to obtain the corresponding I and Q respresentations. Note that only a lookup table of I and Q representations of the integer phase indices in the range from 0 to M/4 – 1 = 2n-2 – 1, corresponding to the phase angles in [0, π/4), need to be stored since multiplication by ±1 or ±j can be computed with little complexity. We estimate that the generation of the integer indices of the sequence requires 1 multiplication and 3~4 additions per sequence element. As will be demonstrated below, the modified CAZAC sequences can be extended to form a sequence set with low cross-correlation energy for use as preambles with high resistant to adjacent cell interference or as channel sounding sequences. In the case that these or other CAZAC-like sequences (e.g. the GCL sequences specified in the current draft) are used as sounding sequences, the aforementioned lookup table and phase index computations can be shared for the generation of both preambles and sounding sequences. Consequently, the implementation cost of the preamble generator can be much reduced.

Our PAPR values are estimated for continuous-time waveforms using an oversampling factor of 4. Without oversampling, the obtained PAPR values may be over-optimistic. The PAPR values of our proposed preambles for different modes are listed in Table 1 for M = 128 and 32. Table 2 lists the PAPR values when the number of bonded TV channels Nbands is larger than 1.

Table 1. PAPR of Preambles for FFT Size = 2048

| FFT Size = 2048 Null subcarriers [L=184, DC, R=183] | Modified CAZAC (128 phases) | Modified CAZAC (32 phases) |
| --- | --- | --- |
| Frame Short Preamble (Decimation factor = 4) | 1.88 dB | 2.03 dB |
| Frame Long Preamble (Decimation factor = 2) | 1.81 dB | 2.02 dB |
| Superframe Short Preamble (Decimaction. Factor = 4) | 1.93 dB | 2.07 dB |
| Superframe Long Preamble (Decimation factor = 2) | 1.81 dB | 1.97 dB |

Table 2. PAPR of Preambles for Nbands > 1

| FFT Size = 2048*Nbands Number of Null Subcarriers = 368* Nbands | Modified CAZAC (128 phases) | Modified CAZAC (32 phases) |
| --- | --- | --- |
| Frame Short Preamble (Nbands = 2, Decimation factor = 4) | 1.79 dB | 2.08 dB |
| Frame Long Preamble (Nbands = 2, Decimation factor = 2) | 1.69 dB | 2.09 dB |
| Frame Short Preamble (Nbands = 3, Decimation factor = 4) | 1.75 dB | 2.04 dB |
| Frame Long Preamble (Nbands = 3, Decimation factor = 2) | 1.75 dB | 2.14 dB |

From the tables, the PAPR reduction as compared to the preambles based on PN sequences in the current Draft v0.2 is at least 5.87 dB. It can be observed that by reducing the phase alphabet size from 128 to 32 and thus the lookup table size from 32 to 8 pairs of I/Q representations, the resultant PAPR values are still very low and the worst-case PAPR is only increased mildly from 1.93dB to 2.14dB. However, in our opinion, the memory requirement for the proposed 128-phase sequences is very affordable.

To reduce the adverse effect of adjacent cell interference on the synchronization and channel estimation accuracy, a set of low-PAPR preambles with low cross-correlation energy may be applied. Our preamble design can also be extended to generate a set of low-PAPR sequences. The PAPR of our modified CAZAC sequence set and that of Chu set (i.e. the GCL set mentioned in the current draft) are shown in Figure 1, where the set size is assumed to be 114. It can be seen from Figure 1 that the worst case PAPR of the proposed set is 2.55dB, which is about 2.2dB better than the Chu set. The result also justifies the aforementioned claim that the proposed preamble design can be extended to a set of low-PAPR sequences suitable for use as channel sounding sequences.

Figure 1. PAPR of A Set of 114 Modified CAZAC Sequences

The cumulative distribution function (CDF) of this set of 114 modified CAZAC sequences is shown in Figure 2.

Figure 2. The CDF of PAPR of A Set of 114 Modified CAZAC Sequences

3.  Conclusion

CAZAC sequences based on the unified PRUS construction [2] were modified to obtain preambles with very low PAPR (£ 1.93dB for the 2K, 4K and 6K-FFT modes). It was also shown that the preamble design can be extended to form a set with preambles or sounding sequences with worst case PAPR £ 2.55dB for up to 114 interfering cells. The implementation cost of the preamble generator can be much reduced by sharing the common lookup table and phase index computations for generating both preambles and CAZAC-like sounding sequences. We therefore propose the use of modified CAZAC sequences based low-PAPR preambles to replace the existing PN-sequence based preambles specified in the current draft.

References:

[1] 	IEEE 802.22/D0.2, Draft Standard for Wireless Regional Area Networks Part22: Cognitive Wireless RAN Medium Access Control (MAC) and Physical Layer (PHY) specifications: Policies and procedures for operation in the TV Bands, IEEE Standard, November 2006.

[2] 	W. H. Mow, “A New Unified Construction of Perfect Root-of-Unity Sequences,” in Proceedings of IEEE 4th International Symposium on Spread Spectrum Techniques and Applications (ISSSTA'96), Germany, September 1996, pp. 955-959.

[3] 	H. D. Lüke, et al. “Binary and quadriphase sequences with optimal autocorrelation properties: a survey,” IEEE Transactions on Information Theory, vol. 49, pp.3271-3282, December 2003.