IEEE P802.22
Wireless RANs

| Completed Definition of Duo-Binary Turbo Code with sub-channel concatenation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-01-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| John Benko | France Telecom R&D | 801 Gateway Blvd. Suite 500, South San Francisco, CA 94080 | 650-875-1593 | john.benko@orange-ftgroup.com |

Abstract

Completed definition of Duo-Binary Turbo Code.  Definition and code is not changed.  Additional information includes support for a large range of block sizes to optimize code for each MCS/sub-channelization.  Sub-channel concatenation rules are also defined for OFDMA transmission.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

      - Duo-binary convolutional Turbo code (CTC) mode (optional)

        - Duo-binary convolutional turbo coding

The duo-Binary Turbo Code is illustrated in Figure 1. It uses a Circular Recursive Systematic Convolutional (CRSC) Code as component codes, with double-binary input.

The bits of the data to be encoded are alternately fed to A and B, starting with the MSB of the first byte being fed to A. The encoding system is fed by blocks of k bits or N couples (k=2xN). N is a multiple of 4 (k is a multiple of 8) and should be between 32 and 4096.

- —Duo-binary convolutional turbo code: Encoding scheme

The polynomials, which shall be used for the connections, are described in octal and symbolic notations as follows:

- for the feedback branch: 15 (in octal), equivalently 1+D+D3 (in symbolic notation);

- for the Y1 and Y2 parity bits, 13, equivalently 1+D2+D3;

The input A shall be connected to tap “1” of the shift register and the input “B” shall be connected to the input taps “1”, D and D2.

This first encoding is called C1 encoding. After initialisation by the circulation state Sc1, the encoder shall be fed by the sequence in the natural order with incremental address i = 0,…,N-1.

This second encoding is called C2 encoding. After initialisation by the circulation state Sc2, the encoder shall be fed by the interleaved sequence with incremental address j = 0,… N-1.

The function (j) that gives the natural address i of the considered couple, when reading it at place j for the second encoding, is given in 1.1.1.2.2.

        - CTC interleaver

In the CTC interleaver, the permutation shall be done on two levels:

- The first one inside the couples (level 1)

- The second one between couples (level 2)

The permutation is described in the following algorithm.

- Set the permutation parameters P0, P1, P2 and P3. These parameters depend on the size of the sequence to be encoded. The table below gives the block size, code rates, and code parameters for the different modulation and coding schemes.  Optimized code parameters are given for data block sizes upto 240 bytes.

- j = 0,… N-1.

- level 1

if j mod. 2 = 0, let (A,B) = (B,A) (invert the couple)

- level 2

- if j mod. 4 = 0, then P = 0;

- if j mod. 4 = 1, then P = N/2 + P1;

- if j mod. 4 = 2, then P = P2;

- if j mod. 4 = 3, then P = N/2 + P3.

- i = P0*j + P + 1 mod. N

| Data block size (byte) | Encoded data block size (bytes) |  |  |  |  |  |  |  |  |  | N | P0 | P1 | P2 | P3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QPSK |  | 16QAM |  |  |  | 64QAM |  |  |  |  |  |  |  |  |
|  | 1/2 | 3/4 |  | 1/2 | 3/4 | 1/2 |  | 2/3 | 3/4 | 5/6 |  |  |  |  |  |
| 6 | 12 | - |  | - | - | - |  | - | - | - | 24 | 5 | 0 | 0 | 0 |
| 9 | - | 12 |  | - | - | - |  | - | - | - | 36 | 11 | 18 | 0 | 18 |
| 12 | 24 | - |  | 24 | - | - |  | - | - | - | 48 | 13 | 24 | 0 | 24 |
| 18 | 36 | 24 |  | - | 24 | 36 |  | - | - | - | 72 | 11 | 6 | 0 | 6 |
| 24 | 48 | - |  | 48 | - | - |  | 36 | - | - | 96 | 7 | 48 | 24 | 72 |
| 27 | - | 36 |  | - | - | - |  | - | 36 | - | 108 | 11 | 54 | 56 | 2 |
| 30 | 60 | - |  | - | - | - |  | - | - | 36 | 120 | 13 | 60 | 0 | 60 |
| 36 | 72 | 48 |  | 72 | 48 | 72 |  | - | - | - | 144 | 17 | 74 | 72 | 2 |
| 45 | - | 60 |  | - | - | - |  | - | - | - | 180 | 11 | 90 | 0 | 90 |
| 48 | 96 | - |  | 96 | - | - |  | 72 | - | - | 192 | 11 | 96 | 48 | 144 |
| 54 | 108 | 72 |  | - | 72 | 108 |  | - | 72 | - | 216 | 13 | 108 | 0 | 108 |
| 60 | 120 | - |  | 120 | - | - |  | - | - | 72 | 240 | 13 | 120 | 60 | 180 |
| 66 | 132 | - |  | - | - | - |  | - | - | - | 264 | 23 | 2 | 160 | 30 |
| 72 | 144 | 96 |  | 144 | 96 | 144 |  | 108 | - | - | 288 | 23 | 50 | 188 | 50 |
| 78 | 156 | - |  | - | - | - |  | - | - | - | 312 | 23 | 102 | 64 | 38 |
| 81 | - | 108 |  | - | - | - |  | - | 108 | - | 324 | 11 | 172 | 164 | 16 |
| 90 | 180 | 120 |  | - | 120 | 180 |  | - | - | 108 | 360 | 29 | 56 | 0 | 68 |
| 96 | 192 | - |  | 192 | - | - |  | 144 | - | - | 384 | 29 | 68 | 140 | 56 |
| 99 | - | 132 |  | - | - | - |  | - | - | - | 396 | 29 | 36 | 128 | 76 |
| 102 | 204 | - |  | - | - | - |  | - | - | - | 408 | 29 | 124 | 204 | 40 |
| 108 | 216 | 144 |  | 216 | 144 | 216 |  | - | 144 | - | 432 | 13 | 0 | 4 | 8 |
| 114 | 228 | - |  | - | - | - |  | - | - | - | 456 | 31 | 100 | 224 | 104 |
| 117 | - | 156 |  | - | - | - |  | - | - | - | 468 | 31 | 98 | 220 | 98 |
| 120 | 240 | - |  | 240 | - | - |  | 180 | - | 144 | 480 | 31 | 52 | 240 | 52 |
| 132 | 264 | - |  | 264 | - | - |  | - | - | - | 528 | 31 | 24 | 36 | 104 |
| 135 | - | 180 |  | - | - | - |  | - | 180 | - | 540 | 31 | 42 | 248 | 34 |
| 138 | 276 | - |  | - | - | - |  | - | - | - | 552 | 35 | 14 | 136 | 6 |
| 144 | 288 | 192 |  | 288 | 192 | 288 |  | 216 | - | - | 576 | 31 | 42 | 232 | 18 |
| 150 | 300 | - |  | - | - | - |  | - | - | 180 | 600 | 37 | 20 | 152 | 0 |
| 153 | - | 204 |  | - | - | - |  | - | - | - | 612 | 37 | 6 | 164 | 14 |
| 156 | 312 | - |  | 312 | - | - |  | - | - | - | 624 | 37 | 312 | 156 | 468 |
| 162 | 324 | 216 |  | - | 216 | 324 |  | - | 216 | - | 648 | 37 | 62 | 160 | 34 |
| 171 | - | 228 |  | - | - | - |  | - | - | - | 684 | 37 | 108 | 136 | 8 |
| 174 | 348 | - |  | - | - | - |  | - | - | - | 696 | 37 | 0 | 128 | 12 |
| 180 | 360 | 240 |  | 360 | 240 | 360 |  | - | - | 216 | 720 | 37 | 92 | 100 | 68 |
| 186 | 372 | - |  | - | - | - |  | - | - | - | 744 | 37 | 54 | 196 | 50 |
| 192 | 384 | - |  | 384 | - | - |  | 288 | - | - | 768 | 19 | 384 | 216 | 600 |
| 198 | 396 | 264 |  | - | 264 | 396 |  | - | - | - | 792 | 41 | 0 | 228 | 24 |
| 204 | 408 | - |  | 408 | - | - |  | - | - | - | 816 | 37 | 408 | 204 | 612 |
| 207 | - | 276 |  | - | - | - |  | - | - | - | 828 | 41 | 136 | 288 | 192 |
| 216 | 432 | 288 |  | 432 | 288 | 432 |  | 324 | 288 | - | 864 | 19 | 2 | 16 | 6 |
| 222 | 444 | - |  | - | - | - |  | - | - | - | 888 | 43 | 10 | 220 | 18 |
| 225 | - | 300 |  | - | - | - |  | - | - | - | 900 | 43 | 8 | 56 | 20 |
| 228 | 456 | - |  | 456 | - | - |  | - | - | - | 912 | 43 | 96 | 8 | 124 |
| 234 | 468 | 312 |  | - | 312 | 468 |  | - | - | - | 936 | 43 | 120 | 140 | 124 |
| 240 | 480 | - |  | 480 | - | - |  | 360 | - | 288 | 960 | 43 | 52 | 120 | 28 |

        - Determination of the circulation states

The state of the encoder is denoted S (0  S  7) with S = 4.s1 + 2.s2 + s3 (see Table 2). The circulation states Sc1 and Sc2 shall be determined by the following operations:

- Initialize the encoder with state 0. Encode the sequence in the natural order for the determination of Sc1 or in the interleaved order for the determination of Sc2 (without producing redundancy). In both cases, the final state of the encoder is denoted SN−10.

- According to the length N of the sequence, the following correspondence shall be used to find Sc1 and Sc2 (see Table 1).

- —Circulation state correspondence table

| SN−10 Nmod.7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Sc=0 | Sc=6 | Sc=4 | Sc=2 | Sc=7 | Sc=1 | Sc=3 | Sc=5 |
| 2 | Sc=0 | Sc=3 | Sc=7 | Sc=4 | Sc=5 | Sc=6 | Sc=2 | Sc=1 |
| 3 | Sc=0 | Sc=5 | Sc=3 | Sc=6 | Sc=2 | Sc=7 | Sc=1 | Sc=4 |
| 4 | Sc=0 | Sc=4 | Sc=1 | Sc=5 | Sc=6 | Sc=2 | Sc=7 | Sc=3 |
| 5 | Sc=0 | Sc=2 | Sc=5 | Sc=7 | Sc=1 | Sc=3 | Sc=4 | Sc=6 |
| 6 | Sc=0 | Sc=7 | Sc=6 | Sc=1 | Sc=3 | Sc=4 | Sc=5 | Sc=2 |

        - Code rate and puncturing

Four code rates are defined here, to match those define for the mandatory convolution code: R = ½, 2/3, ¾ and 5/6.  These rates shall be achieved through selectively deleting (punct~C4~C4.1~251~4~T~The duo-binary turbo is missing some parts.  Specifically the interleaver parameters (P,P1,P2,P3) are not defined for relevant block sizes for 802.22.  In addition the block concatention scheme is not defined.~The missing parts are included in an updated version of the duo-binary turbo code.  The changes are only the addition of the interleavers parameters, the concatention scheme, and a puncturing scheme for a rate 5/6 code.  Everything else remains the same~~X~Ouring) the parity bits. The puncturing pattern defined in Table 2 shall be applied, to both codes C1 and C2, even when the number of couples is not a multiple of 5 for rate 5/6, or of 6 for rates ½, 2/3, and ¾ .

- —Puncturing patterns for turbo codes (“1”=keep, “0”=delete)

| Code Rate | Puncturing vector |
| --- | --- |
| 1/2 | Y = [1 1 1 1 1 1] |
| 2/3 | Y = [1 0 1 0 1 0] |
| 3/4 | Y = [1 0 0 1 0 0] |
| 5/6 | Y = [1 0 0 0 0] |

        - Channel Concatenation

The concatenation scheme of the CTC blocks is defined to allow data block sizes up to 240 bytes.  The rules to be used with the concatenation scheme are defined fully in Tables x and y.  In Table Y 'slot' refers to the number of subchannels slots allocated (in both time and frequency).   This is used for OFDMA subchannelization with QPSK, 16-QAM and 64-QAM.

Table x – Encoding subchannel concatenation for different rates in CTC

| Modulation and rate | j |
| --- | --- |
| QPSK 1/2 | j = 40 |
| QPSK 3/4 | j = 26 |
| QAM16 1/2 | j = 20 |
| QAM16 3/4 | j = 13 |
| QAM64 1/2 | j = 13 |
| QAM64 2/3 | j = 10 |
| QAM64 3/4 | j = 8 |
| QAM64 5/6 | j = 8 |

Table y – Subchannel concatenation rule for CTC

| Number of Subchannels | Subchannels concatenated |
| --- | --- |
| n ≤= j AND n mod 7 ≠ 0 | 1 block of n slots |
| n<=j AND n mod 7 = 0 | 1 block of 4n/7 slots 1 block of 3n/7 slots |
| n > j | If( n mod j = 0 ) q blocks of j slots else (q-1) blocks of j slots 1 block of Lb1 slots 1 block of Lb2 slots Where: q = n div j Lb1 = ceil((n-(q-1)*j)/2) Lb2 = floor((n-(q-1)*j)/2) If (Lb1 mod 7= 0) or (Lb2 mod 7= 0) Lb1 = Lb1 + 1; Lb2 = Lb2 – 1; |