IEEE P802.22
Wireless RANs

| PHY Minutes from February 8, 2007 Conference Call |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-02-27 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ramon Khalona | Nextwave Broadband | 12670 High Bluff Drive San Diego, CA 92130 U.S.A. | 858-480-3172 | rkhalona@nextwave.com |

Abstract

This document contains the minutes from the PHY conference call on February 22, 2007.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

The conference call started at 7 PM PT and ended at about 8:20 PM PT on Feb. 8.  A list of participants is attached.

We started by discussing logistics about setting up a bridge for future conference calls.  Gerald volunteered to set one up and the bridge information is given below (see “Next Conference Call”).

Eli Sofer [Runcom] continued presenting his contribution entitled “OFDMA Single Channel Harmonization (v.3),” which he had started two weeks ago.  He illustrated the effect of a typical antenna pattern in a multi-cell deployment where each cell is divided into three sectors and the effect of interference reduction from adjacent sectors due to a combination of antenna pattern and different PN sequences used in each of the three sectors (slide 8).  He also illustrated the disadvantages of 4-sector division (slide 11) due to increased interference. Eli also illustrated, through simulation results, the increase in DL capacity resulting from frequency reuse 1/3, relative to frequency reuse 1/1 (slides 12 and 13).  Gerald requested more information as to why the shape of the CDF curve for the frequency reuse 1/3 case has a more pronounced “knee” and Eli will investigate further.  It was noted (Gerald, Monisha) that these results only include increase in PHY capacity and that MAC efficiency would have to be taken into account to get a better idea of the overall capacity increase.

Eli then reviewed the Ranging process with an example illustration showing user distributions and the effect of channel estimation (slides 15-22).  On Slide 22, an overall preamble processing gain of 36 dB is claimed, out of which 9 dB is assumed to be due to boosting of preamble pilots.  Gerald indicated that in the case of three sectors this boost factor may not be applicable because this would yield increased interference.

Eli then proceeded to show some partial simulation results of O-PUSC (slides 24-31) for the following assumptions

- Bandwidth =6MHz.

- FFTSize=2048.

- FEC Size=480;

- Modulation =QPSK

- CTC coding.

- Coding rate=1/2.

- Guard Interval=256.

The BER and PER curves of slides 30 and 31, respectively, show the performance of O-PUSC for pilot symbols ranging from 3 to 15, with the performance due to 15 symbols approximating the performance of perfect channel estimation to within 0.7 dB at PER = 1% (the performance degradation w.r.t. perfect channel estimation due to 3 symbols is approximately 2.8 dB at this PER).   Eli concluded that in order to get better performance frame aggregation (three to five frames) could be used

These results prompted a discussion of comparing these results with the results simulated by ETRI (which assume 2 pilot symbols) if 4-symbol repetition were used.  ETRI indicated that performance results for that case could be available by the March meeting for a more complete comparison and they agreed to generate these results.

In summary, the performance of 3x3 tiling, which was proposed in London by Eli, does not seem to provide the expected performance and the group will focus on the 7x1 tiling previously simulated by ETRI.

Due to a schedule conflict caused by the time change of this conference call, next week’s conference call will be held on Wednesday, 2/28 (see details below).  Edward Au (Huawei) submitted an updated contribution on CAZAC sequences, which he will present at the next conference call.

Next Conference Call:  The next conference call will be on Wed, Feb. 28 at 6 AM PT (9AM ET).  The bridge information is

Telephone number:	888-265-0903	North-America

613-954-9003	International

Access code:	082942

Attendance for PHY conference call on February 22, 2006:

|  | 2/22/07 |
| --- | --- |
| Myungsun Song | X |
| Gwangzeen Ko |  |
| Sung Hyun Hwang | X |
| Chang Joo Kim |  |
| Jung Sun Um |  |
| Patrick Pirat |  |
| John Benko |  |
| Soo-Young Chang | X |
| Edward Au | X |
| Yuchun Wu |  |
| Wai Ho Mow | X |
| Ying-Chang Liang |  |
| Zander Zhongding Lei | X |
| Weng Seng Leon |  |
| Chang Long Xu |  |
| Anh Tuan Hoang |  |
| Yonghong Zeng |  |
| Steve Kuffner |  |
| Ramon Khalona | X |
| Monisha Ghosh | X |
| Carlos Cordeiro |  |
| Eli Sofer | X |
| Doron Ezri |  |
| David Mazzarese |  |
| Baowei Ji |  |
| Wendong Hu |  |
| Wen Gao |  |
| Carl Stevenson |  |
| Gerald Chouinard | X |
| Upkar Dhaliwal | X |
| Chris Clanton | X |
| George Vlantis | X |