IEEE P802.22
Wireless RANs

| Network Synchronization Proposed Text Changes to Section 8.9 Control Mechanisms and Section 6.21.5 Synchronization of Overlapping BSs in IEEE P802.22/D0.2 Draft Standard for WRAN |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Wendong Hu | STMicroelectronics | 1060 East Brokaw Road, San Jose CA 95131 | 1-408-467-8410 | wendong.hu@st.com |
| George Vlantis | STMicroelectronics | 1060 East Brokaw Road, San Jose CA 95131 | 1-408-451-8109 | George.vlantis@st.com |

Abstract

This document provides proposed text changes to Section 8.9 Control Mechanisms and Section 6.21.5 Synchronization of Overlapping BSs, with description of Network Synchronization mechanism (very similar to the one for IEEE 802.16-2004/802.16h standards) for IEEE 802.22 WRAN. This document provides recommended remedy for comment CID# 244.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Text Change:

[Rename Section 6.21.5 CBP-based Synchronization of WRANs]

[Insert the following text before Section 6.21.5. and renumber the document]

- 6.21.5 Synchronization of the IEEE 802.22 Networks

All base stations will use a common clock to synchronize their MAC frames. The common clock shall be available to all IEEE 802.22 WRANs. Such a clock can be provided by global navigational systems such as GPS or can be distributed by other means such as Ranging. Every BS upon activation will as a first step ensure the derivation of the common system clock.

All synchronized IEEE 802.22 base stations shall either synthesize or derive a 1 pps clock broadcast by a global navigational system or other means such as Ranging.

Every base station equipped with a global navigational system receiver would be capable of receiving a UTC synchronized 1 pps timing signal. The accuracy of the clock pulses derived from using the global navigational system are accurate to +/- 100 usec and the pulses that are derived typically have rise times within +/- 2.5 nsec.

[Add the following text to Section 8.9 Control Mechanisms]

- 8.9.4 Network Synchronization

For TDD and FDD realizations, it is required that all BSs be time synchronized to a common timing signal. In the event of the loss of the network timing signal, BSs shall continue to operate and shall automatically resynchronize to the network timing signal when it is recovered. The synchronizing reference shall be a 1 pps timing pulse and a 10 MHz frequency reference. These signals are typically provided by a global navigational system receiver.

For both FDD and TDD realizations, frequency references derived from the timing reference may be used to control the frequency accuracy of Base-Stations provided that they meet the frequency accuracy requirements of 8.11. This applies during normal operation and during loss of timing reference.

[Add the following text to Section 8]

- 8.11 Frequency Control Requirements

8.11.1 Center frequency and symbol clock frequency tolerance

At the BS, the transmitted center frequency, receive center frequency, and the symbol clock frequency shall be derived from the same reference oscillator. At the BS, the reference frequency accuracy shall be better than ±2*10–6.

At the CPE, both the transmitted center frequency and the symbol clock frequency shall be synchronized to the BS with a tolerance of maximum 2% of the subcarrier spacing.

During the synchronization period, the CPE shall acquire frequency synchronization within the specified tolerance before attempting any uplink transmission. During normal operation, the CPE shall track the frequency changes and shall defer any transmission if synchronization is lost.

References: