IEEE P802.22
Wireless RANs

| Proposed Normative Text for Resolutions of Comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Dave Cavalcanti | Philips | 345 Scarborough Rd, Briarcliff Manor, NY | 914-945-6083 | Dave.Cavalcanti@philips.com |
| Carlos Cordeiro | Philips | 345 Scarborough Rd, Briarcliff Manor, NY | 914-945-6091 | Carlos.Cordeiro@philips.com |

Abstract

This submission describes the proposed normative text to resolve the following comments: 18, 21, 126, 188, 190, 87, 237, 526, 177, 178, 11, 511, 100, 175, 180, 191, 206, 207, 208, 209, 210, 211, 212, 213, 214, 215, 216, 217, 218, 219, 220, 221, 246, 504.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

This document contains proposed normative text to address the following comments: 18, 21, 126, 188, 190, 87, 237, 526, 177, 178, 11, 511, 100, 175, 180, 191, 206, 207, 208, 209, 210, 211, 212, 213, 214, 215, 216, 217, 218, 219, 220, 221, 246, 504.

# Comment #18

    - Frame Control Header

In clause 6.5.2 Frame Control Header, replace Table 4 by the following:

- Tabl 4 —Frame control header format

| Syntax | Size | Notes |
| --- | --- | --- |
| Frame_Control_Header_Format() { |  | Transmitted with well-known modulation/coding (see 8.3.2.2) |
| DS-MAP Length | 8 bits | Size in bytes. If there are any unused IEs, the first unused IE shall have all fields encoded as zeros. |
| US-MAP Length | 8 bits | Size in bytes. If there are any unused IEs, the first unused IE shall have all fields encoded as zeros. |
| DCD Length | 8 bits | Size in bytes. If there are any unused IEs, the first unused IE shall have all fields encoded as zeros. |
| UCD Length | 8 bits | Size in bytes. If there are any unused IEs, the first unused IE shall have all fields encoded as zeros. |
| HCS | 8 bits | Header Check Sequence See Error: Reference source not found |
| } |  |  |

# Comment #21

      - Channel IE

In Table 26, replace the TTG (Element ID 4) by the following:

| TTG | 4 | 1 | TTG in 8 samples increment. |
| --- | --- | --- | --- |

In Table 26, replace RTG by the following:

| RTG | 5 | 1 | RTG in 8 samples increment. |
| --- | --- | --- | --- |

In Table 26, delete Frame Duration Code (Element ID 8)

In Table 26, delete element ID 16 Channel Number for Backup and element ID 17 Number of Channels for Backup, and insert the following IEs:

| Number of Channels for Backup | 16 | 1 | Number of backup channels available. |
| --- | --- | --- | --- |
| For (i=0; i < Number Channels for Backup; i++) { |  |  | List of backup channels in order of priority to be used by CPEs in case of loss of communication with the BS due to incumbents. The backup channels shall be a disjoint set with the current operating channels. |
| Channel Number for Backup [i] | 17 | 1 |  |
| } |  |  |  |

In Table 26, replace Element ID 18 Sensing RTG by the following:

| Sensing Durantio | 18 | 1 | If this field is set to a value different from 0 (zero): It indicates the number of slots in the end of the frame that shall be used by to perform sensing. |
| --- | --- | --- | --- |

In Table 26, replace element ID 19 Channel Number for Sensing RTG by the following:

| Channel Number for Sensing | 19 | 1 | The channel number that is to be sensed during the Sensing Duration. |
| --- | --- | --- | --- |

Delete Clause 6.8.1.1.1. Frame Duration Code

# Comment #87

  - General Frame Structure

In clause 6.4 General Frame Structure, replace Figure 4 by the following figure:

- Figure 4—Frame structure

# Comment #126

In Table 26 insert the following new field after element ID 10 (Channel Action) and renumber the remaining information element as follows:

| Action Mode | 11 | 1 | This is valid only for channel switch (Action = 1) or channel remove (Action = 3) Indicates any restrictions on transmission until the specified Channel Action is performed. The BS shall set the Action Mode field to either 0 or 1 on transmission. A value of 1 means that the CPE to which the frame containing the element is addressed shall transmit no further frames until the scheduled Channel Action is performed. A Action Mode set to 0 does not impose any requirement on the receiving CPE. |
| --- | --- | --- | --- |

In Table 26 insert the following new fields after element ID 13 (Action Duration) and renumber the remaining information element as follows:

| Number of Quiet Period Purposes | 14 | 1 | This is valid only for quiet periods (Action = 4). Indicates the number of Quiet Period Purpose IE |
| --- | --- | --- | --- |
| for (i=0; i<Number of Quiet Period Purposes; i++) { |  |  |  |
| Quiet Period Purpose IEs | 15 | 3 | This is valid only for quiet periods (Action = 4). See Table 141 for specification. |
| } |  |  |  |

In Table 26 delete the Reserved field with element ID 3

    - Channel Management

At the end of paragraph 1, line 26, page 164, Section 6.21.4 Channel Management, add the following sentence:

The BS and CPEs shall support both these schemes, while the decision on which one to use and when is at the discretion of the BS.

At the end of paragraph 2, line 6, page 165, Section 6.21.4 Channel Management, add the following sentence:

Similar to all other IEs, the IEs related to the embedded channel management (see Section 6.8.1.1) scheme shall only be transmitted by the BS when this feature is used.

# Comment #188

  - Network Entry and Initialization

In clause 6.15 Network Entry and Initialization, replace Figure 20 by the following figure and update the figure’s caption as follows:

- Figure 20 – Scenario where a new TV station starts operation in an overlapping area with an existing 802.22 WRAN.

# Comment #190

- 6.15 Network Entry and Initialization

In clause 6.15 Network Entry and Initialization, insert the following new step 1 in line 22 and page 116, and renumber the remaining steps:

- Perform incumbent detection in TV channels and create spectrum map.

In clause 6.15 Network Entry and Initialization, insert the following new step 8 after step 7 (Perform registration) in line 2 and page 117, and renumber the remaining steps:

- Perform neighboring network discovery.

In clause 6.15 Network Entry and Initialization, replace Figure 21 by the following:

- Figure 21–CPE network entry and initialization procedure

In clause 6.15 Network Entry and Initialization, replace Figure 22 by the following:

- Figure 22–Obtaining downstream parameters

    - BS Initialization

Replace the text in clause 6.15.1 BS Initialization by the following:

The WRAN BS initialization procedure shall consist of the following steps:

- Access any available TV channel usage database.

- Perform incumbent detection in all usable TV channels and create spectrum map

- Perform neighboring network discovery (see Section 6.21.2.1.3) on selected channel(s).

- Commence operation on the selected operating channel(s).

Include new clause 6.15.6 Neighboring Network Discovery as follows:

    - Neighboring Network Discovery

After a CPE has registered with a WRAN BS, it shall perform network discovery in order to identify neighboring WRANs and eneable efficient self-coexistence. The neighboring network discovery involves listening to the medium for CBP packets or BS SCH transmitted by other WRAN BSs. This network discovery mechanism is described in Section 6.21.2.1.3.

# Comment #237

        - Fast Sensing Allocation

In clause 6.21.3.2.1 Fast Sensing Allocation, replace Figure 59 by the following figure:

- Figure 59—Frame structure with fast sensing period

# Comments #11/177/178/526

        - DIUC Allocations

Replace the content of clause 6.8.2.11 DIUC Alloations by the following:

illustrates the various DIUC values used in CMAC. In particular, it is important to highlight the use of the self-coexistence DIUC interval in Passive Mode, which can be seen as an in-frame quiet period, where the intention is to sense for the presence of other co-located 802.22 cells in the area. More specifically, this interval is used by 802.22 stations to look for both CBP and BS beacons. This allows flexibility for the 802.22 system to efficiently coexist with other nearby 802.22 systems.

The DIUC = 12 may be used for allocation of channels for PAPR reduction schemes.

In Table 33, delete DIUC 0 Self-Coexistence (Active Mode) and updated table as follows

- Table 33—DIUC values

| DIUC | Usage |
| --- | --- |
| 0 | Self-Coexistence in Passive Mode |
| 1-11 | Burst Profiles |
| 12 | PAPR Reduction |
| 13 | End of Map |
| 14 | Extended DIUC |

# Comment #511

    - Obtaining Downstream Parameters

In clause 6.15.3 Obtaining Downstream Parameters, delete the words the following words from the second sentence in the first paragraph (page 119, line 2):

“in case a long supergrame is in use by theBS”

# Comment #519

      - Incumbent Detection Recovery

In clause 6.21.1.5 Incumbent Detection Recovery, replace Figure 42 by the following figure:

- 42—IDRP at CPE

# Comments: 100, 175, 180, 191, 206, 207, 208, 209, 210, 211, 212, 213, 214, 215, 216, 217, 218, 219, 220, 221, 246, 504

The resolution to all these comments is in document: 22-07-xxxx-00-integrated-coexistence-mechanisms.doc (March 2007)

References: