IEEE P802.22
Wireless RANs

| The normative text for 8.10.2 Ranging |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-04-27 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Chang-Joo Kim | ETRI | Korea | +82-42-860-1230 | cjkim@etri.re.kr |
| Myung-Sun Song | ETRI | Korea | +82-42-860-5046 | mssong@etri.re.kr |
| Gwang-Zeen Ko | ETRI | Korea | +82-42-860-4862 | gogogo@etri.re.kr |
| Sung-Hyun Hwang | ETRI | Korea | +82-42-860-1133 | shwang@etri.re.kr |
| Jung-Sun Um | ETRI | Korea | +82-42-860-4844 | korses@etri.re.kr |

Abstract

This document presents normative text intended for insertion into the 802.22 standard.  This text is intended to describe the ranging which will be inserted into Section 8.10.2 of v0.2.

This version (r2) contains the comments of Gerald Chouinard and “JS”  (whose identity needs to be confirmed).

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

    - Ranging

Capacity shall be reserved in the upstream sub-frame for ranging, BW request and UCS notification.  It shall be composed of one or more groups of five adjacent subchannels transmitted by using the distributed subcarrier permutation The groups shall be defined starting from the first logical subchannel and it will be called the ranging subchannel. Subchannels are considered adjacent if they have successive logical subchannel numbers. The indices of the subchannels that compose the ranging subchannel are specified in the US-MAP_IE. CPEs will be allowed to collide on this ranging channel. To effect a ranging transmission, each CPE shall randomly choose one ranging CDMA code from the subgroup of specified binary codes.

      - Initial-ranging transmission

The initial ranging transmission shall be used by any CPE that wants to synchronize to the system for the first time. An initial-ranging transmission can be performed during two, three or four consecutive symbols following the  normal TTG for cell coverage of up to 30km or following the normal TTG plus one symbol period for cell coverage of  up to 75 km, and following the normal TTG plus two symbol periods for cell coverage up to a maximum of 100km. The same ranging code shall be repeated on each symbol on the ranging subchannel and this code shall be BPSK modulated with phase rotation according to the symbol index and the subcarrier index. These symbols shall be generated according to equation (1). This initial-ranging transmission shall maintain phase continuity between two contiguous symbols after the CP is inserted in front of each OFDM symbol. A time-domain illustration of the two or three initial-ranging transmission symbols is shown in Figure 1.

,              (1)

[Rather than 0 for k not contained in R, it should be:  “used for other purposes” such as data payload transmission.]

Other CPEs can use the subcarriers k, which is not contained in R, for data transmission. However, s(n,l) of equation represents the transmission symbol of the only CPE which try to perform the ranging, BW request or UCS notification. So, this transmission symbol includes the CDMA codes only on the ranging subchannel and null on the other subcarriers except for R. If you want, we can change from 0 to null.

Where

s(n,l)  is the l-th OFDMA symbol for initial ranging with the sample index n. l is [0,1,2,3].

k         is the subcarrier index in the TV channel.

Ck       is the ranging code defined in 1.1.1.3.

R        is the set of index of subcarriers within the ranging subchannel.

NFFT   is the size of 2K FFT, 2048.

NCP      is the size of the cyclic prefix (= guard interval).

- Using two consecutive OFDMA symbols

- Using three consecutive OFDMA symbols

- Initial-ranging transmission for OFDMA [Don’t understand why the CP’s of successive symbols need to concatenate in each symbol.  Is this the only way to maintain phase continuity?]

An OFDMA symbol basically consists of the CP and symbol(of which size is FFT size).  This is the unit size of the OFDM symbol. CP is required for the consistency of duration of each OFDMA symbols.

So, the way to maintain the phase continuity is performed by shifting the FFT size symbol in the time domain according to its symbol index, with using the equation (1), before inserting the CP.

      - Periodic-ranging, BW-request and UCS notification transmission

Periodic-ranging transmissions shall be sent for system periodic ranging. Bandwidth-requests transmissions shall be for requesting upstream allocations from the BS. UCS notification transmissions shall be used for reporting the existence of incumbents in the current operating TV channel. These transmissions shall be sent only by CPEs that have already synchronized to the system. To perform periodic-ranging, bandwidth-request or UCS notification transmission, the CPE can send a transmission in one of the following ways:

a) Modulate one ranging code on the ranging subchannel for a period of one OFDMA symbol. Ranging subchannels shall be dynamically allocated by the MAC layer at the BS and indicated in the US-MAP. A time domain illustration of the periodic-ranging, bandwidth-request or UCS notification transmission is shown in Figure 2.

- Periodic-ranging or bandwidth-request transmission for OFDMA using one code

b) Modulating three consecutive ranging codes (starting code shall always be a multiple of 3) on the ranging subchannel for a period of three OFDMA symbols (one code per symbol). Ranging subchannels shall be dynamically allocated by the MAC layer at the BS and indicated in the US-MAP. A time domain illustration of the periodic-ranging, bandwidth-request or UCS notification transmission is shown in Figure 3.

- Periodic-ranging or bandwidth-request transmission for OFDMA using three codes

When do we use 1 or 3?  Why one symbol and then 3?  Why not 2 symbols and any other concatenation depending on the payload to transmit?

It was based on the 802.16 stand. The reason of using different CDMA codes on the 3 different symbols is to improve the detection probability. In my opinion, 3 symbols may be related to the 3x3 tile structure in 802.16. We can change two symbols.

      - Ranging codes

The binary codes shall be the pseudonoise codes produced by the PRBS described in Figure 4, which illustrates the polynomial generator . The PRBS generator shall be initialized by the seed b15...b1 = 0,0,1,0,1,0,1,1,s0,s1,s2,s3,s4,s5,s6 where s6 is the LSB of the PRBS seed, and s6:s0 = US_Idcell [Definition? I just saw this word in the table 39. If it is not defined, we can use the seed which consists of 15 bits.], where s6 is the MSB of US_IDcell.

- PRBS for ranging code generation

The binary ranging codes shall be subsequences of the pseudonoise sequence appearing at its output Ck. The length of each ranging code is 120 bits. [5 subchannels*24 data carriers=120 data carriers and 20 pilot carriers.  Are the pilot carriers to be spread with the CDMA code as well?  Then, the normal pilot pattern may not be retained. When the ranging, BW request and UCS notification transmission use the CDMA code, these transmissions don’t require the channel estimation because BS only detects the transmitted CDMA codes by using the correlation method based on the maximum likelihood. So, the subcarriers for pilots also can be used for transmission of CDMA code and total possible CDMA codes are 140 bits.

At this time, after the ranging subchannels are allocated on the several OFDMA symbols in the upstream, the remaining symbols of the same subchannel on which other CPE can transmit the data payload should be 7 more symbols for channel estimation. One comment is that the channel information is different between CPE using the ranging and CPE transmitting the data even though those use the same subchannel.]  These bits shall be used to modulate the subcarriers in a group of five adjacent subchannels,. The bits shall be mapped to the subcarriers in increasing frequency order of the subcarriers, such that the lowest indexed bit modulates the subcarrier with the lowest frequency index and the highest indexed bit modulates the subcarrier with the highest frequency index. The index of the lowest numbered subchannel in the five shall be an integer multiple of five. The ranging subchannel shall be referenced in the ranging, Bandwidth Request and UCS notification messages by the index of lowest numbered subchannel.  [It is supposed to always start with the subchannel index 0 since it is the first logical subchannel as indicated above.  Is this necessary? It doesn’t mean that the starting index to allocate the ranging subchannel should be from the first logical subchannel, but it means that BS indicates the ranging subchannel with lowest subchannel of the five subchannel in the CDMA_Allocation_IE ( you can find in table 48 of the v0.2). ]

For example, the first 120 bit code obtained by clocking the PN generator as specified, with US_IDcell = 0, the first code shall be 00110000010001... The next ranging code shall be produced by taking the output of the 121th to 240th clock of the PRBS generator, etc.

The number of available codes is 256, numbered 0 to 255. Each BS uses a subgroup of these codes, where the subgroup is defined by a number S, 0 <= S <= 255. The group of codes will be between S and ((S+N+M+L+I) mod 256).

—  The first N codes shall be for initial-ranging and shall be generated by clocking the PRBS generator 120 × (S mod 256) times to 120 × ((S+ N) mod 256) – 1 times.

—  The next M codes shall be for periodic-ranging and shall be generated by clocking the PRBS generator 120 × ((N + S) mod 256) times to 120 × ((N + M + S) mod 256) – 1 times.

- The next L codes shall be for bandwidth-requests and shall be generated by clocking the PRBS generator 120 × ((N + M + S) mod 256) times to 140 × ((N + M + L + S) mod 256) -1 times.

- The next I codes shall be for UCS notification and shall be generated by clocking the PRBS generator 120 × ((N + M + L + S) mod 256) times to 140 × ((N + M + L + I + S) mod 256) -1 times.

The BS shall separate colliding codes and extract timing (ranging) and power information following the method described below …. In the process of CPE code detection, the BS shall get the Channel Impulse Response (CIR) of the code under the following specific format …, thus acquiring for the BS the following information about the channel and condition of each CPE. [Specify information and format of information to enable compliance verification.]  The time (ranging) and power measurements shall be used by the system to compensate for the various BS-CPE-BS propagation distances. [Specify tolerances.]

I think it is related to the implementation issue. Basically, the BS can easily separate colliding codes by correlation with possible CDMA codes such as CDMA system and use the correlation value for transmission power. CIR will be just the estimation results in the process of CPE code detection to be used for compensation algorithm of timing offset. So, it would not be specific format or information.

The timing offset tolerance may be +/- 50% of the minimum CP duration.

Reference-8.3.12 Frequency and timing requirements in 802.16-2004  : All SSs shall acquire and adjust their timing such that all uplink OFDM symbols arrive time coincident at the Base-Station to a accuracy of ±50% of the minimum guard-interval or better.

    - Ranging, BW request and UCS notification opportunity window

For ranging, BW-request and UCS notification, the ranging opportunity window in the time domain is the number of symbols required to transmit the appropriate ranging/BW-request/UCS notification code, and is denoted by N1. N2 denotes the number of subchannels required to transmit a ranging, BW-request and UCS notification code. In each ranging/BW request/UCS notification allocation, the opportunity window (N1xN2) is set in the time domain by the size of the US subframe as conveyed by the US_MAP_IE. The ranging allocation is subdivided into slots of N1 OFDMA symbols by N2 subchannels, in a row/column order [where is this order defined? At the BS?  This is supposed to be opportunistic from a number of independent CPEs. It is defined by BS in the table 44 and table 38 of the draft v0.2. And I guess the UCS notification opportunity size should be also defined in the table 38 from the MAC team.], i.e. the first opportunity begins on the first symbol of the first subchannel of the ranging allocation, the next opportunities appear in ascending order in the same subchannel, until the end of the ranging/BW-request/UCS notification allocation (or until the end of the required allocation if there are less than N1 symbols in the current subchannel), and then the number of subchannels is incremented by 1 up to N2. The ranging allocation is not required to be a whole multiple of N1 symbols, so a gap may be formed. Each CDMA code shall be transmitted at the beginning of the corresponding slot except in the cases of the first time slot of each group of sub-channels where the distance between the CPE and the base station is larger than what can be absorbed by the TTG.  For distances larger than 30 km and less than 75 km, the CDMA code will need to be sent one OFDMA symbol later and for distances larger than 75 km, two OFDMA symbol delay will be needed. See Figure 5.

- Ranging/BW request/UCS notification opportunities

[I am not clear on how the CPE MAC address is carried in these coded messages and how the payload is carried (e.g., the parameters needed to indicate the bandwidth request).]

When the CDMA codes are used, CPEs doesn’t transmit any its MAC address or data payload but just transmit the CDMA code which is randomly selected. Then, BS can response to CPEs with CDMA_Allocation_IE and specific CID. You can see more detail process in the section 6.12.1.2 Contention-based CDMA Request.

In my opinion, CPE using the CDMA codes is in the condition which BS doesn’t assign the burst to that CPE for any reason, such as there are currently no data transmissions between BS and CPE or CPE is in the process of network entry and initialization.

[CDMA works well if the signal power differential is small.  How can this power differential as received at the BS can be kept within proper tolerance especially in the case of the initial association?]

The CPE shall start from the minimum transmit power level defined by the BS in the initial ranging. If the CPE does not receive a response from BS, the CPE shall send a new CDMA code at the next appropriate Initial Ranging transmission opportunity at one step higher power level.

[How are the pilot carriers recovered in collision mode?  Are the pilot carriers also spread with the same CDMA codes?  Then the normal pilot pattern would not be retained.] As I stated above, the subcarriers for pilots are also used for transmission of CDMA codes.