IEEE P802.22
Wireless RANs

| [Comment Resoluton for CID 53 on Precise Cell Synchronization] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Baowei Ji | Samsung Telecom. America, LP | 1301 E. Lookout Dr. Richardson, TX 75082 | +1-972-761-7167 | Baowei.ji@samsung.com |

Abstract

Text for resolving Comment on Section 6.21.5.

The propagation delays between the transmitter and receiver of SCH and CBP packets shall be considered in order to establish precise synchronization of overlapping cells. The frame offset between neighbor cells cannot be calculated precisely without knowing the propagation delay.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Insert a new section at the end of Section 6 .21.5 and right above Section 6.21.6

Section 6.21.5.4: Precise cell synchronization

The propagation delays between the transmitter and receiver of SCH and CBP packets in different cells shall be considered in order to establish precise synchronization of overlapping cells. As shown in Figure 1, the frame offset (Δ) between neighbor cells cannot be calculated precisely without knowing the propagation delay (τ) between the transmitter (could be a BS or a CPE) and the receiver (could be a BS or a CPE).

When SCH is used for synchronizing overlapping cells, the location information of the transmitting BS shall be included in the SCH, so that a SCH receiver can calculate and cancel out the propagation delay between the transmitter and the receiver. This is done using the Location Configuration IE.

Similarly when CBP is used for synchronizing overlapping cells, the location information of a CBP transmitter shall also be included in the CBP packet, so that a receiver of the CBP packet can calculate and cancel out the propagation delay between the transmitter and the receiver. For CPEs, this implies that its location information must be known in advance so that the propagation delay can be canceled out.

Node_a

Node_b

time

Tx offset

Rx offset





Without , cannot calculate 

Figure 1. Illustation of the need for cancelling out the propagation delay for precise cell synchronization

Insert one row in Table 9 on Page 3 of doc [3]

- —CBP IEs

| Element ID | Name |
| --- | --- |
| 0 | Beacon IE |
| 1 | DS/US Boundary IE |
| 2 | BS IP Address IE |
| 3 | Inter-BS Capabilities IE |
| 4 | CC_REQ IE |
| 5 | CC_REP IE |
| 6 | CC_ACK IE |
| 7 | RR-REQ IE |
| 8 | RA-RSP IE |
| 9 | RA-ACK IE |
| 10 | RC-REQ IE |
| 11 | RC-RSP IE |
| 12 | RC-ACK IE |
| 13 | RE-REQ IE |
| 14 | RE-RSP IE |
| 15 | RE-ACK IE |
| 16 | RS-SEM IE |
| 17 | RS-ADV IE |
| 18 | BS Channel Parameter IE |
| 19 | CBP Location IE |

- Insert a new section on Page 10 of doc [3]

6.6.1.2.1.19 CBP Location IE

- —CBP Location IE

| Syntax | Size | Notes |
| --- | --- | --- |
| Location_Configuration_Measurement_Report_Format() { |  | An LCI with Latitude resolution, Longitude resolution and Altitude resolution set to zero shall indicate that the location is not known. |
| Element ID | 8 bits |  |
| Length | 8 bits | Shall be 16 octets |
| Report Mode | 4 bits | Error: Reference source not found |
| Latitude Resolution | 6 bits | Latitude resolution indicates the number of valid bits in the fixed-point value of Latitude |
| Latitude | 34 bits | Latitude is a fixed point value consisting of 9 bits of integer and 25 bits of fraction |
| Longitude Resolution | 6 bits | Longitude resolution indicates the number of valid bits in the fixed-point value of Longitude |
| Longitude | 34 bits | Longitude is a fixed point value consisting of 9 bits of integer and 25 bits of fraction |
| Altitude Type | 4 bits | Altitude Type encodes the type of altitude. Codes defined are: 1: Meters – in 2s-complement fixed-point 22-bit integer part with 8-bit fraction; 2: Floors – in 2s-complement fixed-point 22-bit integer part with 8-bit fraction; Altitude type = 2 for Floors enables representing altitude in a form more relevant in buildings which have different floor-to-floor dimensions. |
| Altitude Resolution | 6 bits | Altitude resolution indicates the number of valid bits in the altitude |
| Altitude | 30 bits | Altitude is a value defined by the Altitude type field |
| Datum | 8 bits | Datum is encodes the horizontal and vertical references used for the coordinates. The Datum octet has 256 possibilities, of which 3 have been registered with the Internet Assigned Numbers Authority (IANA): 1: WGS 84 (Geographical 3D) – World Geodesic System 1984, Coordinate Reference System (CRS) Code 4327, Prime Meridian Name: Greenwich; 2: NAD83 – North American Datum 1983, CRS Code 4269, Prime Meridian Name: Greenwich; The associated vertical datum is the North American Vertical Datum of 1988 (NAVD88); 3: NAD83 – North American Datum 1983, CRS Code 4269, Prime Meridian Name: Greenwich; The associated vertical datum is Mean Lower Low Water (MLLW). The WGS 84 datum shall be used when referencing locations anywhere. The GeoConf Option referred to in IETF RFC 3825 defines two fields for which the IANA maintains a registry: The Altitude type (AT) field and the Datum field. The initial values of the Altitude registry are as follows: AT = 1 meters of Altitude defined by the vertical datum specified; AT = 2 building Floors of Altitude. |
| } |  |  |

References:

[1] 22-07-0021-02-0000_Revisit_CBP_and_ Synchronization_of_Overlapping_ WRAN.ppt

[2] 22-06-0259-00-0000_v0.2_with_line_numbers.doc

[3] 22-07-0121-01-0000-integrated-coexistence-v7.doc