IEEE P802.22
Wireless RANs

| ETRI-Channel Classification Section 6.21.4.1 in IEEE P802.22/D0.2 Draft Standard for WRAN |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-02 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Chang-Joo Kim | ETRI | Korea | +82-42-860-1230 | cjkim@etri.re.kr |
| Myung-Sun Song | ETRI | Korea | +82-42-860-5046 | mssong@etri.re.kr |
| Gwang-Zeen Ko | ETRI | Korea | +82-42-860-4862 | gogogo@etri.re.kr |
| Sung-Hyun Hwang | ETRI | Korea | +82-42-860-1133 | shwang@etri.re.kr |
| Jung-Sun Um | ETRI | Korea | +82-42-860-4844 | korses@etri.re.kr |
| Bub-Joo Kang | ETRI | Korea | +82-42-860-5446 | kbj64370@etri.re.kr |

Abstract

This document provides proposed text changes to Section 6.21.4.1. Proposed channel classification scheme is devised in order to efficiently manage the WRAN’s operating channels depending on channel occupancy status. This document provides recommended remedy for comment CID #127, # 128 and #240.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- Response for Comments & Resolution

Related with this part, there are three comments ( #127, #128 and #240). The comments mainly concern with definition of active channel sets. To reflect the comments, Active channel set is redefined as shown in the following text.

Replace the content of clause 6.21.4.1 by the following text:

6.21.4.1 Channel Classification

      - Channel Classification

For efficient channel management, four different channel sets are defined: active set, candidate/backup set, occupied set, and null set. These sets are respectively maintained in BS and CPE. If needed, each set is updated in every quiet period. Note that a set of channels that do not belong to either active set, candidate/backup set or occupied set is defined as a null set and it is maintained only the BS. Each channel set is defined as follows:

- Active set: a set of  channels being used by WRAN systems

- Active set 1: a set of channels being used by BS and CPEs

- Active set 2: a set of channels being used by other CPEs or BSs

- Candidate/Backup set: a set of clean channels available for a certain BS or CPE

- Occupied set: a set of occupied channels by incumbent user which a certain CPE finds

- Disallowed Set: a set of channels whose access are not allowed by regulation

- Null set: a set of channels that are not classified as one of above five sets

Note that the allowed set is defined by the union of the candidate/backup set and the null set depending on a channel’s SIR level. Also, the disallowed set is not considered in channel management scenarios because it does not change frequently.

In order to maintain the channel sets, each BS maintains four channel sets (i.e., Active, Occupied, Candidate/Backup, and Null). Also, each CPE maintains three channel sets (i.e., Active, Candidate/Backup, and Occupied). These individual sets have different update duration. For example, in CPE side, Active sets are updated after every receiving of SCH, Candidate sets are updated after receiving DCD and Occupied and Null sets are updated after receiving CHO-UPD message(See 6.8.21.9). After synchronization, BS should send CHO-UPD message to CPE to update the Occupied set for newly connected CPE. These relations are summarized in Table 1.  Whereas, in BS case, channel sets are updated after each quiet period either at a periodic interval or aperiodic interval.

Table 1 Update channel set information in CPE

| Message | Field | Information | Note |
| --- | --- | --- | --- |
| SCH | TxID,CN,NC | Active set Bonded Active set |  |
| DCD | Channel Number for Backup Number of Channles for Backup | Candidate set |  |
| CHO-UPD (Channel Occupancy Update) | Channel number and channel state | Occupied set |  |
|  |  | Other channels are decided in null set |  |

      - Transition Diagrams for Channel Sets

Any channel belongs to one of possible channel sets in the BS. At the end of quiet period, it may transit to other set as shown by the state transition diagram in Figure 11. Depending on the activities of incumbent users and channel quality, 8 possible transitions can be observed. The condition for each transition is described as follows:

  - The channel in the sets other than occupied set becomes useless as incumbent service appears.

  - Incumbent service releases the channel and its quality is better than an existing member of the candidate set, then it is classified as a member of candidate set.

  - Incumbent service releases the channel and its quality is worse than all member of the candidate set, then it is classified as a member of null set.

  - If the channel quality is better than an existing member of the candidate set, then it replaces the member of candidate set.

  - The channel becomes active by new connection of a WRAN service.

  - The channel is switched as a member of null set as its quality is worse than newly selected candidate set from null (4), active (7) and occupied (2) sets.

  - The channel is released due to the finish of its usage and its quality is better than an existing member of the candidate set.

  - The channel is classified as a member of null set as the WRAN service releases the channel and its quality is worse than all member of the candidate set.

- —Channel set transition diagram

Add the following text to consecutive sub-section:

      - Initialization and Updating Channel Sets

When a CPE turn on its power, the CPE executes channel sets initialization procedure as shown in Figure 2. This procedure is closely related with obtaining downlink parameters procedure (See. 6.15.4). After a new CPE connected to BS, BS shall send the CHO-UPD message to indicate occupied channel information to the CPE. Therefore the CPE initializes the Occupied Set by receiving CHO-UPD message. Channel sets in the CPE are updated after receiving corresponding messages such as SCH, DCD, and CHO-UPD. In the case of BS, after each sensing period, BS received BLM-REP message. If channel sets are changed depending on BLM-REP messages. BS sends CHO-UPD to corresponding CPEs and updates new channel information in its channel sets. This procedure is depicted in Figure 3.

| - —CPE Channel Sets initialization | - —BS Channel Sets update |
| --- | --- |