IEEE P802.22.1
Wireless RANs

| FEC Text for IEEE Std. 802.22.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-04-03 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Wu Yu-chun | Huawei Hisi | Xinxi Rd., Shangdi St, Haidian Dist, Beijing 100085 China | 86-10-8282-9018 | wuyuchun@huawei.com |
| Soo-Young Chang | Huawei Technologies | Davis, CA, U.S. | 1-916 278 6568 | sychang@ecs.csus.edu |
| Jianwei Zhang | Huawei Technologies | Shanghai, China | 86-21-68644808 | zhangjianwei@huawei.com |

Abstract

FEC on Sync Burst and FEC on PSDU were approved in Orlando meeting by the TG1 group to be included in the TG1 draft. As requested, this file provides the text on FEC on Sync Burst and FEC on PSDU to be incorporated into the Draft.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

1.  Encoding of FEC on Sync Burst

The Sync Frame is transmitted in I channel and its structure is as following:

Sync Frame

With Index

X

Sync Frame

With Index

X-1

Sync Frame

With Index

1

。。。

PN Sequence

(15 bits)

Index

(7 bits)

Parity

(8 bits)

Reserve(2 bits)

Figure 1.  Sync frame structure.

1) The 15bits of PN Sequence b(0) ~ b(14) are defined in Table 15 (Please refer to [1]);

2) The 7bits of Index b(15) ~ b(21) indicates how many slots left before the next Beacon Super frame starts. To improve the receive performance of the Index, a (15, 7) Linear Block Code is used. The coding of Index is as in the following step;

3) The 8 parity bits b(22) ~ b(29) are used for both error-correction and error-detection for the index. These 8 parity bits are generated by the generator polynomial: g(D) = D8 + D7 + D6 + D4 + 1 from the 7 index bits b(15) ~ b(21) in a systematic form, which means that, in GF(2), the polynomial:

-	b(15)D14 + b(16)D13 +...+b(21)D8 + b(22)D7 + b(23)D6 +...+ b(28)D1 + b(29);

-	b(22) - b(29): the parity bits;

-	b(15) - b(21): the index bits;

When divided by g(D), this step yields a remainder equal to 0.

4) The two reserved bits b(30) ~ b(31) should be set to 0.

2.  Decoding of FEC on Sync Burst

Suppose the received Index and Parity bits are r(0) ~ r(14). Then the decoding is done as in the following steps:

1) Calculate the syndrome sequence:

s=(s0,s1⋯s14)=rHT=(r0,r1⋯r14)h0,0 h0,1 ⋯ h0,14h1,0 h1,1 ⋯ h1,14 ⋮

where the parity check matrix H is obtained by following way:

The first row vector of His h0=1 0 1 0 0 0 1 0 0 0 0 0 0 0 1, the (i+1)th row vector hiis obtained by cyclic right shifting the ith row vector hi−1 by one element. Then the parity check matrix Hwill be:

H=1 0 1 0 0 0 1 0 0 0 0 0 0 0 11 1 0 1 0 0 0 1 0 0 0 0 0 0 00 1 1 0 1 0 0 0 1 0 0 0 0 0 00 0 1 1 0 1 0 0 0 1 0 0 0 0 00 0 0 1 1 0 1 0 0 0 1 0 0 0 00 0 0 0 1 1 0 1 0 0 0 1 0 0 00 0 0 0 0 1 1 0 1 0 0 0 1 0 00 0 0 0 0 0 1 1 0 1 0 0 0 1 00 0 0 0 0 0 0 1 1 0 1 0 0 0 11 0 0 0 0 0 0 0 1 1 0 1 0 0 00 1 0 0 0 0 0 0 0 1 1 0 1 0 00 0 1 0 0 0 0 0 0 0 1 1 0 1 00 0 0 1 0 0 0 0 0 0 0 1 1 0 11 0 0 0 1 0 0 0 0 0 0 0 1 1 0

2) Calculate the error vector, e=e0,e1⋯e62. For the ith element ei, we first establish the row index set A(i)=j1,j2,⋯j4, where each element of A(i)is a row index of H and the ith element of this row is “1”. For example, fromH, we can easily find that A(0)=0, 1, 9, 13, A(14)=0, 8, 12, 14). If more than half of the sA(i)= is “1”, then ei=1, otherwise ei=0.

3) Calculate z=(r+e)%2.

4) To detect errors, if zHT=0, we extractz0,z1⋯z6as the Index. Otherwise, the decoding failed and we should continue to process the next Sync Frame to obtain Index information.

3.  Encoding of FEC on Beacon Header

Define Set S={ 1	5	9	13	17	21	25	29	33	37	41	45	49	53	57	61	65	69	73 78	82	86	90	94	98	102	106	110	114	118	122	126	130	134	138	142	146	150 155	159	163	167	171	175	179	183	187	191	195	199	203	207	211	215	219	223	227 232	236	240	244	248	252	256	260	264	268	272	276	280	284	288	292	296	300	304}

Suppose the input data block length is N (here N = 18*8 = 144), and these bits are represented by b(0) ~ b(N-1).

- CRC appending:

4 CRC bits are appended to the input data for error-detection. These 4 CRC bits b(N) ~ b(N + 3) are generated by the generator polynomial: g(D) = D4 + D + 1.

The encoding is performed in a systematic form, which means that, in GF(2), the polynomial:

-	b(0)DN+3 + b(1)DN+2 +...,+b(N-1)D4 + b(N)D3 + b(N+1)D2 + b(N+2)D1 + b(N+3);

-	b(N) - b(N+3): the parity bits;

-	b(0) - b(N-1):  the input data bits.

When divided by g(D), this step yields a remainder equal to 0.

- Adding tail bits

Then six tail bits are appended. All these six tail bits are set to 0. After adding tail bits, the data block is

b(0) b(1) ,… b(N-1) b(N) … b(N+3) b(N+4) … b(N+9)

where b(N+4) = b(N+5) = …b(N+9) = 0

3) Convolutional coding

Then the data b(0) ~ b(N+9) is encoded using a rate ½ binary convolutional encoder. (Please see section 8.5.2.1.1 of [3].) The constraint length of this coder is equal to 7 and its generator polynomials are 171o and 133o. 2 shows the pictorial depiction of the generator polynomials. Outputs A and B represent the first and second output bits respectively of this encoder.

The convolutional coder shall be initialized to all 0 state at the beginning of each block of data.

- Figure 2.  Rate ½ convolutional coder with generator polynomials, 171o and 133o.

- The delay element represents a delay of 1 bit.

After applying convolutional coding, the length of the data block is 2*(N+4+6)=2N+20. These bits are represented by v(0) v(1) … v(2N+19).

4) Repeating

To meet the performance requirements and at the same time to make the data length after FEC coding as short as possible, we need to repeat or puncture some output bits from the convolutional coder. Repeating and puncturing can be done as in the following:

v(0) v(1) … v(2N+19) was fed into the repeating module. Suppose the data block length after repeating is Y (here Y = 384bits corresponding to 40ms) and these bits are represented by t(0) t(1) … t(Y-1). Then the repeating is done as follows:

For all input data v[i], i = 0 ~ 2N+19

{

If (i is one element of set S)		repeating is performed, i.e, output bit v[i] twice.

Else				output bit v[i] once.

}

4.  Decoding of FEC on Beacon Header

Suppose the received data block length is Y (here Y = 384 bits corresponding to 40ms), and these data are represented by r(0) ~ r(Y-1).

- De-repeating

For all input data r[i], i = 0 ~ (Y-1)

{

If (i is one element of set S)

{

the output value equals to the average value of r[i] and r[i+1], and then jump to process r[i+2].

}

Else

{

the output value equals to r[i], and then continue to process next input data r[i+1].

}

}

The data block length after de-repeating is 2*(N+4+6)=2N+20 (here N = 144), and these data are represented by v’(0) ~ v’(2N+19).

- De-convolutional coding

The v’(0) ~ v’(2N+19) were fed into the decoding module of the convolutional code, which usually adopts a Viterbi decoding. (Please refer to [2].) The data block length after the de-convolutional coding module is N+10 (here N = 144) and these bits are represented by b’(0) ~ b’(N+9).

- Removal of tail bits

The six tail bits were removed. After that, the data block Llength will be N+4 (here N = 144). These bits are represented by b’(0) ~ b’(N+3).

- CRCcCheck module

In GF(2), this block can be represented by b’(0)DN+3 + b’(1)DN+2 +...+b’(N-1)D4 + b’(N)D3 + b’(N+1)D2 + b’(N+2)D1 + b’(N+3). When it is divided by g(D), this step yields a remainder equal to 0. We recover the Beacon Header successfully. Otherwise, we failed to recover the Beacon header.

Reference:

[1] IEEE802.22-07-0071-01-0001_TG1_Preliminary_Draft_Clause_5&6

[2] Digital Modulation and Coding by Stephen G. Wilson;

[3] IEEE P802.22/D0.2 Draft Standard for Wireless Regional Area Networks Part 22: Cognitive Wireless RAN Medium Access Control (MAC) and Physical Layer (PHY) specifications: Policies and procedures for operation in the TV Bands