IEEE P802.22
Wireless RANs

| Proposed Resolutions for Comments Related to the Superframe |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-MM-DD |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Dave Cavalcanti | Philips | 345 Scarborough Rd, Briarcliff Manor, NY | 914-945-6083 | Dave.Cavalcanti@philips.com |
| Carlos Cordeiro | Philips | 345 Scarborough Rd, Briarcliff Manor, NY | 914-945-6091 | Carlos.Cordeiro@philips.com |

Abstract

This submission describes the proposed normative text to resolve the following comments: 16, 116, 117, 118.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

This document contains proposed normative text to resolve the following comments related to the superframe structure: 16, 116, 117, and 118.

# Comments #16, 116, 117, 118

    - Superframe Control Header

In clause 6.5.1 Superframe Control Header, replace the text between 30 and 43 by the following:

The SCH specification is shown in Error: Reference source not found. The SCH provides essential information about the 802.22 cell, which is used to control over the time a device takes to join the network, support coexistence with incumbents and FCC Part 74 devices, support self-coexistence mechanisms, and support the two-stage mechanism for management of quiet periods for sensing (fast and fine sensing, as described in 6.21.3.2).

Transmission of a SCH indicates two possible types of content may follow: a superframe (see Error: Reference source not found) or a coexistence beacon (see Error: Reference source not found). The CT field serves to identify the purpose for the transmission of the SCH. This distinction is needed to support CBP which is employed to implement better self-coexistence and sharing of the radio spectrum with other 802.22 systems. The use of the remaining fields in the SCH is described in Error: Reference source not found. Since a SCH may contain further IEs, the Length field is used to specify the total length of the SCH.

In clause 6.5.1 Superframe Control Header, delete the following fields from Table 1:

| FS | 4 bits | Frames per Superframe Indicates the number of frames within a superframe. Frames within a superframe have a fixed size (Error: Reference source not found) which shall not change across superframes. |
| --- | --- | --- |
| FDC | 8 bits | Frame Duration Code See Error: Reference source not found |
| PP | 1 bit | Preamble Present Indicates whether the preamble of the following frame is present or not. For example, a frame preamble may not be needed when the cell is operating in a single physical (i.e., TV) channel |
| NC | 2 bits | Number of Channels In case channel bonding is used, this field indicates the number of additional consecutive physical (i.e., TV) channels used by the BS. In the basic mode, NC = 2 (i.e., two additional TV channels). This translates into a total of 3 physical channels being bonded. |
| AW Present | 1 bit | Alert Window (AW) present Determines whether the AW (see Error: Reference source not found and Error: Reference source not found) is present in the superframe structure as part of the first frame. 0 = AW not present 1 = AW present |
| BFB | 16 bits | Bonded Frame Bitmap In case the BS is operating under bonding mode but single channel CPEs have to be supported, this field indicates frame-by-frame in the superframe which ones are bonded and which ones are single channel (see Error: Reference source not found). Bit = 0: the corresponding frame is bonded Bit = 1: the corresponding frame per TV channel |
| GIF | 1 bit | Guard Interval Factor Specifies the GIF used by the PHY in the frame transmissions of this superframe. Pre-determined values are: 4 = Default mode used for superframe transmission |