IEEE P802.22
Wireless RANs

| Channel Models with Non-Integer Delays |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-04-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Stephen Kuffner | Motorola | 1301 E. Algonquin Road Schaumburg IL, USA | 847-538-4158 | stephen.kuffner@ motorola.com |

Abstract

For channel models with multipath delays that are integer multiples of 1 µsec as described in [1], the resulting channel response demonstrates 1 MHz periodicity.  This is not very representative of actual physical channels, but by randomly perturbing the delays as in [2], the channel can be made more realistic.  For system simulation convenience, it would be better to use integer multiples of the system clock rate.  This contribution shows how the integer multiple system clock rate delays “randomize” the channel frequency response compared to the nominal delays of [1].

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

### Nominal Delay Spread

The nominal delays from [1] are shown below in Table I.

| # Profile A | Path 1 | Path 2 | Path 3 | Path 4 | Path 5 | Path 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Excess delay (μsec) | 0 | 3 | 8 | 11 | 13 | 21 |
| Relative amplitude | 0 | -7 dB | -15 dB | -22 dB | -24 dB | -19 dB |
| # Profile B | Path 1 | Path 2 | Path 3 | Path 4 | Path 5 | Path 6 |
| Excess delay (μsec) | -3 | 0 | 2 | 4 | 7 | 11 |
| Relative amplitude | -6 dB | 0 | -7 dB | -22 dB | -16 dB | -20 dB |
| # Profile C | Path 1 | Path 2 | Path 3 | Path 4 | Path 5 | Path 6 |
| Excess delay (μsec) | -2 | 0 | 5 | 16 | 24 | 33 |
| Relative amplitude | -9 dB | 0 | -19 dB | -14 dB | -24 dB | -16 dB |

Table I.  Nominal delay spread profiles from [1].

### System Clock-Related Delay Spreads

The assumption for the system clock is 6 MHz x 8/7 or 6.857142… MHz.  The clock-rate related delays are the closest integer multiple of the clock period.  The corresponding profiles are shown in Table II.

| # Profile A | Path 1 | Path 2 | Path 3 | Path 4 | Path 5 | Path 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Nominal excess delay (μsec) | 0 | 3 | 8 | 11 | 13 | 21 |
| Clock-related delay (cycles) | 0 | 20 | 54 | 75 | 89 | 144 |
| Clock-related delay (μsec) | 0 | 2.9167 | 7.8750 | 10.9375 | 12.9792 | 21.0000 |
| Relative amplitude | 0 | -7 dB | -15 dB | -22 dB | -24 dB | -19 dB |
| # Profile B | Path 1 | Path 2 | Path 3 | Path 4 | Path 5 | Path 6 |
| Nominal excess delay (μsec) | -3 | 0 | 2 | 4 | 7 | 11 |
| Clock-related delay (cycles) | -20 | 0 | 13 | 27 | 48 | 75 |
| Clock-related delay (μsec) | -2.9167 | 0 | 1.8958 | 3.9375 | 7.0000 | 10.9375 |
| Relative amplitude | -6 dB | 0 | -7 dB | -22 dB | -16 dB | -20 dB |
| # Profile C | Path 1 | Path 2 | Path 3 | Path 4 | Path 5 | Path 6 |
| Nominal excess delay (μsec) | -2 | 0 | 5 | 16 | 24 | 33 |
| Clock-related delay (cycles) | -13 | 0 | 34 | 109 | 164 | 226 |
| Clock-related delay (μsec) | -1.8958 | 0 | 4.9583 | 15.8958 | 23.9167 | 32.9583 |
| Relative amplitude | -9 dB | 0 | -19 dB | -14 dB | -24 dB | -16 dB |

Table II.  Modified delay spread profiles.

Figure 1.	Channel model A with nominal delays (top) and integer multiple of the system clock delays (bottom).  Note 1 MHz periodicity with nominal delays.

Figure 2.	Channel model B with nominal delays (top) and integer multiple delays (bottom).

Figure 3.	Channel model C with nominal delays (top) and integer multiple delays (bottom).

### Conclusion

The nominal delays that were assumed in [1] resulted in a periodic-in-frequency channel response that was not representative of physical channels.  Randomizing the delays would remove the periodicity, but delays that are integer multiples of the system sampling clock are more convenient for simulations and also remove the periodicity of the response.

References:

[1]	IEEE 802.22-05/0055r7, “WRAN Channel Modeling,” Eli Sofer, Gerald Chouinard, August 2005.

[2]	IEEE 802.22-07/0166r0, “802.22 TG1 Fading Statistics,” Stephen Kuffner, April 2007.