IEEE P802.22
Wireless RANs

| WRAN Discrete Channel B |  |  |  |  |
| --- | --- | --- | --- | --- |
| Last Updated - Date: 2007-03-23 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Yonghong Zeng | Institute for Infocomm Research | 21 Heng Mui Keng Terrace, Singapore 119613 | 65-68748211 | yhzeng@i2r.a-star.edu.sg |
| Changlong Xu | Institute for Infocomm Research | 21 Heng Mui Keng Terrace, Singapore 119613 | 65-68748211 | clxu@i2r.a-star.edu.sg |
| Ying-Chang Liang | Institute for Infocomm Research | 21 Heng Mui Keng Terrace, Singapore 119613 | 65-68748225 | ycliang@i2r.a-star.edu.sg |

Abstract

This document presents the discrete channel Matlab code for WRAN channel B for the IEEE 802.22 WRANs.

.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Document Policy and Procedures: The contributor is familiar with the IEEE 802 Document Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of document(s), including document applications, provided the IEEE receives assurance from the document holder or applicant with respect to documents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of document information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if documented technology (or technology under document application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Document Committee Administrator at <patcom@ieee.org>.

%% File: chan_b_test.m

%% Test of the discrete channels for WRAN channel B based on the

%% IEEE 802.22 WRAN Channel Models

%% Author: Yonghong Zeng, Chang-Long Xu and Ying-Chang Liang from

%% the Institute for Infocomm Research (I2R), Singapore

randn('state',100); %% choose the seed of the random number generator

h=wran_chan_b

%% File: wran_chan_b.m

%% Discrete channel for WRAN channel B based on the IEEE 802.22 WRAN Channel Models

%% Assume that the rectangular function is used as the pulse shaping filter

%% The channel is time variant, but assumed to be invariant within one OFDM

%% block, because the Doppler shift is relatively small

%% Author: Yonghong Zeng, Changlong Xu and Ying-Chang Liang from

%% the Institute for Infocomm Research (I2R), Singapore

function h=wran_chan_b;

%% h: the output discrete channel

cj=sqrt(-1);

Ts=7/48; %% sampling period in micro-seconds (us) for 6 MHz bandwidth

%% and over-sampling factor 8/7

path_delay=zeros(6,1);     %% path delays in micro seconds (us)

path_fadding=zeros(6,1);  %% expectation of path amplitute (relative to the strongest path)

Doppler_shift=zeros(6,1);  %% Doppler shifts for signals in different paths (Hz)

path_delay=[-3;0;2;4;7;11];

path_fadding=[10^(-0.6);1;10^(-0.7);10^(-2.2);10^(-1.6);10^(-2.0)];

Doppler_shift=[0.1;0.0;0.13;2.5;0.17;0.37];

NT1=floor(-3/Ts);  %% start time of the channel is NT1*Ts

NT2=ceil(11/Ts);   %% end time of the channel is NT2*Ts

NT=NT2-NT1+1;

h=zeros(NT,1);      %% the discrete channel B

chan_time=0;

cjp=cj*2*pi;

for m=1:6

fadding=(randn(1,1)+cj*randn(1,1))/sqrt(2);

for n=1:NT

frac_delay=n-1+NT1-path_delay(m)/Ts;

match_coe=0;   %% matched filter coefficient

if (frac_delay>-1)*(frac_delay<=0)>0;

match_coe=1+frac_delay;

end

if (frac_delay>0)*(frac_delay<=1)>0;

match_coe=1-frac_delay;

end

h(n)=h(n)+path_fadding(m)*fadding*exp(cjp*Doppler_shift(m)*chan_time*1e-6)*match_coe;

end

end

h=h/norm(h);