IEEE P802.22
Wireless RANs

| Proposed MAC management messages resolutions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-09-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Dave Cavalcanti | Philips | 345 Scarborough Rd, Briarcliff Manor, NY | 914-945-6083 | Dave.Cavalcanti@philips.com |

Abstract

This submission contains the normative text proposed to resolve MAC comments related to measurement messages and to harmonize the MAC layer with the Spectrum Sensing Function  241, 244, 245.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

This document includes the proposed resolutions to comments 241, 244, 245.

Comment 241

In clasue 6.9.22.1.1.1 Signal Specific Measurement Request, insert the following fields in Table 167 after the field Duartion:

| Sensing Mode | 3 bits | Specifies which SSF outputs are valid and in some cases it specifies the behaviour of the SSF. 000 – Sensing Mode 0 001 – Sensing Mode 1 010 – Sensing Mode 2 011 – Sensing Mode 3 (see definition of Sensing Modes in Section 9.3.1.1) |
| --- | --- | --- |
| Maximum Probability of False Alarm | 8 bits | As defined in the SSF (Section 9.3.1.1) |

Comment 244

In clause 6.9.22.3.1.1 Signal Specific Measurement Report, insert the following fields in Table 178, after the field Channel Number:

| Sensing Mode | 3 bits | Specifies which SSF outputs are valid and in some cases it specifies the behaviour of the SSF. 000 – Sensing Mode 0 001 – Sensing Mode 1 010 – Sensing Mode 2 011 – Sensing Mode 3 (see definition of Sensing Modes in Section 9.3.1.1) |
| --- | --- | --- |
| Signal Present Output | 1 bits | Indicates the decision from the CPE sensing function regarding the presence of the target signal in the channel measured: 0 – FALSE: Indicates the signal is NOT present in the channel (see Section 9.3.1.2) 1 – TRUE: Indicates the signal is present in the channel (see Section 9.3.1.2) |
| Confidence Metric | 8 bits | This number indicates confidence level in the signal present decision reported in the Signal Present Output. A confidence metric varies between a minimum of zero (0) indicating no confidence in the signal present decision and a maximum of one (1) indicating total confidence in the signal present decision. If Report Mode =0, the confidence metric shall be set to zero. |

In Table 178, replace the contents of the Notes field for Duration, Value and Precision by the following:

| Duration | 16 bits | The actual duration, in units of symbol period, of the measurement |
| --- | --- | --- |
| Value | 10 bits | Estimate of the strenght of signal type measured at the location of the station (base station or CPE). This is the Field Strength Estimate Value measured in dBV/m (see definition in Section 9.3.1.2) |
| Precision | 6 bits | The standard deviation of the Field Strength Estimated reported. |

Comment 245

In clause 6.8.22.3.1.1 Signal Specific Measurement Report, insert the following field in Table 179 after the field Unmeasured:

| No_decision | 1 bit | This bit shall be set to 1 to indicate the measurement was carried out but no decision was made (see Section 9.3.1.2). |
| --- | --- | --- |

In Tables 181, 183, 186, 188, and 189, increase the size of the Report Mode field from 4 bits to 5 bits