IEEE P802.22
Wireless RANs

| Protection Criteria in the Broadcast Bands |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-05-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Charles W. Einolf, Jr. | NABA | 3007 Argentina Place Mitchellville, MD 20716 USA | +1-301-806-1544 | c.einolf@ieee.org |

Abstract

This document discusses the internationally accepted protection criteria required for devices such as wireless RANs having emissions in the broadcast bands.  These criteria must be considered to ensure adequate interference protection to licensed services having a frequency allocation in the ITU-R Radio 
Regulations.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Protection Criteria in the Broadcast Bands

The IEEE 802.22 Standards Project has as its scope to specify “the air interface … of fixed point-to-multipoint wireless regional area networks operating in the VHF/UHF TV broadcast bands between 54 MHz and 862 MHz.”  The project purpose goes on to say that “this standard is intended to enable deployment of interoperable 802 multivendor wireless regional area networks products, …, while preventing harmful interference to incumbent licensed services in the TV broadcast bands.

The International Telecommunications Union – Radiocommunication Sector (ITU-R) has addressed the issue of interference from devices that produce emissions in the frequency bands allocated to the terrestrial broadcasting services.  The ITU-R has adopted and approved Recommendations ITU-R BT.1786 and ITU-R BS.1768 entitled “Criterion to assess the impact of interference to the terrestrial broadcasting service.”  These Recommendations set the following standard in its Recommends:

“that the total interference to systems operating in the broadcasting service, from all sources of interference, as prescribed in the Recommendation” ITU-R SM.1757, “should at no time exceed one per cent of the total receiving system noise power”

Note that the total interference includes the sum of all interference sources.  Furthermore, the protection criterion of one percent is equivalent to – 20 dB.  Therefore, since TV broadcast receivers are expected to operate in noise limited environments at their noise threshold, the total interference from IEEE 802.22 devices must remain 20 dB below the noise threshold at the receiver RF input.

The IEEE 802.22 devices are not afforded a frequency allocation in accordance with the ITU-R Radio Regulations.  Consequently, in order for the IEEE 802.22 project to fullfill is stated purpose that “the standard may be applicable in global regulatory environments”, the IEEE 802.22 standard must be compliant with the internationally recognized standards, Recommendations ITU-R BT.1786 and ITU-R BS.1786.  It is requested that the IEEE 802.22 standard refer to these ITU-R Recommendations as a minimum interference criterion.

References:

Recommendation ITU-R BT.1786, “Criterion to assess the impact of interference to the

terrestrial broadcasting service (BS)”, International Telecommunications Union, April 2007.

Recommendation ITU-R BS.1786, “Criterion to assess the impact of interference to the

terrestrial broadcasting service (BS)”, International Telecommunications Union, April 2007.