IEEE P802.22
Wireless RANs

| Merging of ranging and geolocation functions for the WRAN systems |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-05-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Gerald Chouinard | CRC | 3701 Carling Avenue, Ottawa, Ontario, Canada K2H 8S2 | 1-613-998-2500 | gerald.chouinard@crc.ca |

Abstract

The ranging and geolocation functions for the 802.22 WRAN systems relate to very fundamental PHY timing functions and their requirements should normally refer to common system functionality.  This contribution tries to rationalize the requirements for both functions and identify how they can be achieved by specifying fundamental system behavior to accommodate both.

This version (r2) contains comments from “JS” who needs to be identified, and responses from the original author, Gerald Chouinard.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Merging of ranging and geolocation functions for the WRAN systems

The ranging and geolocation functions for the 802.22 WRAN systems relate to very fundamental PHY timing functions and their requirements should normally refer to common system functionality.  This contribution tries to rationalize the requirements for both functions and identify how they can be achieved by specifying fundamental system behavior to accommodate both.

The ranging requirement for proper WRAN operation is to be able to align the CPE transmission timing so that its upstream burst arrives at the base station within ¼ of the cyclic prefix.  On top of allowing for the absorption of the channel time spreading which would otherwise create inter-symbol interference, the cyclic prefix is used as a buffer to absorb timing misalignment of US bursts coming from various CPEs when it arrives a the base station. Since the smallest cyclic prefix proposed for the WRAN standard is 9.3 μsec (1/32), the tolerance for the initial ranging is therefore 2.33 μsec.

The geolocation requirement can be much tighter, down to a few nanoseconds to allow for the needed precision in the triangulation process to find the precise location of each CPE.

When a CPE tries to associate to a WRAN base station, it is supposed to be synchronized in time and in frequency with the BS after having received, demodulated and decoded the downstream burst.  Having done that, it is capable of transmitting an upstream burst on the right sub-carriers and at the right time, that is ‘its local time’.  However, since the ranging has not taken place yet, it does not know at what time it should send its burst so that it arrives in time at the base station.

The best thing that it can do is to send its ranging burst at the ‘local time’ of the start of the upstream sub-frame (i.e., after the TTG) as indicated in the US-MAP of the frame header.  If this CPE is located close to the base station, this burst will arrive at the BS at the start of the US sub-frame with a negligible delay.  However, if the CPE is located at 100 km from the base station, the extra delay due to the signal propagation in both directions will amount to 666.7 usec for the 200 km path length, that is about 2 symbols delay.

I have a comment which can be another solution. BS shall deteremine the TTG with considering the RF switching time and the maximum round trip delay of the cell coverage.

As can be seen in the 22-06-0264-04-0000_OFDMA_Parameters.xls spreadsheet, the TTG is defined set at 210 usec, that is 10 usec for the CPE switching time between receive and transmit modes and the propagation time for 30 km, forward and back.

When CPE performs the ranging, CPE can transmit the ranging burst just after the receiving the downstream and rx-tx switching time. If this CPE is located close to the base station, this burst will arrive at any time within the TTG period of the BS, that is, earlier than the starting time of the US sub-frame in the BS.

You would want to have the ranging burst arrive at the BS at the start of the US sub-frame as for the other US bursts so that normal BS operation can take place.  Since the maximum distance that a CPE can be located is 110 km, the round trip delay would be as long as 666.7 usec, the equivalent of 2 symbols.

If this CPE is located at cell edge, this burst will arrive at the almost start time of the US sub-frame. In this case, BS can demodulate on the first symbol of the US sub-frame. It means ranging can be performed 3 three symbols except a buffer of 2 symbols.

As indicated above, the round trip delay is as large as 2 symbols.  I agree that if the new CPE knew at what distance it is from the base station, it could skip the TTG and send its ranging burst right away.  This would shorten the largest propagation delay from 666.7 usec to 466.7 usec.  We would still need a buffer of 2 symbols.  As you can see below, I propose to skip the TTG in very special circumstance to bridge the gap between 94 kM and 100 km for the very few CPE’s that could be located at such large distance but this would not be common.  In fact, now that we know the size of the symbols, we could define the maximum distance to be cavered at 94 km and we would not need this special case.

To be more precise, the distance that can be absorbed by a two symbol delay and a cyclic prefix is indicated in the following table for the four cyclic prefixes considered in the 808.22 standard:

| Cyclic prefix | Time buffer | Distance |
| --- | --- | --- |
| 1/4 | 2*373+75= 821 μsec | 123 km |
| 1/8 | 2*336+37= 709 μsec | 106 km |
| 1/16 | 2*317+19= 653 μsec | 98 km |
| 1/32 | 2*308+ 9= 625 μsec | 94 km |

It is therefore safe to keep a buffer of 2 symbols to allow for the round trip delay of the ranging burst from the distant CPE (a simple software switch could be used at the distant CPE installations to skip the TTG, therefore increasing the maximum distance from 94 km to 124 km).

This means that, depending on the distance of the CPE, the start of the ranging burst can arrive at the BS at any time within this two symbol buffer.  It also means that the base station will have to decode for a possible ranging burst over a period of 3 symbols to be able to capture the one-symbol ranging burst to be able to demodulate it properly.  In order to avoid that the base station demodulates only on a portion of a ranging burst, it will have to wait 2 symbols from the start of the upstream sub-frame and begin its demodulation on the third symbol.  As a consequence, in order to allow the ranging burst of the close-in CPEs to be demodulated properly, the ranging burst will also need to extend over three OFDMA symbols.

Since the round-trip delay can fall anywhere within the "two symbols and a cyclic prefix" as established above, this ranging burst will need to meet two conditions:

- the same set of subcarriers will need to be used throughout the three symbols, and

- phase continuity will need to be preserved throughout this 3-symbol ranging burst

As a result of the second condition, special measures will need to be taken with the cyclic prefix to make sure that there is phase continuity across the 3-symbol ranging burst.

As a result of the first condition, the normal subcarrier diversity distribution will not be used for the ranging sub-channels and the normal pilot carriers will not be allowed in these ranging sub-channels.  This means that a special static subcarrier distribution will need to be defined for these ranging sub-channels (i.e., subcarrier spreading will be the same for successive symbols).  In order to keep these ranging sub-channels robust against frequency selective fading, this subcarrier distribution will need to distribute these subcarriers across the entire TV channel as much as possible.  While this is done, it will be easy to design the distribution to also meet the geolocation requirements such as providing for large carrier spacing close to the channel width to provide for the necessary accuracy (6 MHz gives 1 m precision with 64-QAM demodulation), as well as close carrier spacing to allow geolocation over an extended range (3 kHz carrier spacing allows for up to 100 km range).

The normal distributed subcarrier allocation in the upstream can be also used for the ranging subchannel since distributed subcarrier allocation of US is different from DS.

This does not seem to be the case for the ranging sub-channels where a CDMA code which will modulate all the 140 carriers (none left for pilots) will be used to improve upon noise at initial association.

In US, BS can obtain the channel information from pilots.

True for normal operation but not for the CDMA ranging burst.

So, the same set of subcarriers are allocated for US subchannel during at least 7 symbols or US-subframe since only one pilot is located during the 7 symbols on each subcarrier.

True for normal operation but not for the CDMA ranging burst.  It is not clear to me whether the distribute and AMC permutations can switch between all carriers minus the pilot carriers and minus the 140 ranging carriers and the normal operation where it will randomize on all carriers minus the pilot carriers.  I believe that for the sake of simplicity, one a carrier spreading scheme is used at the start of a sub-frame, it would be applicable throughout the whole US sub-frame.

Besides, even if it was allowed to switch, it could only do so for US sub-frames equal to or larger than 12 symbols since the CDMA ranging burst occupies 5 symbols and 7 symbols are needed for normal carrier spreading to recover the channel state information.  It would only be applicable for a fraction of the time when the US capacity needs to be high.  This is why I suggest that the opportunistic ranging should last the entire US sub-frame and then we could take advantage of sending the other CDMA bursts for UCS notification and BW-request in the symbols following the prime ranging 5-symbol window to reduce the contention.

Anyway, if the specific distributed subcarrier allocation is needed to support the geolocation requiremtns, you can define another distributed subcarrier allocation, as you mentioned, and ranging can also use this specific allocation.

Then we would have to concatenate three different pilot distribution schemes.  I feel that since the distribution of the ranging sub-carriers is static over the 5-symbol window, it would be easier to include the geolocation requirement in the definition of this static distribution.  It couls also be done as you propose but I think it would be more complex.

Special attention is therefore needed to properly design the static subcarrier distribution that will be used for the ranging sub-channels.  Once these specific subcarriers are selected, they will be set aside, similarly to the pilot subcarriers, and the subcarriers left will be the ones used for normal subcarrier distribution for the data payload, either for the normal distributed or the AMC arrangements.

This special distribution of carriers for the symbols transmitted in the upstream ranging subchannels will allow the computation of the precise geolocation at the base station in parallel with the normal processing of the ranging, UCS-notification and BW-request bursts.  The output of the FFT for these specific ranging subcarriers will be sent to two different signal processing circuits.  One will recover the CDMA code to deal with the appropriate function (ranging, UCS-notification or BW-request) and the specific CPE that sent it, and the other circuit will process the phase of the subcarriers to help establish the precise distance to the specific CPE.

Since this special subcarrier distribution will be used in the ranging sub-channels and that no pilot carriers will be transmitted, these ranging sub-channels will not be available for other uses than for ranging, UCS-notification and BW-request bursts. The payload data will be transmitted on the other 55 sub-channels.

Knowing that the minimum upstream sub-frame will need to be 7 symbols for the normal sub-channels to recover the channel state information, and that the number of symbols for the ranging burst is 3 symbols with a delay buffer of 2 symbols, there will be 2 symbols available at the end of the sub-frame that could carry UCS-notification and BW request information.  Since the CPEs are synchronized to the BS when they are in a position to send these UCS-notification and BW-request bursts, they only need one symbol to do it.  This means that these UCS-notification and BW-request bursts could predominantly be transmitted during these two last symbols rather than using the previous symbols of the ranging sub-channels.  This would reduce the contention on the first 5 symbols of the sub-frame.

This reduction in contention will apply even more if the upstream sub-frame is longer than the minimum 7 symbols because of the variable DS/US split.  As an example, if the US sub-frame is 11 symbols wide, this would give 6 symbols to be chosen from for the UCS-notification and BW-request bursts.  A mechanism to randomly spread these burst on the available symbols would need to be included in the CPE local MAC routines.

Having established this reduction in the contention for the ranging sub-channels because of the temporal distribution of the ranging, UCS-notification and BW-request bursts, it is fair to ask whether 5 sub-channels are really needed for these functions.  Five sub-channels over 60 represents 8.3% in overhead in the US sub-frame.  Any reduction in this number of ranging sub-channels would be welcome.  This would mean a shorter CDMA code, which would result in a small number of orthogonal codes, which would also be in line with the reduced contention on the ranging sub-channels. As we saw above, since the ranging, UCS-notification and BW-request bursts can be distributed in time, less contention is needed.

The other aspect is the noise improvement brought up by the use of the CDMA code.  This has to be considered with the fact that if less sub-channels are used for the transmission of these special bursts, each subcarrier would be transmitted at a higher level for the same total transmitted power at the CPE.  Some simulation work should be done to identify the difference in SNR between the various CDMA spreading power resulting from the use of 1, 2, 3, 4 and 5 sub-channels and the SNR for the normal robust data payload (QPSK, rate: 1/2).  In fact, it is not clear what this difference should be for sufficiently reliable CPE ranging and association as well as UCS-notification and BW-request.

22-06-0264-04-0000_OFDMA_Parameters.xls

22-07-0208-00-0000-Ranging for WRAN system.ppt

22-07-0015-00-0001_Reede_Geoloc_OFDM_Simulations.ppt