IEEE P802.22
Wireless RANs

| Proposed Normative Text for the Resolution of Comments Related to the Spectrum Manger |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-05-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Carlos Cordeiro | Philips | 345 Scarborough Rd, Briarcliff Manor, NY | 914-945-6091 | Carlos.Cordeiro@philips.com |
| Dave Cavalcanti | Philips | 345 Scarborough Rd, Briarcliff Manor, NY | 914-945-6083 | Dave.Cavalcanti@philips.com |

Abstract

This submission contains the normative text proposed to resolve MAC comments related to the spectrum manager. The following comments are addressed: 4, 5, 6, 7, 8, 507, 385, 386, and 387.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Resolution to CID 5

In clasue 6.1 Overview, Page 11, Line 2, delete the following text:

“and typically broadcast, while CPEs shall listen only to those messages addressed to them”

Resolution to CID 6, 7, 8, 507, 385, 386, 387

In clasue 6.1.1 Reference Architecture, page 11, replace the current text betweeen lines 43-51 by the following text:

The unique and distinctive characteristic of this architecture is that it is scalable and so its capacity can be expanded over time, as the need arises. Hence, it is comprised of at least one PHY/MAC air interface module and a unique entity called Spectrum Manager (SM). This is supported by the architecture shown in Error: Reference source not found.

In clasue 6.1.1 Reference Architecture, page 12, replace Figure 6 by the following Figure

In clasue 6.1.1 Reference Architecture, page 12, replace the sentences betweeen lines 4-13 by the following text:

The SM has a key role in the overall architecture as it allows the system to take advantage of multiple channels while keeping the simplicity of the MAC/PHY and allowing the system to scale (and also evolve) over time. In other words, the SM allows for an effective multiple channel mechanism to be implemented. The SM is responsible for maintaining an updated global view of the target RF spectrum and assigning the identified vacant channels for use by the various MAC/PHY modules for operation (similar to a resource allocator).

Through the MAC, the SM provides input to the Spectrum Sensing Function described in Section 9.1 and receives the outcome of its sensing measurements and from the Channel Selection Function as described in Section 9.2.1, which includes the ACL, and UCL. The SM shall combine the information from the SSF and CSF with any other information about the availability of the channels obtained from other sources other than the SSF (e.g. geolocation information). Based on this combined information and on predefined sharing rules,,the SM assigns channels to the MAC/PHY modules. This could be based on several criteria such as number of CPEs associated to each of these modules, traffic requirements, ranging (e.g., lower frequency channels could be assigned to the module dealing with farther away terminals), and so on.

In clasue 6.1.1 Reference Architecture, page 12, delete Figure 7

In clasue 6.1.1 Reference Architecture, page 12, delete text between lines 21-27

In clasue 6.1.1 Reference Architecture, page 13, delete text between lines 1-14

Resolution to CID 4

After applying the resolutions above, move and renumber the updated Section 6.1.1 Reference Architecture as Section 1.4 System Overview