IEEE P802.22
Wireless RANs

| TG1 Link Margin Simulation Results Using Static Multipath Magnitudes |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-05-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Stephen Kuffner | Motorola | 1301 E. Algonquin Rd., Schaumburg, IL, USA | 1-847-538-4158 | Stephen.Kuffner@ Motorola.com |

Abstract

Successful coexistence between a Part 74 deployment and a WRAN network is dependent on two physical links: the WRAN-to-microphone-receiver interference link, which results in reduced sensitivity for wireless microphone reception, and the beacon-to-WRAN-receiver detection link, which carries the parameters of the microphone deployment required for protection.  The required link margin in the detection link will depend on the margin in the interference link.  The difference between the two links depends on two parameters: the expected upfade of the microphone interference and the expected downfade of the beacon signal, expressed as a required signal to noise ratio (as Ec/No or Es/No) for the beacon link, both using an assumed channel model.  This set of results assumes the channel models described in [1], using normalized static ray magnitudes, random phase draws for each ray for each packet, but zero Doppler shifts.  Subsequent results will compare the performance for Rayleigh faded rays for both links.  The actual physical channel will likely be bounded by the fixed rays and the Rayleigh faded rays.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

### Introduction

Previous submissions (see e.g. [2], [3]) have considered the TG1 interference scenario as being dependent on two links: the WRAN-to-microphone-receiver interference link and the TG1-transmitter-to-WRAN-receiver beacon link.  These links were considered to have the same large scale properties (such as average path loss), but would experience different frequency selective fading because 1) they are relatively narrowband waveforms on different frequencies (albeit within the same TV channel) and 2) the microphone receiver and beacon transmitter are in slightly different locations, giving slightly different relative phase shifts on each ray.  The analysis is framed on the premise that the beacon packet need not be successfully detected if the WRAN interference into the microphone receiver is below some maximum value.

The conservative assumptions in the previously cited work had the average gain of the microphone interference upfaded by 6 dB while the required Es/No for the beacon link, assuming Rayleigh faded rays, needed to be on the order of 20 – 30 dB, depending on the desired PER.  The present analysis examines the statistics of the maximum microphone interference channel across a 6 MHz TV channel in tandem with the required Ec/No values for beacon detection.  No FEC is assumed at this time since the objective is to determine the resulting link margin and the corresponding FEC needs.

### Simulation Model

The functional block diagram of the simulation is shown in Figure 1.  The simulator assembles the sync word and index bursts into a sync frame for the I-channel and uses a random Q-channel.  A frame structure based on the 3-octet multiple is assumed, giving an overall length of 21 octets (168 bits).  These octets are differentially encoded and DQPSK modulated, spread by the spreading sequence of [1-j -1-j 1+j 1-j 1-j 1-j -1+j -1-j], and square root raised cosine filtered for transmission through the channel.  The receiver applies a matched filter and despreader prior to DQPSK detection and differential decoding.  Timing is based on block processing and the best despreader correlation.  A packet error is only accounted if there is an error in the Q channel (beacon header).  The frequency offset is assumed to be zero in this model.

The microphone interference per subchannel is calculated analytically (see Eq. 3 in [4]), and the maximum of all the 200 kHz subchannels (starting with the one centered 500 kHz above the lower channel edge to avoid operating over the beacon) is found over 10K Monte Carlo trials.

### Channel Model

Four channel models are considered in this submission: an AWGN channel and three multipath channels as described in [1] with the exception of zero Doppler spread.  The ray magnitudes are normalized to give average power gain of 1 for both the beacon communication channel and the microphone interference channel.

Figure 1.  Functional block diagram for simulation.

### Results

Figure 2 shows the packet error rate results for all four channels.  Recall that in this work an error is accounted only when there is an error in the Q channel.  The results are shown in terms of Ec/No.  With a spreading factor of 8, the Es/No is 9 dB higher.  According to this chart, an Ec/No of about 8.5 dB (Es/No = 17.5 dB) is required for channel B and 1% PER, while about 13.2 dB Es/No would suffice for 10% PER in channel B.

Table I shows the statistics for the maximum microphone interference over the same channel models with 200 kHz bandwidth.  Channel B is the worst channel in this direction as well, with the highest mean and maximum interference gain.

| Channel Model | Mean (dB) | Max (dB) | Standard Deviation (dB) |
| --- | --- | --- | --- |
| A | 2.68 | 2.81 | 0.073 |
| B | 3.45 | 3.80 | 0.208 |
| C | 2.87 | 3.03 | 0.056 |

Table I.  Maximum microphone interference gain statistics.

Figure 2.	Simulated packet error rates vs. chip Ec/No for 168-bit packets.  Only errors in the Q-channel are accounted as packet errors for the beacon header reception.

Figure 3.	Max microphone interference gain histogram for channel A.

Figure 4.	Max microphone interference gain histogram for channel B.

Figure 5.	Max microphone interference gain histogram for channel C.

### Conclusion

For channel model B and 1% PER, the previous assumptions in [2] and [3] were that an approximately 30 dB Es/No (based on channel model B with Rayleigh faded rays) would be required along with a 6 dB upfade margin F↑ as a conservative guess.  The results presented here for static ray magnitudes indicate that an Es/No of about 17.5 dB would be adequate, and that the upfade margin for channel B may be reduced from 6 dB to about 3.5 – 3.8 dB.  The combination would improve the link margin on the order of 16 dB.  For channel models A and C, which have a single large ray (-7 or -9 dB, respectively) followed by multiple weaker rays (below about -15 dB), the link margin improvement is even better, requiring only about 13 dB Es/No for 1% PER and about 3 dB maximum microphone interference, picking up about 20 dB of link margin compared to previous assumptions (channel C has slightly better Es/No than channel A but slightly worse peak microphone interference upfade, so the total link margin improvement is about the same for both).

References:

[1]	IEEE 802.22-05/0055r7, “WRAN Channel Modeling,” E. Sofer, G. Chouinard, 30 Aug 2005.

[2]	IEEE 802.22-06/0195r2, “Interference-Based Bandwidth Constraints for Proposed IEEE 802.22 TG1 Transmitters – Hata Model,” S. Kuffner, 14 Mar 2007.

[3]	IEEE 802.22-07/0148r0, “802.22 TG1 Link Margin Calculator,” S. Kuffner, 17 Mar 2007.

[4]	IEEE 802.22-07/0166r0, “802.22 TG1 Fading Statistics,” S. Kuffner, 3 Apr 2007.

Note the WRAN interference is in general not narrowband but as far as the 200 kHz bandwidth microphone receiver is concerned, it is a 200 kHz-wide emission.

The maximum interference channel is considered here since in a multi-microphone deployment there could be many 200 kHz subchannels occupied within a 6 MHz channel.  An interference condition exists if any one of those channels is experiencing interference.  The maximum is a conservative estimate.