IEEE P802.22
Wireless RANs

| MAC message for AMC subchannel |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-05-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Chang-Joo Kim | ETRI | Korea | +82-42-860-1230 | cjkim@etri.re.kr |
| Myung-Sun Song | ETRI | Korea | +82-42-860-5046 | mssong@etri.re.kr |
| Gwang-Zeen Ko | ETRI | Korea | +82-42-860-4862 | gogogo@etri.re.kr |
| Sung-Hyun Hwang | ETRI | Korea | +82-42-860-1133 | shwang@etri.re.kr |
| Jung-Sun Um | ETRI | Korea | +82-42-860-4844 | korses@etri.re.kr |

Abstract

This document is shown the MAC message to support AMC subchannel.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- MAC Message for subchannel allocation

Add the following entities in the table 48 DS-MAP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| DS_MAP_Message_Format() { |  |  |
| AMC_Subchannel | 1 bits | Indicates whether the adjacent subcarrier permutation is used in the downstream or not. |
| if (AMC_Subchannel){ |  |  |
| DS_BAND_MAP | Variable |  |
| } |  |  |
| } |  |  |

Table xx    DS_BAND_MAP

| Syntax | Size | Notes |
| --- | --- | --- |
| DS_BAND _MAP () { |  |  |
| New format Indicator | 1bit | 0 = Use the latest DS_BAND_MAP 1 = New DS_BAND_MAP It can be 1 or 0 on the first frame of the superframe, but only 0 on the other frames. |
| if(New format Indicator) |  |  |
| { |  |  |
| Nband | 6bits | The total number of AMC bands selected for AMC subchannel |
| if(Nband < 10 or Nband > 50){ |  |  |
| for(i=0 ; i<Nband ; i++) |  |  |
| BandIndex | 6bits | If Nband is less than 10, each BandIndex indicates the index of selected AMC bands for AMC subchannels. If Nband is larger than 50, it indicates the index of diversity bands not using for AMC subchannels. |
| }else{ |  |  |
| BandBITMAP | 60bits | The n-th LSB represents the n-th band. If n-th band is AMC band for AMC subchannel, it is 1. |
| } |  |  |
| } |  |  |

Table 50    DS_MAP_IE

| Syntax | Size | Notes |
| --- | --- | --- |
| DS_MAP_IE () { |  |  |
| AMC_Subchannel | 1bit |  |
| If(AMC_Subchannel){ |  |  |
| If(frame index == 0){ |  | Able to change the bands for AMC subchannel on the only first frame due to using the same band during the superframe. |
| Selected Band BITMAP | variable | It represents the index of bands to be allocated for CPE among the all AMC bands selected for AMC subchannel. It consists of the bits with same value of Nband. The n-th bit is 1 if n-th band is allocated. |
| } |  |  |
| }else{ |  |  |
| Channel Offset | 7bits |  |
| Number of Channels | 7bits |  |
| Slot Offset | 8bits |  |
| Number of Slots | 8bits |  |
| } |  |  |
| } |  |  |

Add the following entities in the table 60 US-MAP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| US_MAP_Message_Format() { |  |  |
| AMC_Subchannel | 1 bits | The identifier whether the adjacent subcarrier permutation is used in the US or not. |
| If (AMC Subchannel){ |  |  |
| US_BAND_MAP | Variable |  |
| } |  |  |
| } |  |  |

Table xx    US_BAND_MAP

| Syntax | Size | Notes |
| --- | --- | --- |
| DS_BAND _MAP() { |  |  |
| New format Indicator | 1bit | 0 = Use the latest US_BAND_MAP 1 = New US_BAND_MAP It can be 1 or 0 in the first frame of the superframe, 0 in the other frames. |
| if(New format Indicator) |  |  |
| { |  |  |
| Nband | 6bits | The total number of AMC bands selected for AMC subchannel |
| if(Nband < 10 or Nband > 50){ |  |  |
| for(i=0 ; i<Nband ; i++) |  |  |
| Band Index | 6bits | If Nband is less than 10, each BandIndex indicates the index of selected AMC bands for AMC subchannels. If Nband is larger than 50, it indicates the index of diversity bands not using for AMC subchannels. |
| }else{ |  |  |
| BandBITMAP | 60bits | The n-th LSB represents the n-th band. If n-th band is AMC band for AMC subchannel, it is 1. |
| } |  |  |
| } |  |  |
| } |  |  |

Table 61    US_MAP_IE

| Syntax | Size | Notes |
| --- | --- | --- |
| DS_MAP_IE () { |  |  |
| AMC_Subchannel | 1bit |  |
| If(AMC_Subchannel){ |  |  |
| If(frame==0){ |  | Able to change the bands for AMC subchannel on the only first frame due to using the same band during the superframe. |
| SelectedBandBITMAP | variable | It represents the index of bands to be allocated for CPE among the all AMC bands selected for AMC subchannel. It consists of the bits with same value of Nband. The n-th bit is 1 if n-th band is allocated. |
| } |  |  |
| } |  |  |

Channel measurement Report for Request/Response (CMR-REQ/RSP)

Add the following entities in the Table 43.

| type | Message | Description | Connection |
| --- | --- | --- | --- |
| 60 | CMR-REQ | Channel measurement Report Request | Primary Management |
| 61 | CMR-RSP | Channel measurement Report Response | Primary Management |

CMR-REQ

BS can request the channel quality information with a CINR measurement to specific CPE to allocate the AMC subchannel.

| Syntax | Size | Notes |
| --- | --- | --- |
| CMR_REQ_Message_format () { |  |  |
| Management Message Type = 60 | 8bits |  |
| Report Type | 1bit | The CINR of new bands = 0 The differential CINR of the last assigned bands = 1 |
| } |  |  |

CMR-RSP

To respond to the CMR-REQ or request the AMC subchannel, CPE can report the CMR-RSP message containing the channel quality information with a CINR measurement of the specific bands to BS. The Mean for CINR shall be reported in units of dB. To prepare such reports, statistics shall be quantized in 1dB increments, ranging from a minimum of -10dB to a maximum of 53dB.

| Syntax | Size | Notes |
| --- | --- | --- |
| CMR_RSP_Message_format () { |  |  |
| Management Message Type = 61 | 8bits |  |
| Report Type | 1bit | The CINR of new bands = 0 The differential CINR of the last assigned bands = 1 |
| If(ReportType = 1){ |  |  |
| New Band CINR | variable |  |
| }else{ |  |  |
| Differential CINR | variable | It includes the differential of CINR of the selected bands by BS (increment: 1 and decrement: 0 with a step of 1 dB). |
| } |  |  |
| } |  |  |

New Band CINR : there are 60 bands and CPE can report the CINR of the maximum 16 bands. 60 bands are grouping into 15 groupbands including the 4 bands which are indicated by 2 bits (00, 01, 10 and 11). If CPE want to use specific band in the n-th groupband, the n-th LSB bit is 1 within the 15 bits and then 8bits follow this one bit. 2bits indicate the band in the n-th groupband, 5 bits represent the channel quality information and the last one bit indicate whether other band is reported in the n-th groupband or not. If last one bit is 1, there is other band and another 8bits follow this bit.