IEEE P802.22
Wireless RANs

| MAC message for AMC subchannel |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-05-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Chang-Joo Kim | ETRI | Korea | +82-42-860-1230 | cjkim@etri.re.kr |
| Myung-Sun Song | ETRI | Korea | +82-42-860-5046 | mssong@etri.re.kr |
| Gwang-Zeen Ko | ETRI | Korea | +82-42-860-4862 | gogogo@etri.re.kr |
| Sung-Hyun Hwang | ETRI | Korea | +82-42-860-1133 | shwang@etri.re.kr |
| Jung-Sun Um | ETRI | Korea | +82-42-860-4844 | korses@etri.re.kr |

Abstract

This document is shown the MAC message to support AMC subchannel.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- MAC Message for AMC subchannel allocation

In clause 6.8.2 Downstream Map(DS-MAP), insert the following red colored rows in the Table 46 DS-MAP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| DS-MAP_Message_Format() { |  |  |
| Management Message Type = 1 | 8 bits |  |
| Synchronization Field | 16 bits | Error: Reference source not found |
| DCD Count | 8 bits | Matches the value of the configuration change count of the DCD, which describes the downstream burst profiles that apply to this map. |
| BS ID | 48 bits | See Error: Reference source not found |
| AMC_Subchannel | 1 bits | Indicates whether the adjacent subcarrier permutation is used in the downstream or not. 0= AMC subchannel is not used 1= AMC subchannel is used |
| if (AMC_Subchannel){ |  |  |
| DS_BAND_MAP | Variable | See Table xx |
| } |  |  |
| Begin PHY Specific Section { |  |  |
| for (i = 1; i  n; i++) { |  |  |
| DS-MAP_IE() | Variable | PHY specific (Error: Reference source not found) |
| } |  |  |
| } |  |  |
| Padding Nibble | 4 bits |  |
| } |  |  |

In clause 6.8.2 Downstream Map(DS-MAP), create  the following new table for downstream band map after Table 47 Synchronization field

Table xx    DS_BAND_MAP

| Syntax | Size | Notes |
| --- | --- | --- |
| DS_BAND _MAP () { |  |  |
| New format Indicator | 1bit | 0 = Use the latest DS_BAND_MAP 1 = New DS_BAND_MAP It can be 1 or 0 on the first frame, but only 0 on the other frames in the superframe. |
| if(New format Indicator) |  |  |
| { |  |  |
| Nband | 6bits | The total number of AMC bands which are selected for AMC subchannel |
| if(Nband < 10 or Nband > 50){ |  |  |
| for(i=0 ; i<Nband ; i++) |  |  |
| BandIndex | 6bits | If Nband is less than 10, each BandIndex indicates the index of the selected AMC band for AMC subchannels. If Nband is larger than 50, it indicates the index of the diversity band not using for AMC subchannels. |
| }else{ |  |  |
| BandBITMAP | 60bits | The n-th LSB represents the n-th band. If n-th band is AMC band for AMC subchannel, it is 1. |
| } |  |  |
| } |  |  |

In clause 6.8.2.1 DS-MAP-IE, insert the following red colored rows in the Table 48 DS-MAP information elements

| Syntax | Size | Notes |
| --- | --- | --- |
| DS-MAP_IE() { |  |  |
| DIUC | 4 bits | Error: Reference source not found |
| If (DIUC == 15) |  |  |
| Extended DIUC Dependent IE | Variable | Error: Reference source not found |
| else { |  |  |
| If (INCLUDE_CID) { |  | The DS-MAP starts with INCLUDE_CID =0. INCLUDE_CID is toggled between 0 and 1 by the CID-SWITCH_IE() – see Error: Reference source not found |
| N_CID | 7 bits | Number of CIDs assigned for this IE |
| for (i=0; i<N_CID; i++) |  |  |
| CID | 16 bits | Unicast/Multicast/Broadcast CID. In case of a dummy allocation, the BS shall use the Padding CID – see Error: Reference source not found |
| } |  |  |
| AMC_Subchannel | 1bit | Indicates whether AMC subchannel is used for the burst of this CID 0= AMC subchannel is not used 1= AMC subchannel is used |
| If(AMC_Subchannel){ |  |  |
| If(frame index == 0){ |  | The same band should be used during the superframe. |
| Selected Band BITMAP | variable | It represents the index of bands to be allocated for this CID among the all AMC bands selected for AMC subchannel. It consists of the bits with same value of Nband. The n-th bit is 1 if n-th band is allocated. |
| } |  |  |
| }else{ |  |  |
| Channel Offset | 7 bits |  |
| Number of Channels | 7 bits |  |
| Slot Offset | 8 bits |  |
| Number of Slots | 8 bits | In number of PHY symbols. Specification of the duration in the downstream allows for the BS to introduce gaps in the downstream data transmission, and hence provide better support for coexistence. |
| } |  |  |
| Boosting | 3 bits | 000: normal (not boosted) 001: +6dB 010: -6dB 011: +9dB 100: +3dB 101: -3dB 110: -9dB 111: -12dB |
| } |  |  |
| If (!byte_boundary) |  |  |
| Padding Nibble | 4 bits |  |
| } |  |  |

In clause 6.8.4 Upstream Map(US-MAP), insert the following red colored rows in the Table 58 US-MAP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| US-MAP_Message_Format() { |  |  |
| Management Message Type = 3 | 8 bits |  |
| Upstream Channel ID | 8 bits | The identifier of the upstream channel to which this message refers. |
| UCD Count | 8 bits | Matches the value of the Configuration Change Count of the UCD, which describes the upstream burst profiles that apply to this map. |
| Allocation Start Time | 8 bits | Effective start time (in MAC slots) of the upstream allocation defined by the US-MAP |
| AMC_Subchannel | 1 bits | Indicates whether the adjacent subcarrier permutation is used in the upstream or not. 0= AMC subchannel is not used 1= AMC subchannel is used |
| If (AMC Subchannel){ |  |  |
| US_BAND_MAP | Variable | See Table yy |
| } |  |  |
| Begin PHY Specific Section { |  |  |
| for (i = 1; i  n; i++) { |  |  |
| US-MAP_IE() | Variable | PHY specific (Error: Reference source not found) Define upstream bandwidth allocations. Each US-MAP message shall contain at least one IE that marks the end of the last allocated burst. |
| } |  |  |
| } |  |  |
| If (!byte_boundary) |  |  |
| Padding Nibble | 4 bits |  |
| } |  |  |

In clause 6.8.4 Upstream Map(US-MAP), create the following new table for upstream band map after Table 58 US-MAP message format

Table yy    US_BAND_MAP

| Syntax | Size | Notes |
| --- | --- | --- |
| US_BAND _MAP() { |  |  |
| New format Indicator | 1bit | 0 = Use the latest US_BAND_MAP 1 = New US_BAND_MAP It can be 1 or 0 on the first frame, but only 0 on the other frames in the superframe. |
| if(New format Indicator) |  |  |
| { |  |  |
| Nband | 6bits | The total number of AMC bands selected for AMC subchannel |
| if(Nband < 10 or Nband > 50){ |  |  |
| for(i=0 ; i<Nband ; i++) |  |  |
| Band Index | 6bits | If Nband is less than 10, each BandIndex indicates the index of the selected AMC band for AMC subchannels. If Nband is larger than 50, it indicates the index of the diversity band not using for AMC subchannels. |
| }else{ |  |  |
| BandBITMAP | 60bits | The n-th LSB represents the n-th band. If n-th band is AMC band for AMC subchannel, it is 1. |
| } |  |  |
| } |  |  |
| } |  |  |

In clause 6.8.4.1 US-MAP-IE, insert the following red colored rows in the Table 59 US-MAP information elements

Table 59   US_MAP_IE

| Syntax | Size | Notes |
| --- | --- | --- |
| US-MAP_IE() { |  |  |
| CID | 16 bits | Error: Reference source not found |
| UIUC | 4 bits | Error: Reference source not found |
| If ((UIUC  0) && (UIUC  5)) { |  |  |
| Channel Offset | 7 bits |  |
| Number of Channels | 7 bits |  |
| Slot Offset | 8 bits |  |
| Number of Slots | 8 bits |  |
| Reserved | 2 bits |  |
| If (UIUC ≤ 1) { |  |  |
| US-MAP CBP Channel IE | 16 bits | 6.8.4.1.2.3 |
| } |  |  |
| } else if (UIUC == 15) { |  |  |
| US_Extended_IE() | Variable | Error: Reference source not found |
| } else if (UIUC == 14) { |  |  |
| CDMA_Allocation_IE() | 32 bits | Error: Reference source not found |
| } else { |  |  |
| AMC_Subchannel | 1bit | Indicates whether AMC subchannel is used for the burst of this CID 0= AMC subchannel is not used 1= AMC subchannel is used |
| If(AMC_Subchannel){ |  |  |
| If(frame==0){ |  | The same band should be used during the superframe. |
| SelectedBandBITMAP | variable | It represents the index of bands to be allocated for this CID among the all AMC bands selected for AMC subchannel. It consists of the bits with same value of Nband. The n-th bit is 1 if n-th band is allocated. |
| }else{ |  |  |
| Duration | 8 bits | In number of MAC slots |
| } |  |  |
| MDP | 1 bit | Measurement Data Preferred Used by the BS to indicate to the CPE that this upstream allocation is to be preferably used by the CPE for the specific purpose of reporting back any measurement data. The measurement data to be reported is in connection to the specified Transaction ID. In case the CPE does not have anything to report, it can use this allocation for any other data. This is useful, for example, after a quiet period. 0 = Measurement data not required (default) 1 = Measurement data preferred |
| MRT | 1 bit | Measurement Report Type In case MDP == 1, this field indicates which type of report the BS wants the CPE to send back. 0 = Detailed (see Error: Reference source not found thru Error: Reference source not found) 1 = Consolidated (see Error: Reference source not found) |
| CMRP | 1 bit | Channel Management Response Preferred Used by the BS to indicate to the CPE that this upstream allocation is to be used for confirming or not the receipt of the channel management command with the Transaction ID specified. 0 = Channel management response not required (default) 1 = Channel management response required |
| Transaction ID | 16 bits | The transaction ID of the corresponding channel management or measurement request for which a response/report is required by the BS. |
| Preamble Present | 1 bit | Determines whether CPE shall send preamble before any data transmission 0 = Preamble shall not be used (default) 1 = Preamble shall be used |
| } |  |  |
| If (!byte_boundary) |  |  |
| Padding Nibble | 4 bits |  |
| } |  |  |

In clause 6.8 Management Messages, insert the following red colored rows in the Table 41 Management messages after the AAS-CFB-RSP:

| type | Message | Description | Connection |
| --- | --- | --- | --- |
| 60 | CMR-REQ | Channel measurement Report Request | Primary Management |
| 61 | CMR-RSP | Channel measurement Report Response | Primary Management |

Create the following clause and tables after the clause 6.8.29 AAS Channel Feedback Request/Response (AAS-CFB-REQ/RSP)

6.8.30 Channel measurement Report for Request/Response (CMR-REQ/RSP)

BS can request the channel quality information with a CINR measurement of bands to specific CPE in order to allocate the AMC subchannel. There can be two types of CINR measurements to request. The type of New CINR is used for initially allocating the AMC subchannel to specific CPE which will report the CINR measurement of favorable bands according to the CMR-REQ. The differential CINR is used for knowing the difference between previous and current CINR of bands which are already assigned to specific CPE for AMC subchannel.

Table yy CMR-REQ Message format

| Syntax | Size | Notes |
| --- | --- | --- |
| CMR_REQ_Message_format () { |  |  |
| Management Message Type = 60 | 8bits |  |
| Report Type | 1bit | 0 = New CINR 1 = Differential CINR of the assigned bands |
| Number of bands | 4bits | The number of bands for CPE to report the CINR measurement |
| Reserved | 3bits |  |
| } |  |  |

To respond to the CMR-REQ or request the AMC subchannel, CPE can report the CMR-RSP message containing the channel quality information with either the new CINR measurement of the several favorable bands or the differential CINR measurement of the already assigned bands. The Mean for CINR shall be reported in units of dB and quantized in 1dB increments, ranging from a minimum of -10dB to a maximum of 53dB.

Table zz CMR_RSP Message format

| Syntax | Size | Notes |
| --- | --- | --- |
| CMR_RSP_Message_format () { |  |  |
| Management Message Type = 61 | 8bits |  |
| Report Type | 1bit | 0 = New CINR 1 = Differential CINR of the assigned bands |
| If(ReportType = 1){ |  |  |
| New CINR | variable |  |
| }else{ |  |  |
| Differential CINR | variable | The differential CINR of each assigned band will be reported by using two bits from lower band (increment: 01, decrement: 10 and equality: 00 with a step of 1 dB). |
| } |  |  |
| } |  |  |

New CINR : there are 60 bands and CPE can report the CINR of the maximum 16 bands. 60 bands are grouping into 15 groupbands including the 4 bands which are indicated by 2 bits (00, 01, 10 and 11). If CPE want to use specific band in the n-th groupband, the n-th LSB bit is 1 within the 15 bits and then 9 bits follow this one bit. 2bits indicate the band in the n-th groupband, 6 bits represent the channel quality information and the last one bit indicate whether other band is reported in the n-th groupband or not. If last one bit is 1, there is other band and another 8bits follow this bit.