IEEE P802.22
Wireless RANs

| Limited ‘Spectrum Management’ functions at the CPE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-06-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Gerald Chouinard | CRC | 3701 Carling Avenue, Ottawa, Ontario, Canada K2H 8S2 | 1-613-998-2500 | gerald.chouinard@crc.ca |

Abstract

This contribution is an attempt at listing the automated functions that would exist at the CPEs which are related to the ‘Spectrum Management’ functions residing at the Base Stations.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Limited ‘Spectrum Management’ functions at the CPE

The operation of the CPE should be under the complete control of the Base Station so that the WRAN operation has maximum control of its network operation and its potential impact on the operation of the incumbent services in the band.  However, there will be instances where the CPE will need to control its behavior locally before the base station can take over.  These are:

- at the initial turn-on of the CPE before the association with the base station is established;

- when the CPE looses contact with its base station; and

- during idle time when the base station has not attributed any specific task to the CPE.

- These are the only cases where local intelligence related to spectrum sensing akin to a ‘spectrum management’ function at the base station will be required at the CPE.  The CPE local spectrum sensing should be considered more as a set of CPE MAC routines rather than ‘spectrum manager’ functions which should reside at the base station.  The functionality of the CPE local spectrum sensing for these three specific cases is described below.

- 1- Initial turn-on

At initial turn-on and self-test, the CPE sweeps all the TV channels that are within its range of operation plus 15 channels below and above this range, or up to the extent of the relevant TV band (low-VHF, high-VHF and UHF).  Normally, the scanning performs  a RSSI measurement in each TV channel and a fast identification of the type of signal (in presence of sufficiently high RF signal).  The result of the measurement should be stored locally so that it can later be sent to the base station when association is established and upon request from the BS.

For those TV channels where the RF energy was found to be insufficient to provide for a reliable RSSI measurement, fine sensing will be applied to determine the presence of broadcast incumbents and their type.  Again, the information on whether there is a broadcast incumbent in a channel or not and its type will be stored locally so that it can later be sent to the base station when association is established and upon request from the BS.

The CPE should then sweep the TV channels where WRAN type emissions with sufficient RF energy was found.  For each of these TV channels, the CPE will try to get information from the local WRAN base station by capturing the superframe header.  Depending on the implementation, the information on the local WRAN operation, as determined by the Station Management environment at the base station may be presented at the local interface of the CPE so that it could be displayed on the user terminal to allow for a choice among available local WRAN networks by the potential subscriber.

Finally, the CPE may attempt to associate with the WRAN service selected by the user through the initial CDMA ranging burst.  Upon successful association, the base station may request the CPE to send the results of the initial sensing and, in turn, it will download the list of available channels including the current channel on which the association has taken place and the list of back-up channels available in the area.  The length of this list could be set (beyond a minimum of, say, 3 channels) by the WRAN operator on the basis of the local channel availability and the dynamic nature of the environment.  However, for the purpose of developing an air-interface standard, a maximum number of channels will need to be determined for this list.

- 2- Loss of contact with base station

When the CPE looses contact with its base station, local intelligence should make sure that sufficient attempts are made at trying to re-connect with the base station while avoiding any potential interference to broadcast incumbent.

An initial sensing of the channel should take place to identify whether or not a WRAN RF signal is still present.  If this is the case, attempts to re-associate should be made through the BW Request opportunistic burst or upon CPE specific invitation by the BS.  If this does not work, the re-association should start from an earlier stage with the CDMA ranging burst..  If the sensing results in the presence of a broadcast incumbent or an absence of RF signal, then the CPE will, within TBD seconds, cyclically select the next TV channel on its back-up list and try to capture the superframe header to lock to the base station on this new channel.

If the superframe capture is not successful, then, the same process described in the previous paragraph for the re-association on the main channel will take place.  If the superframe capture is again unsuccessful, then the CPE will, within TBD seconds, cyclically select the next TV channel on its backup list and repeat the process until a successful superframe capture.

Upon capture of the superframe, the CPE will synchronize with the BS and acquire the frame header.  The CPE will then analyze the downstream channel characteristics, as revealed by the frame preamble and the pilot carriers to allow it to respond to the CDMA ranging invitation when the spectral characteristics of the initial ranging burst is favorable   The CPE will then attempt association by sending a CDMA ranging burst at the appropriate time.  Upon successful association, the base station will send the updated list of available channels.

- 3- CPE idle time

If a CPE is not allocated any time to transmit during the upcoming sub-frame from the US-MAP and has not been asked to do specific in-band or out-of-band sensing, the CPE should have the necessary routines to automatically sense, in turn, the TV channels in its ‘list of available channels’ and report to the base station any presence of incumbents in these channels using the UCS opportunistic window in the following frame.  The sensing scheme that will be used may need to meet the intra-frame sensing constraint to avoid the sensing to continue into a subsequent frame where the CPE may be asked to transmit.  Inter-frame sensing could also be used but at the risk of an invalid result if the CPE is requested to transmit during the multi-frame period.  Note that the sensing of the operating channel needs to be managed by the base station since appropriate quiet periods will need to be scheduled to empty the channel from any WRAN operation.

If a coexistence window at the end of the frame is signaled by the base station in the US-MAP, the CPE will automatically shorten its sensing window and its WRAN receiving path will be enabled during that period to acquire an asynchronous CBP burst.  Upon successful acquisition of a CBP burst, the CPE will send a bandwidth request using the BW-Request opportunistic window and, when scheduled by the BS, send the information extracted from the CBP burst (i.e., the SCH and CBP payload, and the information needed for the geolocation (i.e., phase of the key carriers and the time difference between receive and transmit).

### Extension to the adjacent channels

Since the sensing on the adjacent channels (N+/-1) will also be needed, the CPE should have the capability of re-tuning its sensing RF path to the channels adjacent to those contained in its ‘lisrt of available channels’ during the idle sub-frame periods and repeat the sensing process described above for the sensing of the channels in the ‘list of available channels’.  As explained above, a RSSI measurement should first take place with a fast RF signal type identification.  If insufficient RF signal is found, then an intra-frame sensing scheme should take place to identify possible broadcast incumbents. Inter-frame sensing can be used as long as the CPE is not being asked to transmit during the sensing interval.

The advantage here is that if successful sensing can be done by CPEs during their idles periods, the need for more formal sensing with the scheduled quiet periods is reduced.  Note that such formal channel sensing is needed in the case of the channels adjacent to the operating channels since since appropriate quiet periods will need to be scheduled by the base station to sense these adjacent channels

Similarly, the CPE could tune its WRAN RF path to an adjacent channel during its idle period to try to capture a CBP burst coming from an adjacent channel.  If successful, the CPE, once reverted to its normal operating TV channel would request an upstream time slot to relay the CPB information to its base station so that synchronization of quiet periods among adjacent channels can take place for proper sensing.  It would be desirable that the coexistence windows be scheduled at the same time among the base stations operating on adjacent channels in the same area.  This way, the CPE would decide to re-tune its WRAN RF path and open its circuits for the asynchronous capture of the CBP burst only when the coexistence window is scheduled for its own channel operation.

_______________________________________

A mechanism has to be provided to prevent a CPE from trying to re-associate continuously if the BS refuses to re-associate with it.

There is still a concern as to how the CPE will know when a signal was emitted by an incumbent versus by a HDTV baby monitor or some similar unlicensed devices if they are allowed in the TV bands, and how this will not be made to turn into a big gaping hole to allow denial of service attacks.

Is the UCS notification window only for alerting the BS that an incumbent is found on the operating channel or it can be used to signal incumbents on other channels in the ‘list of available channels’?