IEEE P802.22
Wireless RANs

| Text on Cyclostationary Feature Detector – For Informative Annex on Sensing Techniques |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-06-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Hou-Shin Chen | Thomson | Two Independence Way Princeton, NJ 08540 |  | hou-shin.chen@thomson.net |
| Wen Gao | Thomson | Two Independence Way Princeton, NJ 08540 |  | wen.gao@thomson.net |

Abstract

This document contains the text on the cyclostationary feature detector for the informative Annex on sensing techniques.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

It has been recognized that many random time series encountered in the field of signal processing are more appropriately modeled as cyclostationary, rather than stationary, duo to the underlying periodicities in these signals [1]. Another reason to use cyclostationary signal model is that random signals such as white Gaussian noise are not cyclostationary. Thus, cyclostationarity provides us a way to separate desired signals from noise.

# Cyclostationary Feature of ATSC DTV Signals

According to [2], DTV data are VSB modulated.  Before VSB modulation, a constant of 1.25 is added to the 8-level pulse amplitude modulated signal (8-PAM). Therefore, there is a strong pilot tone on the power spectrum density (PSD) of the ATSC DTV signal. Let s(t) be this pilot tone signal which is a sinusoidal signal in the time domain and further assume that this strong pilot tone is located at frequency f0 , i.e.,

s(t)=2Pcos(2πf0t+θ)⊗h(t)	(1)

where P and θ are the power and  the initial phase of the sinusoidal function respectively. The function h(t) is the channel impulse response and ⊗ is the convolution operator. The received signal must contain the signal

x(t) = s(t)e-j2πνt + w(t) 	(2)

where w(t) is the additive white Gaussian noise (AWGN) and ν is the amount of frequency offset in the unit of Hz. We will assume that w(t) is zero-mean with autocorrelation function Rw(τ) = E[w(t)w*(t-τ)] = σ2δ(τ). The cyclic spectrum of the received signal must contain the cyclic spectrum of x(t) which is given by

Sxα(f)=P2δ(f−f0−ν)+δ(f+f0+ν)H(f)2+σ2 for α=0P2δ(f)H(f−f0−ν)H(f+f0+ν) for α=±2(f0+ν)	(3)

where H(f) is the frequency response of the channel. The parameter α is the cyclic frequency. From (3), ideally, the noise does not contribute to the cyclic spectrum of x(t) when cyclic frequencies α = ±(2f0+ν). Thus, performing spectrum sensing by detecting the peaks on the cyclic spectrum of the signal should be better than that of using PSD.

# Initial Processing of Received Signal

The RF ATSC DTV signal for a given DTV channel is first filtered and down-converted to a given intermediate frequency (IF). The IF signals are usually sampled at a rate that is multiple times of the symbol rate. The samples can be expressed as

y[n] = x[n] + w[n]	(1)

where x[n] are samples of the transmitted DTV signal. The noise w[n] is assumed to be zero-mean with variance σ2. Then, y[n] is used to perform cyclostationarity based sensing algorithms.

# Test Statistic Using Cyclic Spectrum

First, we use a proper narrow band-pass filter to filter y[n] and obtain a small frequency bands which contains the pilot tone. Then, y[n] is down-converted to have lower central frequency. Note that we will perform down-conversion for multiple times. Let zl[n] denote the down-converted signal which has a central frequency fIF+lfΔ. Note that fΔ is chosen to be small, which depends on the sample rate and FFT size used in computation of the cyclic spectrum. We will decimate zl[n] by a proper decimation ratio D to obtain zlD[n] which has a lower sampling rate. Finally, we compute the cyclic spectrum by

Szα(k)=12L+11Δtl=−LLZlD(k+α2)⋅ZlD(k−α2)	(2)

where

ZlD(k)=n=0N−1zlD[n]e−j2πknN.	(3)

Note that in Eq. (2), we use a spectral smoothing method by averaging 2L+1 times to obtain cyclic spectrum. In Eq. (3), N is the number of time samples used to compute short-term Fourier transform. The parameter Δt is the length of data segment which equals to (N-1)Ts where Ts is the time-sampling increment of the signal zlD[n]. Finally, we use

T=maxαSzα(0)	(4)

as our decision statistic.  The range of α depends on  fIF and frequency offset.

# Simulation Results

The performances of the cyclostationarity based algorithm were demonstrated using computer simulations according to the spectrum sensing simulation model [3]. The band-pass filter used to filter the pilot tone has a bandwidth of 40 KHz and fIF is 17 KHz. The decimation factor is 200 and the decimation filter is a ±50 KHz low-pass filter. The size of FFT is 2048. The parameter L in Eq. (2) is 2 and fΔ is set to be half of the carrier spacing divided by 2L+1. We set the false alarm rate equalling to 0.1. The 12 reference Capture Data files are simulated. The required SNR for 0.1 of misdetection rate are given in Table 1, 2, and 3 for the best, worst and average case of the 12 reference Capture Data files. The parameter Δ in the tables is the amount of the noise uncertainty.

| Sensing Time/Sequence | Δ=0 dB | Δ=0.5 dB | Δ=1 dB |
| --- | --- | --- | --- |
|  | Required SNR (dB) |  |  |
| 19.03 ms | -31 | -30.5 | -30 |

Table 1: Required SNR for the cyclostationary feature detector (Best case).

| Sensing Time/Sequence | Δ=0 dB | Δ=0.5 dB | Δ=1 dB |
| --- | --- | --- | --- |
|  | Required SNR (dB) |  |  |
| 19.03 ms | -21 | -20.5 | -20 |

Table 2: Required SNR for the cyclostationary feature detector (Worse case).

| Sensing Time/Sequence | Δ=0 dB | Δ=0.5 dB | Δ=1 dB |
| --- | --- | --- | --- |
|  | Required SNR (dB) |  |  |
| 19.03 ms | -25 | -24.5 | -24 |

Table 3: Required SNR for the cyclostationary feature detector (Average).

.

References

- W. A. Gardner, "Exploitation of Spectral Redundancy in Cyclostationary Signals," IEEE Signal Processing Magazine, Vol. 8, No. 2, pp. 14-36, April 1991.

- Advanced Television Standards Committee, ATSC Digital Television Standard, ATSC A/53E, April 2006.

- S. Mathur, R. Tandra, S. Shellhammer, and M. Ghosh, "Initial Signal Processing of Captured DTV Signals for Evaluation of Detection Algorithms," IEEE 802.22-06/0158r4, Sept. 2006.