IEEE P802.22
Wireless RANs

| Text on covariance based sensing for wireless microphone – For Informative Annex on Sensing Techniques |  |  |  |  |
| --- | --- | --- | --- | --- |
| Last Updated - Date: 2007-07-06 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Yonghong Zeng | Institute for Infocomm Research | 21 Heng Mui Keng Terrace, Singapore 119613 | 65-68748211 | yhzeng@i2r.a-star.edu.sg |
| Ying-Chang Liang | Institute for Infocomm Research | 21 Heng Mui Keng Terrace, Singapore 119613 | 65-68748225 | ycliang@i2r.a-star.edu.sg |

Abstract

This document contains the text on the covariance based sensing for wireless microphone in the informative annex on sensing techniques.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Document Policy and Procedures: The contributor is familiar with the IEEE 802 Document Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of document(s), including document applications, provided the IEEE receives assurance from the document holder or applicant with respect to documents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of document information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if documented technology (or technology under document application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Document Committee Administrator at <patcom@ieee.org>.

- Covariance based sensing algorithms

Let y(t) be the continuous time received signal. Assume that we are interested in the frequency band with central frequency fcand bandwidthW. We sample the received signal y(t) at a sampling ratefs. Let Ts=1fsbe the sampling period. The received discrete signal is thenx(n)=y(nTs). There are two hypothesises:H0: signal not exists; and H1: signal exists. The received signal samples under the two hypothesises are therefore respectively as follows:

H0:x(n)=η(n)

H1:x(n)=s(n)+η(n),

wheres(n) is the transmitted signal passed through a wireless channel (including fading and  multipath effect), andη(n) is the white noise samples. Note thats(n)can be the superposition of multiple signals. The received signal is generally passed through a filter. Let f(k),k=0,1,...,Kbe the filter with k=0Kf(k)2=1. After filtering, the received signal is turned to

x(n)=k=0Kf(k)x(n−k),n=0,1,...

Let

s(n)=k=0Kf(k)s(n−k),n=0,1,...

η(n)=k=0Kf(k)η(n−k),n=0,1,...

Then

H0:x(n)=η(n)

H1:x(n)=s(n)+η(n)

Note that here the noise samples  are correlated. If the sampling rate  is larger than the signal bandwidth W, we can down-sample the signal. Let be the down-sampling factor. If the signal to be detected has a narrower bandwidth thanW, it is better to choose. For notation simplicity, we still use to denote the received signal samples after down-sampling, that is,

Choose a smoothing factor L and define

x(n)=[x(n)x(n−1)...x(n−L+1)]T,n=0,1,...,Ns−1

A suggested value for L is around 10. Define a matrix as

LetG=HHH. Decompose the matrix intoG=Q2, where Q is a L×LHermitian matrix. The matrix  is not related to signal and noise and can be computed offline. If analog filter or both analog filter and digital filter are used, the matrix should be revised to include the effects of all the filters. In general, can be obtained to be the covariance matrix of the received signal, when the input signal is white noise only (this can be done in laboratory offline).  The matrix G and Q are computed only once and only Q is used in detection.

Denote the statistical covariance matrix of the received signal as

Rx=E(x(n)x(n)H)

Then

Rx=Rs+ση2G

where Rs is the statistical covariance matrix of the signal (including fading, multipath and filtering) and ση2 is the noise variance.

Define

Rx=Q−1RxQ−1

Rs=Q−1RsQ−1

Then

Rx=Rs+ση2I

If there is no signal, thenRs=0. Hence the off-diagonal elements of Rx are all zeros. If signal presents, Rsis almost surely not a diagonal matrix. Hence, some of the off-diagonal elements of Rx should not be zeros. Denote the elements of the matrix by rnm.

Let

T1=1Ln=1Lm=1Lr, T2=1Ln=1Lr

T3=1Ln=1Lm=1Lr2, T4=1Ln=1Lr2

Then if there is no signal,T1=T2, and T3=T4. If there is signal, T1>T2, and T3>T4. We obtain two detection methods as follows.

Method 1: The covariance absolute value (CAV) detection

Step 1. Sample and filter the received signal as described above.

Step 2. Choose a smoothing factor L and compute the threshold. is chosen to meet the requirement for the probability of false alarm.

Step 3. Compute the auto-correlations of the received signal

λ(l)=1Nsm=0Ns−1x(m)x(m−l),l=0,1,...,L−1,

and form the sample covariance matrix as

R(Ns)=λ(0)λ(1)...λ(L−1)λ(1)λ(0)...λ(L−2)⋮⋮⋮⋮λ(L−1)λ(L−2)...λ(0)

Note that the sample covariance matrix is Hermitian and Toeplitz.

Step 4. Transform the sample covariance matrix to obtain

R(Ns)=Q−1R(Ns)Q−1

Step 5. Compute

T1(Ns)=1Ln=1Lm=1Lr(Ns)

T2(Ns)=1Ln=1Lr(N)

where rnm(N) are the elements of the sample covariance matrix.

Step 6. Determine the presence of the signal based on T1(Ns),T2(Ns)and the threshold: if , signal exists; otherwise, signal not exists

Method 2: The covariance Frobenius norm (CFN) detection

Step 1. Sample and filter the received signal as described above.

Step 2. Choose a smoothing factor L and compute the threshold.  is chosen to meet the requirement for the probability of false alarm.

Step 3. Compute the auto-correlations of the received signal

λ(l)=1Nsm=0Ns−1x(m)x(m−l),l=0,1,...,L−1,

and form the sample covariance matrix as

R(Ns)=λ(0)λ(1)...λ(L−1)λ(1)λ(0)...λ(L−2)⋮⋮⋮⋮λ(L−1)λ(L−2)...λ(0)

Note that the sample covariance matrix is Hermitian and Toeplitz.

Step 4. Transform the sample covariance matrix to obtain

R(Ns)=Q−1R(Ns)Q−1

Step 5. Compute

T3(Ns)=1Ln=1Lm=1Lr(Ns)2

T4(Ns)=1Ln=1Lr(N)2

where rnm(N) are the elements of the sample covariance matrix.

Step 6. Determine the presence of the signal based on T3(Ns),T4(Ns)and the threshold: if , signal exists; otherwise, signal not exists

- Performance of the algorithms for wireless microphone signal

The threshold in CAV or CFN is determined by the ratio  or and the required probability of false alarm (). When there is no signal, the ratio is not related to noise power at all. Hence, it does not have the noise uncertainty problem. Both methods do not need noise power estimation. The performances of the methods are not only related to SNR but also related to signal statistic properties.

In the following the performances of the methods are given based on simulations, where.  The required SNR is the lowest SNR which meets the requirement of  and the probability of misdetection . Note that the SNR is measured in one TV channel with 6 MHz bandwidth. The performance of the methods can always be improved by increasing the sensing time.

(1) Simulations at IF band and no down-sampling (). The simulation is based on the FM modulated signal defined as

where fc =5.381119 MHz is the central frequency, fΔ=100kHz is the frequency deviation, and wm(τ) is the source signal. The signal and white noise are passed through a filter centred at and with bandwidth 6 MHz.

| method | 4ms | 10ms |
| --- | --- | --- |
| CAV | -18.5dB | -20.4dB |
| CFN | -18.4dB | -20.4dB |

Table 1: Required SNR for wireless microphone signal detection

(2) Simulations at baseband and down-sampling (). For wireless microphone detection, choosing a down-sampling factor  gives better performance. Table 2 gives the simulation results for wireless microphone signals (average on 3 types of signals: soft speaker, loud speaker and silence [2]). The settings and procedures for the simulation are as follows. Baseband microphone signal is generated. The signal is sampled at sampling rate 12 MHz. The signal is then filtered with a low-pass filter with 6 MHz bandwidth. The signal is passed through a multipath simulator (Rayleigh fading with 5 taps). White noise samples (sampling rate 12 MHz) are generated and passed through the same filter.  The signal and scaled noise are added together and then down-sampled (decimated) by a factor.

| method | 4ms | 10ms |
| --- | --- | --- |
| CAV | -20.8dB | -22.8dB |
| CFN | -20.8dB | -22.8dB |

Table 2: Required SNR for wireless microphone signal detection (baseband and down-sampling)

References

- Yonghong Zeng and Ying-Chang Liang, “Covariance based signal detections for cognitive radio”, IEEE DySpan, 2007.

- Chris Clanton, Mark Kenkel and Yang Tang, “Wireless Microphone Signal Simulation Method,” IEEE 802.22-07/0124r0, March 2007.