IEEE P802.22
Wireless RANs

| Text on Dual FPLL pilot sensing – For Informative Annex on Sensing Techniques |  |  |  |  |
| --- | --- | --- | --- | --- |
| Last Updated - Date: 2007-06-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Gene Turkenich Vasanth Gaddam | Philips Research North America | 345 Scarborough Road, Briarcliff Manor, NY 10510 | (914) 945-6370 (914) 945-6424 | gene.turkenich@philips.com vasanth.gaddam@philips.com |

Abstract

This document contains the text on the Dual FPLL pilot sensing techniques for the informative annex on sensing techniques.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Document Policy and Procedures: The contributor is familiar with the IEEE 802 Document Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of document(s), including document applications, provided the IEEE receives assurance from the document holder or applicant with respect to documents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of document information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if documented technology (or technology under document application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Document Committee Administrator at <patcom@ieee.org>.

- Dual FPLL pilot sensing algorithm

The Dual FPLL sensing techniques described in this section are non-blind (ATSC-specific) sensing techniques that meet the sensing sensitivity requirements of 802.22 and hence are classified as a fine sensing technique.

The sensor system, presented in Fig 1, employs two Frequency Tracking Blocks (FTBs). These blocks are

designed to track an a priori known  frequency of the ATSC pilot, fpilot ,  within a signal source, an Intermediate Frequency (IF) at the TV tuner output. The outputs, F1 and F2 , are frequencies, that FTBs are locked into at the given moment. In the absence of the pilot, or when the pilot energy is insufficient, the outputs, F1 and F2 , maintain their initial input preset values, Fini_1   and  Fini_2,   respectively. These preset values are selected to be:

Fini_1   =     fpilot   + 30 kHz;

Fini_2   =     fpilot  -  30 kHz;

The FTBs are initially and thereafter periodically (period = tset) preset to their respective Fini values. The time  period,  tset , is set to anywhere between 30 and 100 msec. The detection criterion is based upon a degree of convergence of these FTBs on the same spectral position of the ATSC pilot within preset time period. The detection is true, when at the end of period, tset ,

|F1 - F2|  <  Fthresh,               where Fthresh   is typically set to  20~30 kHz.

The above equation shows that, though the described sensor system is architecturally synchronous, the actual detection criterion does not require a complete fpilot   acquisition.

Fig 2 outlines a possible architecture for each of the required two FTB blocks in Fig 1.  A Frequency-Phase Locked Loop (FPLL) in Fig 2 is implemented as a version of the Costas Loop.  In this implementation the input/output angular values, φini / φinc ,  are used as a convenient functional equivalents of the frequencies, Fini / Fx , in Fig 1.  The output, φinc, needs to be fed into the Decision Block (Fig 1) as one of two required inputs.

A signal from the ADC contains a pilot (tone) nominally located at fpilot. This signal is converted into a complex form after being multiplied by two sine waves, shifted by 90 degrees, from a Numerically Controlled Oscillator (NCO). An NCO is, in essence, a sine/cosine lookup table that is fed by a modulo 2π accumulator.  The NCO input receives a phase increment, φinc , that is added to the accumulator on each system clock. In the described implementation the system clock is at a constant frequency, Fclk.  When FPLL is completely locked on the incoming tone, fpilot , the NCO advances by an angle equal to 2π/( Fclk / fpilot)   in each system clock,  Fclk,  period.

The Real (Re) and Imaginary (Im) parts of the complex signal are fed into the phase detector. The phase detector contains two identical Low Pass Filters LPFa, and an LPFb, that is needed to achieve a phase shift.  The LPFa  filters define the lock-in range of the FPLL, which is set to +-  100 kHz to accommodate possible deviations of the ATSC pilot position.  The output of the detector contains the phase error value, both a sign and a magnitude.

The Loop Filter integrates the phase error value to derive the frequency error in the Integrator branch. The frequency error is a difference between the initial preset frequency, set by , φini , and the actual pilot frequency in the incoming signal. The Proportional branch adjusts the magnitude of the phase error. The combined phase/frequency error data are added to an initial phase increment, φini ,  to arrive at φinc , a final phase increment value , that defines the NCO output frequency.