IEEE P802.22
Wireless RANs

| Text on eigenvalue based sensing – For Informative Annex on Sensing Techniques |  |  |  |  |
| --- | --- | --- | --- | --- |
| Last Updated - Date: 2007-07-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Yonghong Zeng | Institute for Infocomm Research | 21 Heng Mui Keng Terrace, Singapore 119613 | 65-68748211 | yhzeng@i2r.a-star.edu.sg |
| Ying-Chang Liang | Institute for Infocomm Research | 21 Heng Mui Keng Terrace, Singapore 119613 | 65-68748225 | ycliang@i2r.a-star.edu.sg |

Abstract

This document contains the text on the eigenvalue based sensing techniques for the informative annex on sensing techniques.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Document Policy and Procedures: The contributor is familiar with the IEEE 802 Document Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of document(s), including document applications, provided the IEEE receives assurance from the document holder or applicant with respect to documents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of document information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if documented technology (or technology under document application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Document Committee Administrator at <patcom@ieee.org>.

- Eigenvalue based sensing algorithms

Let y(t) be the continuous time received signal. Assume that we are interested in the frequency band with central frequency fcand bandwidthW. We sample the received signal y(t) at a sampling ratefs. In some applications, such as DTV detection, it is better that the sampling rate is larger than the channel bandwidthW. Let Ts=1fsbe the sampling period. The received discrete signal is thenx(n)=y(nTs). There are two hypothesises:H0: signal not exists; and H1: signal exists. The received signal samples under the two hypothesises are therefore respectively as follows:

H0:x(n)=η(n)

H1:x(n)=s(n)+η(n),

wheres(n) is the transmitted signal passed through a wireless channel (including fading and  multipath effect), andη(n) is the white noise samples. Note thats(n)can be superposition of multiple signals. The received signal is generally passed through a filter. Let f(k),k=0,1,...,Kbe the filter. After filtering, the received signal is turned to

x(n)=k=0Kf(k)x(n−k),n=0,1,...

Let

s(n)=k=0Kf(k)s(n−k),n=0,1,...

η(n)=k=0Kf(k)η(n−k),n=0,1,...

Then

H0:x(n)=η(n)

H1:x(n)=s(n)+η(n)

Note that here the noise samples  are correlated. If the sampling rate  is larger than the channel bandwidthW, we can down-sample the signal. Let  be the down-sampling factor. If the signal to be detected has a much narrower bandwidth thanW, it is better to choose. For notation simplicity, we still use to denote the received signal samples after down-sampling, that is,

Choose a smoothing factor  and define

x(n)=[x(n)x(n−1)...x(n−L+1)]T,n=0,1,...,Ns−1

A suggested value of is about 10. Define a matrix as

LetG=HHH. Decompose the matrix intoG=QQH, where Q is a L×LHermitian matrix. The matrix  is not related to signal and noise and can be computed offline. If analog filter or both analog filter and digital filter are used, the matrix should be revised to include the effects of all the filters. In general, can be obtained to be the covariance matrix of the received signal, when the input signal is white noise only (this can be done in laboratory offline).  The matrix G and Q are computed only once and only Q is used in detection.

Maximum-minimum eigenvalue (MME) detection

Step 1. Sample and filter the received signal as described above.

Step 2. Choose a smoothing factor L and compute the threshold γ to meet the requirement for the probability of false alarm.

Step 3. Compute the sample covariance matrix

R(Ns)=1Nsn=0Ns−1x(n)xH(n)

Step 4. Transform the sample covariance matrix to obtain

R(Ns)=Q−1R(Ns)Q−H

Step 5. Compute the maximum eigenvalue and minimum eigenvalue of the matrix R(Ns) and denote them as λmax and λmin, respectively.

Step 6. Determine the presence of the signal based on the eigenvalues and the threshold: if ,  signal exists; otherwise, signal not exists

Energy with minimum eigenvalue (EME) detection

Step 1. Sample and filter the received signal as described above.

Step 2. Choose a smoothing factor L and compute the threshold γ to meet the requirement for the probability of false alarm.

Step 3. Compute the sample covariance matrix

R(Ns)=1Nsn=0Ns−1x(n)xH(n)

Step 4. Transform the sample covariance matrix to obtain

R(Ns)=Q−1R(Ns)Q−H

Step 5. Compute the average energy of the received signalρ, and the minimum eigenvalue of the matrix R(Ns),λmin.

Step 6. Determine the presence of the signal: if, signal exists; otherwise, signal not exists.

- Performance of the algorithms

The threshold in MME is determined by the ratio and the required probability of false alarm (). When there is no signal, the ratio is not related to noise power at all. Hence, it does not have the noise uncertainty problem. The same is valid for EME. Both methods do not need noise power estimation. The performances of the methods are not only related to SNR but also related to signal statistic properties.

In the following the performances of the methods are given based on simulations, where. The required SNR is the lowest SNR which meets the requirement of  and the probability of misdetection . Note that the SNR is always measured in one TV channel with 6 MHz bandwidth. For DTV, the results are averaged on the 12 specified DTV signals. Note that the performance of the methods can always be improved by increasing the sensing time.

(1) Simulations for DTV  (single channel sensing). The simulation is done at IF band.

| method | 4ms | 8ms | 16ms | 32ms |
| --- | --- | --- | --- | --- |
| MME | -11.6dB | -13.2dB | -15dB | -16.9dB |
| EME | -10.5dB | -12.1dB | -14dB | -15.8dB |

Table 1: Required SNR for DTV signal detection (single channel)

(2) Simulations for wireless microphone. The wireless microphone signal is down-converted into baseband. Table 2 gives the simulation results for wireless microphone signals (average on 3 types of signals: soft speaker, loud speaker and silence [2]). The settings and procedures for the simulation are as follows. Baseband microphone signal is generated. The signal is sampled at sampling rate 12 MHz. The signal is then filtered with a low-pass filter with 6 MHz bandwidth. The signal is passed through a multipath simulator (Rayleigh fading with 5 taps). White noise samples (sampling rate 12 MHz) are generated and passed through the same filter.  The signal and scaled noise are added together and then down-sampled (decimated) by a factor.

| method | 4ms | 10ms |
| --- | --- | --- |
| MME | -21.0dB | -23.1dB |
| EME | -16.4dB | -18.4dB |

Table 2: Required SNR for wireless microphone signal detection

(3) Simulations for DTV (multiple channel sensing).  The method can be used to detect multiple consecutive channels at the same time. Here an example is given for detecting three consecutive channels at the same time. The input signal is the captured DTV signal (one channel is occupied and the remaining two channels are vacant). The signal is down-converted into baseband. The signal and noise are then filtered by a baseband filter with bandwidth 18 MHz.

| method | 4ms | 16ms |
| --- | --- | --- |
| MME | -17.5dB | -20.9dB |
| EME | -15.6dB | -19.1dB |

Table 3: Required SNR for DTV signal detection (three consecutive channels)

References

- Yonghong Zeng and Ying-Chang Liang, “Maximum-minimum eigenvalue detection for cognitive radio”, IEEE PIMRC, 2007.

- Chris Clanton, Mark Kenkel and Yang Tang, “Wireless Microphone Signal Simulation Method,” IEEE 802.22-07/0124r0, March 2007.

- Yonghong Zeng and Ying-Chang Liang, “Simulations for wireless microphone detection by eigenvalue and covariance based methods”, IEEE 802.22-07/0325r0, July 2007.