IEEE P802.22
Wireless RANs

| LDPC Coding Description for 802.22 Draft Standard |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-06-28 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Yufei Blankenship | Motorola | Schaumburg, IL, USA | 847-576-1902 | Yufei.Blankenship@ Motorola.com |
| Stephen Kuffner | Motorola | Schaumburg, IL, USA | 847-538-4158 | Stephen.Kuffner@ Motorola.com |

Abstract

This submission contains the normative and informative LDPC text to include in the 802.22 draft standard.  The text is based on the LDPC text found in [1].  Simulation results for several block sizes and modulation and coding rate options have been separately reported [2][3]. The decoding algorithms and decoding complexity are discussed in [4].

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Page v, Contents

<add text>

Annex B (informative)…………………..LDPC Direct Encoding

<add text> Clause 8.6, p. 252, line 42,

…/duo-binary convolutional turbo coding, low density parity check coding,…

<delete/add text> Clause 8.6.2, p. 254, line 1,

…there are two three additional optional modes.

<add new clause 8.6.2.3 and subclauses 8.6.2.3.1-8.6.2.3.3>

8.6.2.3 Low Density Parity Check codes (LDPC) mode (Optional)

8.6.2.3.1 Code Description

The LDPC code is based on a set of fundamental LDPC codes.  Each of the fundamental codes is a systematic linear block code.  Using the described methods in 8.6.2.3.2 Code Rate and Block Size Adjustment, the fundamental codes can accommodate various code rates and packet sizes.

Each LDPC code in the set of LDPC codes is defined by a parity-check matrix H of size m-by-n, where n is the length of the code and m is the number of parity check bits in the code. The number of systematic bits is k=n-m.

The matrix H is defined as

where Pi,j is one of a set of z-by-z permutation matrices or a z-by-z zero matrix. The matrix H is expanded from a binary base matrix Hb of size mb-by-nb, where n=z⋅nb and m=z⋅mb, with integer z  1. The base matrix is expanded by replacing each 1 in the base matrix with a z-by-z permutation matrix, and each 0 with a z-by-z zero matrix. The base matrix size nb is equal to 24.

The permutations used are circular right shifts, and the set of permutation matrices contains the zz identity matrix and circular right shifted versions of the identity matrix. Because each permutation matrix is specified by a single circular right shift, the binary base matrix information and permutation replacement information can be combined into a single compact model matrix Hbm. The model matrix Hbm has the same size as the binary base matrix Hb, with each binary entry (i,j) of the base matrix Hb replaced to create the model matrix Hbm. Each 0 in Hb is replaced by a blank or negative value (e.g., by –1) to denote a zz all-zero matrix, and each 1 in Hb is replaced by a circular shift size p(i,j)0. The model matrix Hbm can then be directly expanded to H.

Hb is partitioned into two sections, where Hb1 corresponds to the systematic bits and Hb2 corresponds to the parity-check bits, such that Hb=Hb1mb×kbHb2mb×mb.

Section Hb2 is further partitioned into two sections, where vector hb has odd weight, and Hb2 has a dual-diagonal structure with matrix elements at row i, column j equal to 1 for i=j, 1 for i=j+1, and 0 elsewhere:

Hb2=hbHb2'hb01hb1110.1⋱.⋱1.011hbmb−11.

The base matrix has hb(0)=1, hb(mb-1)=1, and a third value hb(j), 0<j<(mb-1) equal to 1. The base matrix structure avoids having multiple weight-1 columns in the expanded matrix.

In particular, the non-zero submatrices are circularly right shifted by a particular circular shift value. Each “1” in Hb2 is assigned a shift size of 0, and is replaced by a zz identity matrix when expanding to H. The two 1s located at the top and the bottom of hb are assigned shift sizes equal to 0, and the third 1 in the middle of hb is given an unpaired shift size greater than 0.

A base model matrix is defined for the largest code length (n=2304) of each code rate. The set of shifts {p(i,j)} in the base model matrix are used to determine the shift sizes for all other code lengths of the same code rate. Each base model matrix has nb=24 columns, and the expansion factor zf is equal to n/24 for code length n. Here f is the index of the code lengths for a given code rate, f=0, 1, 2, … 20.  For code length n=2304 the expansion factor is designated z0=96.

For each code rate, the shift sizes {p(f, i, j)} for a code size corresponding to expansion factor zf are derived from {p(i,j)} by scaling p(i,j) proportionally,

pf,i,j=pi,j,pi,j≤0pi,jzfz0,pi,j>0,

where x denotes the flooring function which gives the nearest integer towards -.

Rate 1/2:

-1 94 73 -1 -1 -1 -1 -1 55 83 -1 -1  7  0 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1

-1 27 -1 -1 -1 22 79  9 -1 -1 -1 12 -1  0  0 -1 -1 -1 -1 -1 -1 -1 -1 -1

-1 -1 -1 24 22 81 -1 33 -1 -1 -1  0 -1 -1  0  0 -1 -1 -1 -1 -1 -1 -1 -1

61 -1 47 -1 -1 -1 -1 -1 65 25 -1 -1 -1 -1 -1  0  0 -1 -1 -1 -1 -1 -1 -1

-1 -1 39 -1 -1 -1 84 -1 -1 41 72 -1 -1 -1 -1 -1  0  0 -1 -1 -1 -1 -1 -1

-1 -1 -1 -1 46 40 -1 82 -1 -1 -1 79  0 -1 -1 -1 -1  0  0 -1 -1 -1 -1 -1

-1 -1 95 53 -1 -1 -1 -1 -1 14 18 -1 -1 -1 -1 -1 -1 -1  0  0 -1 -1 -1 -1

-1 11 73 -1 -1 -1  2 -1 -1 47 -1 -1 -1 -1 -1 -1 -1 -1 -1  0  0 -1 -1 -1

12 -1 -1 -1 83 24 -1 43 -1 -1 -1 51 -1 -1 -1 -1 -1 -1 -1 -1  0  0 -1 -1

-1 -1 -1 -1 -1 94 -1 59 -1 -1 70 72 -1 -1 -1 -1 -1 -1 -1 -1 -1  0  0 -1

-1 -1  7 65 -1 -1 -1 -1 39 49 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1  0  0

43 -1 -1 -1 -1 66 -1 41 -1 -1 -1 26  7 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1  0

Note that the R=1/2 code is designed such that after a model matrix row permutation of [0, 2, 4, 11, 6, 8, 10, 1, 3, 5, 7, 9] consecutive rows do not intersect, which may be used to increase decoding throughput in some layered decoding architectures.

Rate 2/3 code:

2 -1 19 -1 47 -1 48 -1 36 -1 82 -1 47 -1 15 -1 95  0 -1 -1 -1 -1 -1 -1

-1 69 -1 88 -1 33 -1  3 -1 16 -1 37 -1 40 -1 48 -1  0  0 -1 -1 -1 -1 -1

10 -1 86 -1 62 -1 28 -1 85 -1 16 -1 34 -1 73 -1 -1 -1  0  0 -1 -1 -1 -1

-1 28 -1 32 -1 81 -1 27 -1 88 -1  5 -1 56 -1 37 -1 -1 -1  0  0 -1 -1 -1

23 -1 29 -1 15 -1 30 -1 66 -1 24 -1 50 -1 62 -1 -1 -1 -1 -1  0  0 -1 -1

-1 30 -1 65 -1 54 -1 14 -1  0 -1 30 -1 74 -1  0 -1 -1 -1 -1 -1  0  0 -1

32 -1  0 -1 15 -1 56 -1 85 -1  5 -1  6 -1 52 -1  0 -1 -1 -1 -1 -1  0  0

-1  0 -1 47 -1 13 -1 61 -1 84 -1 55 -1 78 -1 41 95 -1 -1 -1 -1 -1 -1  0

Note that the R=2/3 code is designed such that after a model matrix row permutation of [0, 3, 6, 1, 4, 7, 2, 5] consecutive rows do not intersect, which may be used to increase decoding throughput in some layered decoding architectures.

Rate 3/4 code:

6  38  3 93 -1 -1 -1 30 70 -1 86 -1 37 38  4 11 -1 46 48  0 -1 -1 -1 -1

62 94 19 84 -1 92 78 -1 15 -1 -1 92 -1 45 24 32 30 -1 -1  0  0 -1 -1 -1

71 -1 55 -1 12 66 45 79 -1 78 -1 -1 10 -1 22 55 70 82 -1 -1  0  0 -1 -1

38 61 -1 66  9 73 47 64 -1 39 61 43 -1 -1 -1 -1 95 32  0 -1 -1  0  0 -1

-1 -1 -1 -1 32 52 55 80 95 22  6 51 24 90 44 20 -1 -1 -1 -1 -1 -1  0  0

-1 63 31 88 20 -1 -1 -1  6 40 56 16 71 53 -1 -1 27 26 48 -1 -1 -1 -1  0

Rate 5/6 code:

1 25 55 -1 47  4 -1 91 84  8 86 52 82 33  5  0 36 20  4 77 80  0 -1 -1

-1  6 -1 36 40 47 12 79 47 -1 41 21 12 71 14 72  0 44 49  0  0  0  0 -1

51 81 83  4 67 -1 21 -1 31 24 91 61 81  9 86 78 60 88 67 15 -1 -1  0  0

50 -1 50 15 -1 36 13 10 11 20 53 90 29 92 57 30 84 92 11 66 80 -1 -1  0

8.6.2.3.2 Code Rate and Block Size Adjustment

The LDPC code flexibly supports different block sizes for each code rate through the use of an expansion factor. Each base model matrix has nb=24 columns, and the expansion factor (z factor) is equal to n/24 for code length n. In each case, the number of information bits is equal to the code rate times the coded length n.

Table 257 – LDPC Block Sizes and Code Rates

| n (bits) | n (bytes) | z factor | k (bytes) |  |  |  | Number of subchannels |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | R=1/2 | R=2/3 | R=3/4 | R=5/6 | QPSK | 16QAM | 64QAM |
| 384 | 48 | 16 | 24 | 32 | 36 | 40 | 8 | 4 | – |
| 480 | 60 | 20 | 30 | 40 | 45 | 50 | 10 | 5 | – |
| 576 | 72 | 24 | 36 | 48 | 54 | 60 | 12 | 6 | 4 |
| 672 | 84 | 28 | 42 | 56 | 63 | 70 | 14 | 7 | – |
| 768 | 96 | 32 | 48 | 64 | 72 | 80 | 16 | 8 | – |
| 864 | 108 | 36 | 54 | 72 | 81 | 90 | 18 | 9 | 6 |
| 960 | 120 | 40 | 60 | 80 | 90 | 100 | 20 | 10 | – |
| 1056 | 132 | 44 | 66 | 88 | 99 | 110 | 22 | 11 | – |
| 1152 | 144 | 48 | 72 | 96 | 108 | 120 | 24 | 12 | 8 |
| 1248 | 156 | 52 | 78 | 104 | 117 | 130 | 26 | 13 | – |
| 1344 | 168 | 56 | 84 | 112 | 126 | 140 | 28 | 14 | – |
| 1440 | 180 | 60 | 90 | 120 | 135 | 150 | 30 | 15 | 10 |
| 1536 | 192 | 64 | 96 | 128 | 144 | 160 | 32 | 16 | – |
| 1632 | 204 | 68 | 102 | 136 | 153 | 170 | 34 | 17 | – |
| 1728 | 216 | 72 | 108 | 144 | 162 | 180 | 36 | 18 | 12 |
| 1824 | 228 | 76 | 114 | 152 | 171 | 190 | 38 | 19 | – |
| 1920 | 240 | 80 | 120 | 160 | 180 | 200 | 40 | 20 | – |
| 2016 | 252 | 84 | 126 | 168 | 189 | 210 | 42 | 21 | 14 |
| 2112 | 264 | 88 | 132 | 176 | 198 | 220 | 44 | 22 | – |
| 2208 | 276 | 92 | 138 | 184 | 207 | 230 | 46 | 23 | – |
| 2304 | 288 | 96 | 144 | 192 | 216 | 240 | 48 | 24 | 16 |

8.6.2.3.3 Packet Encoding

The encoding block size k shall depend on the number of subchannels allocated and the modulation specified for the current transmission. Concatenation of a number of subchannels shall be performed in order to make larger blocks of coding where it is possible, with the limitation of not exceeding the largest block under the same coding rate (the block defined by the 64-QAM modulation). Tables 258 and 259 specify the concatenation of subchannels for different allocations and modulations.

For any modulation and FEC rate, given an allocation of Nsch subchannels, the following parameters can be defined:

j:	modulation-dependent parameter

Nsch:	number of allocated subchannels

F:	floor(Nsch/ j)

M:	Nsch mod j.

The parameter j for LDPC is determined as shown in Table 258:

Table 258 Parameter ‘j’ for LDPC

| j | Modulation |
| --- | --- |
| 48 | QPSK |
| 24 | 16-QAM |
| 16 | 64-QAM |

Table 259  Subchannel Concatenation

| NSCH | Subchannels concatenated |
| --- | --- |
| NSCH ≤ j | 1 block of NSCH subchannels |
| NSCH > j, M=0 | F blocks of j subchannels |
| NSCH > j, M>0 | (F – 1) blocks of j subchannels 1 block of ceil((M+j)/2) subchannels 1 block of floor((M+j)/2) subchannels |

Control information and packets that result in a codeword size n of less than 384 bits are encoded using convolutional coding (CC), with appropriate code rates and modulation orders, as described in section 8.6.2.1.

<add informative annex>

### Annex

### (informative)

### LDPC Direct Encoding

The code is flexible in that it can accommodate various code rates as well as packet sizes.

The encoding of a packet at the transmitter generates parity-check bits p=(p0, …, pm-1) based on an information block s=(s0, …, sk-1), and transmits the parity-check bits along with the information block. Because the current symbol set to be encoded and transmitted is contained in the transmitted codeword, the information block is also known as systematic bits. The encoder receives the information block s=(s0, …, sk-1) and uses the matrix Hbm to determine the parity-check bits. The expanded matrix H is determined from the model matrix Hbm. Since the expanded matrix H is a binary matrix, encoding of a packet can be performed with vector or matrix operations conducted over GF(2).

One method of encoding is to determine a generator matrix G from H such that G HT = 0. A k-bit information block s1k can be encoded by the code generator matrix Gkn via the operation x = s G to become an n-bit codeword x1n, with codeword x=[s p]=[s0, s1, …,sk-1, p0, p1, …,pm-1], where p0, . . . pm-1 are the parity-check bits; and s0, . . . sk-1 are the systematic bits.

Encoding an LDPC code from G can be quite complex. The LDPC codes are defined such that very low complexity encoding directly from H is possible.

Encoding is the process of determining the parity sequence p given an information sequence s. To encode, the information block s is divided into kb = nbmb groups of z bits. Let this grouped s be denoted u,

u=u0u1⋯ukb−1,

where each element of u is a column vector as follows

.

Using the model matrix Hbm, the parity sequence p is determined in groups of z.  Let the grouped parity sequence p be denoted v,

v=v0v1⋯vmb−1,

where each element of v is a column vector as follows

vi=pizpiz+1⋯pi+1z−1T.

Encoding proceeds in two steps, a) initialization, which determines v(0), and b) recursion, which determines v(i+1) from v(i), 0  i  mb2.

An expression for v(0) can be derived by summing over the rows of Hbm to obtain

Ppx,kbv0=j=0kb−1i=0mb−1Ppi,juj,	(1)

where x , 1  x  mb2, is the row index of hbm where the entry is nonnegative and unpaired, and Pi represents the zz identity matrix circularly right shifted by size i.  Since Ppx,kb is an identity matrix for all LDPC codes defined, (1) can be further simplified to

v0=j=0kb−1i=0mb−1Ppi,juj.

Considering the structure of Hb2, the recursion can be derived as follows,

v1=j=0kb−1Ppi,juj+Ppi,kbv0, i=0,	(2)

vi+1=vi+j=0kb−1Ppi,juj+Ppi,kbv0, i=1,...,mb−2	(3)

where

P−1≡0z×z.

Thus all parity bits not in v(0) are determined by evaluating Equation (2) and (3) for 0  i  mb2.

Equations (1) to (3) completely describe the encoding algorithm.  These equations also have a straightforward interpretation in terms of standard digital logic architectures.  Since the non-zero elements p(i,j) of Hbm represent circular shift sizes of a vector, all products of the form Pp(i,j)u(j) can be implemented by a size-z barrel shifter.

References:

[1]	IEEE 802.16eTM-2005, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems.  Amendment 2: Physical and Medium Access Control Layers for Combined Fixed and Mobile Operation in Licensed Bands and Corrigendum 1,” Feb 28, 2006.

[2]	IEEE 802.22-07/0250r2, “LDPC FEC Simulation Results,” V. Desai, S. Kuffner, June 2007.

[3]	IEEE 802.22-07/0287r0, “LDPC Simulation Results for Additional Block Sizes,” V. Desai, S. Kuffner, June 2007.

[4]	IEEE 802.22-07/0313r0, “LDPC Decoding for 802.22 Draft Standard,” Y. Blankenship, S. Kuffner, June 2007.