IEEE P802.22
Wireless RANs

| Concatenation rule and structure of component codes for SBTC |  |  |  |  |
| --- | --- | --- | --- | --- |
| Last Updated - Date: 2007-06-28 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Xu Changlong | I2R | Heng Mui Keng Terrace, Singapore 119613 | +6568747588 | clxu@i2r.a-star.edu.sg |
| Ying-Chang Liang | I2R | Heng Mui Keng Terrace, Singapore 119613 | +6568747588 | clxu@i2r.a-star.edu.sg |
| Zhongding Lei | I2R | Heng Mui Keng Terrace, Singapore 119613 | +6568747588 | clxu@i2r.a-star.edu.sg |
| Yonghong Zeng | I2R | Heng Mui Keng Terrace, Singapore 119613 | +6568747588 | clxu@i2r.a-star.edu.sg |
| Anh Tuan Hoang | I2R | Heng Mui Keng Terrace, Singapore 119613 | +6568747588 | athoang@i2r.a-star.edu.sg |
| Francois Chin | I2R | Heng Mui Keng Terrace, Singapore 119613 | +6568747588 | chinfrancois@i2r.a-star.edu.sg |

Abstract

This document describes the concatenation rule and structure of component codes for shortened block turbo codes for IEEE 802.22 WRAN system.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Document Policy and Procedures: The contributor is familiar with the IEEE 802 Document Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of document(s), including document applications, provided the IEEE receives assurance from the document holder or applicant with respect to documents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of document information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if documented technology (or technology under document application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Document Committee Administrator at <patcom@ieee.org>.

- Sub-channel concatenation rule

The encoding block size shall depend on the number of subchannels allocated and the modulation specified for the current transmission. Concatenation of a number of subchannels shall be performed in order to make larger blocks of coding where it is possible, with the limitation of not passing the largest block under the same coding rate. Table 2 specifies the concatenation of subchannels for different allocations and modulations. The parameters in Table 1 and 2 shall apply to the BTC encoding scheme.

For any modulation and FEC rate, given an allocation of n subchannels, the following parameters are defined:

-j: parameter dependent on the modulation and FEC rates

-n: number of allocated subchannels

-k: floor (n/j)

-m: n modulo j

The rules used for subchannel concatenation are showed in Table 1.

Table 1- Subchannel concatenation rule

| Number of suchannels | Subchannel concatenated |  |
| --- | --- | --- |
| n <= j | One block of n subchannels |  |
| n > j | m = 0 | k blocks of j subchannels |
|  | m ≠ 0 | (k-l) blocks of j subchannels One block of ceil ((m+j)/2 ) subchannels One block of floor ((m+j)/2) subchannels |

Table 2- Encoding subchannel concatenation for different allocations and modulations

| Modulation and rate | J |
| --- | --- |
| QPSK ½ | 12 |
| QPSK ¾ | 6 |
| 16-QAM 1/2 | 6 |
| 16-QAM 3/4 | 6 |
| 64-QAM 1/2 | 4 |
| 64-QAM 2/3 | 4 |
| 64-QAM 3/4 | 2 |
| 64-QAM 5/6 | 4 |

- Coded block size for one or multiple sub-channel

The BTC is based on the product of two simple component codes, which are Hamming codes including extended Hamming codes or parity check codes from the set depicted in table 4. Table 3 gives the block sizes of BTC for the possible modulation and coding schemes.  Table 4 gives the code parameters of BTC for different data payload and coded block sizes.

- Table 3—Useful data payload for one or multiple sub-channel

| Modulation scheme | QPSK |  | 16QAM |  | 64QAM |  |  |  | Coded Bytes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Encoding rate | 1/2 | 3/4 | 1/2 | 3/4 | 1/2 | 2/3 | 3/4 | 5/6 |  |
| Allowed Data (Bytes) | 3 | – | – | – | – | – | – | – | 6 |
|  | 6 | 9 | 6 | 9 | – | – | – | – | 12 |
|  | 9 | – | – | – | 9 | 12 | – | 15 | 18 |
|  | 12 | 18 | 12 | 18 | – | – | – | – | 24 |
|  | 15 | – | – | – | – | – | – | – | 30 |
|  | 18 | 27 | 18 | 27 | 18 | 24 | 27 | 30 | 36 |
|  | 21 | – | – | – | – | – | – | – | 42 |
|  | 24 | 36 | 24 | 36 | – | – | – | – | 48 |
|  | 27 | – | – | – | 27 | 36 | – | 45 | 54 |
|  | 30 | 45 | 30 | 45 | – | – | – | – | 60 |
|  | 33 | – | – | – | – | – | – | – | 66 |
|  | 36 | 54 | 36 | 54 | 36 | 48 | 54 | 60 | 72 |
|  | For coded block size large than 72 bytes, code construction will be based on concatenation rule |  |  |  |  |  |  |  |  |

Table 4 Code parameters for different data payload coded block sizes

| Data Bytes | Coded Bytes | Constituent | Code parameters |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Ix | Iy | D |
| 3 | 6 | (15,11)(8,7) | 3 | 4 | 0 |
| 6 | 12 | (16,11) (8,7) | 4 | 0 | 1 |
| 9 | 12 | (16,15) (16,15) | 10 | 0 | 3 |
| 9 | 18 | (16,11) (16,15) | 4 | 4 | 5 |
| 12 | 18 | (8,7) (64,63) | 4 | 28 | 9 |
| 15 | 18 | (16,15) (16,15) | 7 | 0 | 0 |
| 12 | 24 | (16,11) (16,15) | 4 | 0 | 9 |
| 18 | 24 | (8,7) (32,31) | 2 | 0 | 11 |
| 15 | 30 | (15,11) (31,26) | 3 | 11 | 0 |
| 18 | 36 | (15,11) (31,26) | 3 | 7 | 8 |
| 24 | 36 | (16,15) (32,26) | 4 | 8 | 6 |
| 27 | 36 | (8,7) (64,63) | 2 | 16 | 19 |
| 30 | 36 | (16,15) (32,31) | 7 | 0 | 8 |
| 21 | 42 | (7,4) (63,57) | 0 | 15 | 0 |
| 24 | 48 | (16,11) (32,26) | 0 | 8 | 6 |
| 36 | 48 | (16,15) (63,57) | 8 | 15 | 6 |
| 27 | 54 | (32,26) (32,26) | 14 | 8 | 0 |
| 36 | 54 | (32,31) (31,26) | 8 | 13 | 11 |
| 45 | 54 | (8,7) (64,63) | 0 | 10 | 11 |
| 30 | 60 | (32,26) (32,26) | 2 | 16 | 0 |
| 45 | 60 | (16,15) (32,26) | 0 | 2 | 0 |
| 33 | 66 | (32,26) (32,26) | 8 | 10 | 24 |
| 36 | 72 | (32,26) (32,26) | 14 | 0 | 24 |
| 48 | 72 | (16,11) (64,63) | 0 | 28 | 1 |
| 54 | 72 | (16,15) (64,57) | 0 | 28 | 3 |
| 60 | 72 | (16,15) (64,63) | 7 | 0 | 24 |

- Complexity

As illustrated on Error: Reference source not found, in the receiver chain, the demodulated symbols are de-mapped, de-scrambled and de-interleaved to form soft bits of SBTC block (a SBTC packet) and are stored in a frame buffer for SBTC decoding. The module “SBTC block de-pack” chops frame data into SBTC blocks, and writes a block into SISO buffer for SBTC decoding with recovered shortening.

Figure 1: SBTC decoder function diagram

The SISO buffer (SISO input) and extrinsic buffers (SISO output) are consisting of 4 pages of dual-port RAM to cater for:

- 4 parallel operation of SISO process engines

- Reading of block data without row/column transpose during row/column decoding iterations.

A duplicated SISO buffer would be planed as ping-pong buffer for simultaneously writing and reading SBTC block data to achieve maximum data throughput.

The output of SISO engine is written to extrinsic buffer for decoding iterations. After iterations of SBTC decoding, the decoded data byte are outputted to MAC module. The SBTC SISO engine and SISO buffer are controlled by a state machine (FSM). The buffer write/read points are generated by the Point Generator module.

The SBTC block size is configurable to cater for different SBTC decoding block size. The extension of SBTC block is configurable too.

The maximum complexity is about 16k slices, i.e. 45% Vrtex4 LX80 resource (Xilinx), according to FPGA implementation.