IEEE P802.22
Wireless RANs

| Forward Coexistence Measurement report to Neighbor Cell |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-06-29 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jinnan Liu | Huawei Hisi | Beijing, China | 86-10-8282-9148 | liujinnan@huawei.com |
| Shulan Feng | Huawei Hisi | Beijing, China | 86-10-8282-9151 | fengsl@huawei.com |
| Wu Yu-chun | Huawei Hisi | Beijing, China | 86-10-8282-9018 | wuyuchun@huawei.com |
| Jianwei Zhang | Huawei Technologies | Shanghai, China | 86-21-68644808 | zhangjianwei@huawei.com |
| Soo-Young Chang | Huawei Technologies | Davis, CA, U.S. | 1-916 278 6568 | sychang@ecs.csus.edu |

Abstract

In order to improve the coexistence capability of WRAN, we propose the measurement reports from CPEs in  multiple overlapping 802.22 cells  should be not only transmitted  to its service BS but also forwarded to its co-existence BSs

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Reference

[1]     IEEE P802.22™/D0.2 Draft Standard for Wireless Regional Area Networks Part 22: Cognitive Wireless RAN Medium Access Control (MAC) and Physical Layer (PHY) specifications: Policies and procedures for operation in the TV Bands

[2]    22-07-0121-03-0000-integrated-coexistence

[3]     22-07-0043-000000_Huawei_Notification_of_Sensing_Capability

# Introduction

In ref.[1]6.20, coexistence mechanisms give coexistence solution to coexist with  both incumbent and other 802.22 cells.

But the coexistence capability of WRAN depends on the distribution of CPEs heavily. The spectrum usage will be blind or semi-blind for BS in the area which is lack of CPEs and far away from BS. Meanwhile the inter-BS communication will be unreliable if there is no B-CPE ( bridge-CPE) as a relay for neighbor cells.

Fig 1   Distributive sensing Capacity of WRAN

Considering figure 1 scenario, WRAN A system works on channel A .The WRAN A includes BS A and four CPEs associated with BS A. The veracity of spectrum sense in WRAN A has flaws in the right part of cell coverage. So the signals in the right part of cell coverage from either incumbent or neighbor WRAN have high missing risks.

# Forward coexistence measurement report to neighbor cell

In order to improve the coexistence capability of WRAN, we propose the CPEs in multiple overlapping 802.22 cells not only transmit the measurement report to its service BS but also forward coexistence measurement report to its co-existence BSs.

After B-CPE ( the CPEs in multiple overlapping 802.22 cells as a bridge) does measurement  under the instruction of its service BS , B-CPE firstly transmits the measurement report to its service BS. If the mearement report belongs to coexistence measurement report, the service BS will assign B-CPE to forward coexistence mearsurement report to its neighbor cells through CBP or send CBP itself.

2.1 Forward Coexistence measurement report to neighbour cell through CBP

In order to delivery coexistence measurement report ,  we introduce a B-CPE Measurement Report IE (BM_REP_IE) in CBP IE[2]  including coexistence measurement report as shown in table 1, the contents of  BM_REP_IE should include at least one type of  coexistence measurement report  in the section 2.2.

Table 1 CBP Information Element

| Element ID | Name |
| --- | --- |
| 0 | Beacon IE |
| 18 | BS Channel Parameter IE |
| XX | BM_REP_IE |

2.2 Coexistence Measurement Report

We redefine a subgroup of measurement reports as coexistence measurement report based on the type of measurement report in ref. [3].

We reclassify measurement reports into two types, coexistence measurement report and non-coexistence measurement report. And we define a subtype which is related coexistence as coexistence measurement report in bold shown in table 2.

Table 1  Type of measurement report

| Element ID (1 byte) | Length (bytes) | Description |
| --- | --- | --- |
| 0 | Variable | TV System Related Measurement Report |
| 1 | Variable | Part 74 System Related Measurement Report |
| 2 | Variable | Beacon (802.22 BS and CPE Related) Measurement Report |
| 3 | Variable | Stop Measurement Report |
| 4 | Variable | CPE Statistics Measurement Report |
| 5 | Variable | Location Configuration Measurement Report |
| 6 | Variable | Beacon (Part 74) Measurement Report |
| 7 | Variable | Channel Feedback Measurement Report |
| 8 | Variable | WLAN System Related Measurement Report |
| 9 | Variable | WPAN System Related Measurement Report |
| 10 | Variable | WMAN System Related Measurement Report |
| 11-128 |  | Reserved |

Note:

Element ID 0~2 ,6, and 8~10 belong to coexistence measurement report,

Where Element ID 0, 1,and  6 report signals from the incumbent, Element ID 2 reports signals from other WRAN system , Element ID 8,9, and10 report signals from other cognitive radio system.

2.3 Scenario

1)   Coexistence with incumbent

As shown in fig 2, BS A and associated CPEs work on channel A ,while BS B and associated CPEs (including B-CPE) work on channel B. At the same time, incumbent works on channel A.B-CPE associated with BS B gets the incumbent on channel A, while BS A can’t detect incumbent and does interference with incumbent.

When B-CPE gets the measurement report about incumbent, B-CPE first reports the measurement result to BS B. And then BS B assigns B-CPE to send CBP with coexistence measurement report to WRAN A system. After BS A gets know the appearance of incumbent on channel A, BS A gives up channel A as soon as possible.

Fig 2 coexistence with incumbent

2)   Coexistence with other WRAN systems

As shown in fig 3, both WRAN A and WRAN C work on channel A, while BS B and associated CPEs (including B-CPE) work on channel B. WRAN A  is hard to discovery WRAN C as neighbor.B-CPE have the capability to get the CBP from both BS A and BS C.

When B-CPE gets the CBP from WRAN A or C, B-CPE sends to BS B. If BS B finds WRAN A is a hidden neighbour for WRAN C, BS B assigns B-CPE to forward WRAN A’s CBP to WRAN C or forward WRAN C’s CBP to WRAN A.

Fig 3 coexistence with other WRAN system

# Conclusion

We propose a method to find hidden incumbent or hidden neighbour by forwarding coexistence measurement report in overlap area. Our proposal can take advantage of measurement report from the non-service CPE in overlapping area and improve coexistence with both incumbents and WRANs.