IEEE P802.22
Wireless RANs

| Complexity Estimates for Duo-Binary Turbo Code |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-06 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| John Benko | France Telecom R&D | 801 Gateway Blvd. Suite 500, South San Francisco, CA 94080 | 650-875-1593 | john.benko@orange-ftgroup.com |
| Jacky Tousch | Turbo Concept |  |  | jacky.tousch@turboconcept.com |

Abstract

This document describes the complexity of a decoder implementation for the duo-binary turbo code, as described in document 22-07-0030-01.  These post-synthesis ASIC results were obtained in December 2004.

.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Description of the turbo code

In this part, the turbo code is described, taken from 22-07-0030-01.

## Turbo Encoding

The turbo encoder is an 8-state turbo encoder, as depicted on Figure 1. It uses a double binary Circular Recursive Systematic Convolutional (CRSC) code. The most significant bit of the first byte of the useful payload is assigned to A, the next bit to B, and so on for the remaining of the data burst content. The encoder is fed by blocks of k bits or N couples (k=2*N bits). N is a multiple of 4 (k is a multiple of 8).

Figure 1 : Encoder block diagram

The polynomials, which shall be used for the connections, are described in octal and symbolic notations as follows:

- for the feedback branch: 15 (in octal), equivalently 1+D+D3 (in symbolic notation);

- for the Y1 and Y2 parity bits, 13, equivalently 1+D2+D3;

The input A shall be connected to tap “1” of the shift register and the input “B” shall be connected to the input taps “1”, D and D2.

After initialisation by the circulation state Sc1, the encoder shall be fed by the sequence in the natural order with incremental address i = 0,…,N-1. This first encoding is called C1 encoding.

After initialisation by the circulation state Sc2, the encoder shall be fed by the interleaved sequence with incremental address j = 0,… N-1. This second encoding is called C2 encoding.

The function (j) that gives the natural address i of the considered couple, when reading it at place j for the second encoding, is given in clause 1.2.

## Turbo Code Permutation

The permutation shall be done on two levels:

- the first one inside the couples (level 1),

- the second one between couples (level 2),

The permutation is expressed in the following algorithm.

- Set the permutation parameters P0, P1, P2 and P3.

For MPEG2-TS packet size (188 bytes): P0 = 19, P1 = 376, P2 = 224 and P3 = 600.

- j = 0,… N-1.

- level 1

if j mod. 2 = 0, let (A,B) = (B,A) (invert the couple)

- level 2

- if j mod. 4 = 0, then P = 0;

- if j mod. 4 = 1, then P = N/2 + P1;

- if j mod. 4 = 2, then P = P2;

- if j mod. 4 = 3, then P = N/2 + P3.

    - i = P0*j + P + 1 mod. N

The interleaving relations shall satisfy the odd/even rule (i.e. when j is even, i is odd and vice-versa) that enables the puncturing patterns to be identical for the two puncturing levels.

## Rates & Puncturing Map

Four code rates are defined  : R=1/2, 2/3, ¾, 5/6.

These rates shall be achieved through selectively deleting the parity bits (puncturing). The puncturing pattern defined in Table 1 shall be applied.

| Code Rate | Puncturing vector |
| --- | --- |
| 1/2 | Y = [1 1 1 1 1 1] |
| 2/3 | Y = [1 0 1 0 1 0] |
| 3/4 | Y = [1 0 0 1 0 0] |
| 5/6 | Y = [1 0 0 0 0] |

Table 1 : Puncturing patterns for turbo codes (“1”=keep, “0”=delete)

In Table 1, the value “1” means that the bit shall be kept, while the value “0” means that the bit shall be deleted.  The puncturing patterns shall be identical for both codes C1 and C2.

# Decoding Algorithm

This section gives an overview of the decoding algorithm and architecture.

## Parameters

The notations adopted for the main parameters of the encoder and decoder are given here. Their default value is also given. If no mention of the contrary, the default value stated here is applicable:

| Parameter | Name | Value |
| --- | --- | --- |
| Maximum payload size (bits) | Kmax | 1536 |
| Minimum code rate | Rmin | 1/2 |
| Quantization input bit LLRs | wd | 5 |
| Quantization extrinsic data | wz | 5 |
| Quantization metrics | wm | 9 |
| Sliding window size | L | 32 |
| Number of states | Ns | 8 |
| Number of iterations | ITmax | 8 |
| Decoder clock frequency | Fck | 200 |
| Number of SISO modules (parallelism) | P | 4 |

Table 2 : Parameters and notations

## The SISO decoder

The basic component decoder is called a Soft-Input Soft Output module (SISO). A symbol SISO module is required for optimal performance. A symbol is defined as a couple of bits S = (A,B). The purpose of the SISO module is to compute a set of 4 symbol extrinsic information for each symbol : The extrinsic corresponding to A=i and B=j is denoted Leij.

The SISO module implements a Max-Log MAP algorithm using forward metrics  and backward metrics .

For each k = 0..N-1, and each state s =0 to 7, the metrics are computed recursively using :

| ak(s)=max(s,s')ak−1(s')+ck(s',s) | (2.2-) |
| --- | --- |
| bk(s)=max(s,s')bk+1(s')+ck+1(s,s') | (2.2-) |

The branch metrics ck are computed with the channel observations Ak, Bk,Yk, their respective binary value A, B and Y on the branch, and the extrinsic value produced by the other SISO module used as a priori information Lk(i,j):

| ck(s',s)=Ak2A−1+Bk2B−1+Yk2Y−1+Lk(i,j) | (2.2-) |
| --- | --- |

The extrinsic value is then obtained

| Leij=max(s',s)Ak=i,Bk=jak−1(s')+ck(s',s)+bk(s)−ckp(s',s) | (2.2-) |
| --- | --- |

where ckp is the following (a priori) part of the branch metric :

| ckp(s',s)=Ak2A−1+Bk2B−1+Lk(i,j) | (2.2-) |
| --- | --- |

## Decoder architecture

The architecture of the decoder is shown in Figure 2. Alternative decoder architectures can be adopted, leading to different complexity results.

The main components are :

- two memories storing input bit LLR samples, each one containing 1 frame ;

- an extrinsic memory containing the extrinsic data for 1 frame.

- P SISO modules working in parallel ;

- a memory containing the decision bits for 1 frame.

Figure 2: architecture of the decoder

The turbo decoder permutation allows for such a parallel implementation. Each of the shared memory of size w words by b bits (input samples and extrinsic memories) is split into P sub-memories, each of size w/P words of b bits.

Each SISO implements a sliding window algorithm with a window size of L trellis steps. A memory of L backward metrics is contained into each SISO module. A SISO module contains a backward recursion unit, a forward recursion unit and a symbol extrinsic computation unit.

A decoding iteration consists in two successive half iterations. The same P SISOs are used successively to perform each one of the two half-iterations, as illustrated in Figure 3 for the case P=2.

Figure 3 : sliding window processing (case P=2)

Each SISO decodes 2 bits per clock cycle per half-iteration, resulting in the following overall decoder throughput (in decoded bits/sec):

D=PITmax⋅Fck

Notes :

- the impact on throughput due to pipe-lining and frame overhead processing has not been taken into account.

- the interleaving / deinterleaving units are not represented. However the complexity estimate take these units into account.

# Complexity estimate – Max Block Size = 2048 bits

This section gives a complexity estimate, in terms of :

- number of operators and memory

- ASIC surface

| Max. payload size Kmax (bits) | min. code rate R | max. coded block size (bits) |
| --- | --- | --- |
| 1536 | 3/4 | 2048 |
| 1365 | 2/3 | 2048 |
| 1024 | 1/2 | 2048 |

Table 3 : Second set of block size and code rate constraints

## Number of operators and memory bits

This section gives the number of operators needed per decoded bit as a function of the code and decoder parameters listed in Table 2, amended by the constraints stated in . The memory requirements are also addressed.

### Number of operators

This section details the computation requirements for each SISO trellis step, and derives the computational requirements needed for each turbo decoded bit.

A backward or forward trellis steps require each:

- the computation of the set of branch metrics : 14 adders

- for each of the 22.Ns branches:

  - addition of a branch metric to a state metric : 4 adders

- for each of the Ns states :

  - selection between 4 concurrent metrics, i.e. 3 max( )

The extrinsic information computation requires for each trellis step :

- for each of the 22.Ns branches:

  - addition of forward and backward metrics

- for each of the 22 symbols:

  - a max over Ns values : Ns-1 max( )

  - a subtraction of the a priori metric.

- selection of the min. value and substation from all 4 values : 3 max() and 4 adders

The hard decision is needed only for the last half iteration, and is the result of a comparison between 4 symbol a posteriori values, which are intermediate results of the extrinsic computation. The additional complexity required is then 3 max() operations.

The following table gives the summary of the complexity in terms of logic operators, as well as the total number of operators per decoded bit per iteration, taking into account that :

- 2 bits are decoded per trellis step,

- 2 SISOs are needed for 1 iteration

|  | Sum | Max |
| --- | --- | --- |
| Backward | 22.Ns+14 = 46 | 3.Ns = 24 |
| Forward | 22.Ns+14= 46 | 3.Ns= 24 |
| Extrinsic | 22.Ns+4 +4= 40 | 4.(Ns-1) +3= 31 |
| Hard decision | 0 | 3 |
| Total per trellis step | 132 | 79 |
| Total per decoded bit per iteration | 132 | 79 |
| Total per decoded bit (8 iteration) | 1056 | 632 |
| Total per decoded bit (5 iteration) | 660 | 395 |

Table 4 : Number of operators

### Memory requirements

The different memory blocks needed are listed below. The resulting memory need for a turbo decoder is given in table 6.

| Name | heigth (words) | width (bits) | size (bits) | instances needed |
| --- | --- | --- | --- | --- |
| input samples - systematic | 768 | 10 | 7680 | 2 |
| input samples - redundancy | 512 | 10 | 5120 | 2 |
| extrinsic | 768 | 17 | 13056 | 1 |
| sliding window | 32 | 72 | 2304 | 6 |
| decision bits | 768 | 2 | 1536 | 1 |

Table 5 : Memory blocks

| P | total memory (bits) | memory bits per decoded bit (case 1536bits) |
| --- | --- | --- |
| 4 | 49408 | 32,1 |

Table 6 : Decoder memory requirements

| Memory | Type |
| --- | --- |
| input samples memory | SinglePort RAM |
| extrinsic memory | DPRAM |
| sliding window memory | SinglePort RAM |

## ASIC oriented complexity

### Parallelism degree

An assumption of 200 MHz has been taken as clock frequency, which is conservative for technologies of 0.13um and beyond.

Using the architecture described above, the required parallelism degree of 4 is needed to achieve decoded bitrate of 100 Mbps at 8 iterations (100 Mbps allows for future channel aggregation and/or bonding).  Higher degrees of parallelism is also possible to higher datarates.

### Complexity figures

The complexity of the turbo decoder corresponding to a parallelism degree of 4 is given in the following table. These results are based on post-synthesis results, using a 0.13um technology of average density of 222 kgates / mm2. The respective proportion associated to logic and memory is also given.  This corresponds to less than 200 kgates.

| parallelism degree P | decoder complexity (mm2) | logic (%) | memory (%) | decoded bitrate @ 8 it. | decoded bitrate @ 5 it. |
| --- | --- | --- | --- | --- | --- |
| 4 | 0.836 | 43 | 57 | 100 | 160 |

Table 7 : complexity figures, 0.13um technology

## Latency

The decoding latency T depends on the frame size k (in bits) , the number of decoding iterations Nit , the decoder clock frequency Fck (Hz) and the parallelism degree P as follows :

T=k⋅NitP⋅Fck seconds

Note that the decoding time is not dependant on the code rate.

The following tables give the decoding time associated to different block sizes(1024, 1365, 1536 payload bits), for different number of iterations. A clock frequency of 200 MHz has been used :

| parallelism degree P | Iterations Nit | Decoding Time T (us) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 1024 bits | 1365 bits | 1536 bits |
| 4 | 5 | 6.4 | 8.5 | 9.6 |
| 4 | 8 | 10.2 | 13.7 | 15.4 |

Table 8 : decoding time, 1024, 1365, and 1536 payload bits

Note that the frame transmission time, being not dependant on the decoder, has not been taken into account.