IEEE P802.22
Wireless RANs

| Efficient way to get and set Multiple MIB attributes |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-03 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jianwei Zhang | Huawei Technologies | 450 Jinyu Road, Pudong, Shanghai, P. R. China, 201206 | 86-21-50993036 | zhangjianwei@huawei.com |
| Mingwei Jie | Huawei Technologies | 450 Jinyu Road, Pudong, Shanghai, P. R. China, 201206 | 86-21-50993037 | jiemingwei@huawei.com |
| Xuesheng Zhu | Huawei Technologies | 450 Jinyu Road, Pudong, Shanghai, P. R. China, 201206 | 86-21-50993036 | zhuxuesheng@huawei.com |

Abstract

Efficient Way to Get and Set Multiple MIB Attributes is suggested.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Efficient getting and setting of Multiple MIB attributes

MLME-GET.request

The MLME-GET.request primitive can only get one MIB attribute each time in the IEEE802.22.1D1.0. The fact is that MIB has 23 attributes. It is not efficient enough when MLME wants to get multiple attributes.

7.1.1.2 MLME-GET.request

The MLME-GET.request primitive is generated by the next higher layer and issued to its MLME to request information about multiple given MIB attributes. Table 28 specifies the parameters for the MLME-GET.request primitive.

Table 28—MLME-GET.request parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| MIBAttribute 1 | Integer | See Table 45 | The identifier of the MIB attribute to read. |
| MIBAttribute 2 | Integer | See Table 45 | The identifier of the MIB attribute to read. |
| …… |  |  |  |
| MIBAttribute n | Integer | See Table 45 | The identifier of the MIB attribute to read. |

On receipt of the MLME-GET.request primitive, the MLME attempts to retrieve the requested MIB attributes from its database. If the requested MIB attribute is successfully retrieved, the MLME will issue the MLMEGET. confirm primitive with a status of SUCCESS. Otherwise, the MLME will issue the MLMEGET. confirm primitive with the appropriate error status (see 7.1.1.3).

7.1.1.3 MLME-GET.confirm

The MLME-GET.confirm primitive is generated by the MLME and issued to its next higher layer in response to an MLME-GET.request primitive, and it reports the results of an information request from the MIB. Table 29 specifies the parameters for the MLME-GET.confirm primitive.

Table 29—MLME-GET.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| MIBAttribute 1 | Integer | See Table 45 | The identifier of the MIB attribute that was read. |
| MIBAttributeValue 1 | Various | Attribute specific; see Table 45 | The value of the indicated MIB attribute that was read. This parameter has zero length when the Status parameter is set to ATTRIBUTE_NOT_FOUND. |
| Status 1 | Enumeration | SUCCESS or ATTRIBUTE_NOT_FOUND | The result of the request for MIB attribute information. |
| MIBAttribute 2 | Integer | See Table 45 | The identifier of the MIB attribute that was read. |
| MIBAttributeValue 2 | Various | Attribute specific; see Table 45 | The value of the indicated MIB attribute that was read. This parameter has zero length when the Status parameter is set to ATTRIBUTE_NOT_FOUND. |
| Status 2 | Enumeration | SUCCESS or ATTRIBUTE_NOT_FOUND | The result of the request for MIB attribute information. |
| …… |  |  |  |
| MIBAttribute n | Integer | See Table 45 | The identifier of the MIB attribute that was read. |
| MIBAttributeValue n | Various | Attribute specific; see Table 45 | The value of the indicated MIB attribute that was read. This parameter has zero length when the Status parameter is set to ATTRIBUTE_NOT_FOUND. |
| Status n | Enumeration | SUCCESS or ATTRIBUTE_NOT_FOUND | The result of the request for MIB attribute information. |

On receipt of the MLME-GET.confirm primitive, the next higher layer is notified of the results of its request to read multiple MIB attributes. If the request to read a MIB attribute was successful, the Status parameter is set to SUCCESS. If the identifier of the MIB attribute was not found in the database, the Status parameter is set to ATTRIBUTE_NOT_FOUND.

MLME-SET.request

The MLME-SET.request primitive can only set one MIB attribute each time. The fact is that MIB has 23 attributes. It is not efficient enough when MLME wants to set multiple attributes.

7.1.1.10 MLME-SET.request

The MLME-SET.request primitive is generated by the next higher layer and issued to its MLME, and it attempts to write the given values to the indicated MIB attributes. The MLME-SET.request primitive may have several pairs of parameters, and each pair will specify the MIB attribute and the corresponding value see Table 35.

- Table 35—MLME-SET.request parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| MIBAttribute 1 | Integer | See Table 45 | The identifier of the MIB attribute to write. |
| MIBAttributeValue 1 | Various | Attribute specific; see Table 45 | The value to write to the indicated MIB attribute. |
| MIBAttribute 2 | Integer | See Table 45 | The identifier of the MIB attribute to write. |
| MIBAttributeValue 2 | Various | Attribute specific; see Table 45 | The value to write to the indicated MIB attribute. |
| …… |  |  |  |
| MIBAttribute n | Integer | See Table 45 | The identifier of the MIB attribute to write. |
| MIBAttributeValue n | Various | Attribute specific; see Table 45 | The value to write to the indicated MIB attribute. |

On receipt of the MLME-SET.request primitive, the MLME attempts to write the given values to the indicated MIB attributes in its database. If the requested MIB attribute is successfully written, the MLME will issue the MLME-SET.confirm primitive with a status of SUCCESS. Otherwise, the MLME will issue the MLME-SET.confirm primitive with the appropriate error status (see 7.1.1.11).

7.1.1.11 MLME-SET.confirm

- The MLME-SET.confirm primitive is generated by the MLME and issued to its next higher layer in response to an MLME-SET.request primitive, and it reports the results of an attempt to write the values to multiple MIB attributes. Table 36 specifies the parameters for the MLME-SET.confirm primitive.

- Table 36—MLME-SET.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| MIBAttribute 1 | Integer | See Table 45 | The identifier of the MIB attribute that was written. |
| Status 1 | Enumeration | SUCCESS, ATTRIBUTE_NOT_FOUND, or INVALID_PARAMETER | The result of the request to write the MIB attribute. |
| MIBAttribute 2 | Integer | See Table 45 | The identifier of the MIB attribute that was written. |
| Status 2 | Enumeration | SUCCESS, ATTRIBUTE_NOT_FOUND, or INVALID_PARAMETER | The result of the request to write the MIB attribute. |
| …… |  |  |  |
| MIBAttribute n | Integer | See Table 45 | The identifier of the MIB attribute that was written. |
| Status n | Enumeration | SUCCESS, ATTRIBUTE_NOT_FOUND, or INVALID_PARAMETER | The result of the request to write the MIB attribute. |

On receipt of the MLME-SET.confirm primitive, the next higher layer is notified of the result of its request to set the values of multiple MIB attributes. If the requested value was written to the indicated MIB attribute, the corresponding Status parameter is set to SUCCESS.

If the MIBAttribute parameter specifies an attribute that is not found in the database, the Status parameter is set to ATTRIBUTE_NOT_FOUND. If the MIBAttributeValue parameter specifies a value that is out of the valid range for the given attribute, the Status parameter is set to INVALID_PARAMETER.