IEEE P802.22
Wireless RANs

| Indication of the losted SPD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-03 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jianwei Zhang | Huawei Technologies | 450 Jinyu Road, Pudong, Shanghai, P. R. China, 201206 | 86-21-50993036 | zhangjianwei@huawei.com |
| Mingwei Jie | Huawei Technologies | 450 Jinyu Road, Pudong, Shanghai, P. R. China, 201206 | 86-21-50993037 | jiemingwei@huawei.com |
| Xuesheng Zhu | Huawei Technologies | 450 Jinyu Road, Pudong, Shanghai, P. R. China, 201206 | 86-21-50993036 | zhuxuesheng@huawei.com |

Abstract

When SPD exits the system abnormally, i.e., not issuing Cease Tx to PPD, PPD should know this change. The mechanism and some complementary MIB attributes are provided to let PPD get the information.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Indication of the losted SPD

To protect the low power licensed devices operating in television broadcast bands, it is natural for each PD to set its Required Need Timer with a rough big conservative time length. In IEEE 802.22.1/D1.0, if the SPD leaves the radio space without notifying the PPD, the PPD shall continue to transmit beacons frames containing the information corresponding to the SPD until the time the PPD itself ceases transmission. The channels protected by that departure SPD will be overprotected in the remaining time of Required Need Timer. In addition, the low power licensed devices without deploying a protecting device should not be protected in principal.

To resolve the overprotection, every SPD shall send at least one RTS burst every macActivePeriodSPD superframes to get one chance to update its record in PPD. If the PPD does not receive any beacon frame from one SPD within macMissedSPDBeacons superframes continually, the PPD will consider this SPD is gone and the MLME shall issue this information to the next higher layer via the MLME-SPD-LOST.indication primitive.

The recommended text and tables update is as following.

### 1. MLME-SPD-LOST.indication primitive

Table 27—Primitives supported by the MLME-SAP

| Name | Request | Indication | Confirm |
| --- | --- | --- | --- |
| MLME-BEACON-LOST |  | 7.1.1.1 |  |
| MLME-GET | 7.1.1.2 |  | 7.1.1.3 |
| MLME-INCOMING-BEACON |  | 7.1.1.4 |  |
| MLME-NPD | 7.1.1.5 |  | 7.1.1.6 |
| MLME-NPD-LOST |  | 7.1.1.7 |  |
| MLME-SCAN | 7.1.1.8 |  | 7.1.1.9 |
| MLME-SET | 7.1.1.10 |  | 7.1.1.11 |
| MLME-SPD-LOST |  | 7.1.1.12 |  |
| MLME-START-BEACON | 7.1.1.13 |  | 7.1.1.14 |

7.1.1.12 MLME-SPD-LOST.indication

The MLME-SPD-LOST.indication primitive is generated by the MLME of a PPD and issued to its next higher layer as a notification that the beacon frames of SPD were not heard in the last macMissedSPDBeacons superframes. This primitive shall only be issued by a PPD and has no parameters.

### 2. Corresponding MIB attributes

Table 45—MIB attributes

| Attribute | Identifier | Type | Range | Description | Default |
| --- | --- | --- | --- | --- | --- |
| … | … | … | … | .. | … |
| macActivePeriodSPD | 0x17 | Integer | 100-500 | SPD should transmit at least one RTS burst every macActivePeriodSPD beacon frames and then send one update beacon frame with ACK from PPD . | 200 |
| macMissedSPDBeacons | 0x18 | Integer | 1000-6000 | The number of consecutive missed SPD beacon frames that will cause the PPD to send a LOST notification to the next higher layer. If the device is operating as an SPD, this attribute shall not apply. | 3000 |

### 3. Beaconing of SPD

7.4.6.2 Secondary protecting device (SPD):

Every SPD shall be responsible for verifying that its protection needs remain satisfied by continuously monitoring the PPD. Every received beacon frame shall be passed to the next higher layer via the MLMEINCOMING- BEACON.indication primitive. If the SPD misses macMaxMissedBeaconsSPD consecutive beacon frames, the MAC sublayer shall notify the next higher layer that it no longer detects the PPD's beacon via the MLME-BEACON-LOST.indication primitive. The action taken by the next higher layer on receipt of this primitive is out of the scope of this standard.

If the SPD wishes to transmit its own beacon frame by interrupting the PPD, it shall follow the procedure described in 7.4.5.

Every SPD shall send at least one RTS burst every macActivePeriodSPD superframes to get one chance to update its record in PPD. If the PPD does not receive any beacon frame from one SPD within macMissedSPDBeacons superframes continually, the PPD will consider this SPD is gone and the MLME shall issue this information to the next higher layer via the MLME-SPD-LOST.indication primitive.

### 4. Ceasing transmissions of SPD

If the SPD leaves the radio space without notifying the PPD, the PPD will delete the corresponding information when it knows this change.