IEEE P802.22
Wireless RANs

| Spectrum Sensing of the DTV in the Vicinity of the Pilot Using Higher Order Statistics |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Apurva Mody | BAE Systems | P.O. Box 868, MER 15-2350, Nashua, NH 03061-0868 | 603-885-2621 404-819-0314 | apurva.mody@baesystems.com, apurva_mody@yahoo.com |

Abstract

We present an algorithm to detect the DTV Signals in Gaussian noise using Higher Order Statistics (HOS). The algorithm performs non-Gaussianity check in the frequency domain in the vicinity of the Pilot of the DTV. The Average results for all the 12 provided DTV Signals are summarized as: At Pfalse alarm  = 0.04,

  - For a capture of 5mS (continuous or staggered), Pdetection= 0.9 at SNR = -16.6 dB.

  - For a capture of 10 mS (continuous or staggered), Pdetection = 0.9 at SNR =  -18.6 dB

  - For a capture of 20 mS (continuous or staggered), Pdetection = 0.9 at SNR =  -21.6 dB

  - For a capture of 40 mS (continuous or staggered), Pdetection = 0.9 at SNR =  -23.7 dB

We use a 2048 point FFT to implement the algorithm which may be used for Spectrum Sensing as well as OFDMA demodulation. Because of the Constant False Alarm Rate (CFAR) nature of the detector, the threshold is built into the technique and NO presetting is required. Fine adjustment is possible if needed.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

1. Spectrum Sensing of the DTV in the Vicinity of the Pilot Using Higher Order Statistics

We present an algorithm to detect the DTV Signals in Gaussian noise using Higher Order Statistics (HOS). The algorithm performs non-Gaussianity check in the frequency domain in the vicinity of the Pilot of the DTV. The Average results for all the 12 provided DTV Signals are summarized as: At Pfalse alarm  = 0.04,

  - For a capture of 5mS (continuous or staggered), Pdetection= 0.9 at SNR = -16.6 dB.

  - For a capture of 10 mS (continuous or staggered), Pdetection = 0.9 at SNR =  -18.6 dB

  - For a capture of 20 mS (continuous or staggered), Pdetection = 0.9 at SNR =  -21.6 dB

  - For a capture of 40 mS (continuous or staggered), Pdetection = 0.9 at SNR =  -23.7 dB

We use a 2048 point FFT to implement the algorithm which may be used for Spectrum Sensing as well as OFDMA demodulation. Because of the Constant False Alarm Rate (CFAR) nature of the detector, the threshold is built into the technique and NO presetting is required. Fine adjustment is possible if needed.

Our technique relies on the non-Gaussianity of a signal to separate it from the Gaussian noise. In the Time Domain, DTV Signals show a Gaussian characteristic. However, in the Frequency Domain they are non-Gaussian. We will exploit this characteristic to detect the signals in AWGN.

2. Signal or Noise Identification Using Higher Order Statistics – Basic Concept

Use of Higher Order Statistics (HOS) is an efficient metric for detecting non-Gaussian signals at low SNRs [1]. In particular, if we assume the noise to be a Gaussian random process, then all the Cumulants of Order greater than 2 are supposed to be zero. Given a set of N samples x = {x0, x1, …, xN-1}, of a random variable x, the  rth order Moment of the collection of samples contained in the vector (x) may be approximated as

mr=1Nn=0N−1(xn−x)r                               (1)

where

x≈1Nn=0N−1xn                                            (2)

Let cr = rth order cumulant of x. Then the relationship between cumulants and the moments may be used to compute the higher order cumulants in a simple fashion as

cn=mn−k=1n−1n−1k−1ckmn−k                              (3)

This relationship may also be written for each of the cumulants by expanding (3) as shown below.

c1=m1c2=m2−m12c3=m3−3m1m2+2m13                                (4)c4=m4−4m1m3−3m22+12m12m2−6m14⋮

Since our data record for the determination of the HOS may not be sufficiently long, the cumulants of Order Greater than 2 may not be exactly zero. Hence, a threshold needs to be set in order to make a decision as to whether the samples belong to Signal or Noise. In this method, we compare the higher order cumulants with the power of second order moment. This is also known as bi-coherency tri-coherency etc. tests to determine if the received waveform belongs to DTV Signal or Noise.

3. Signal Detection for DTV Signals in the Vicinity of the Pilot Using Higher Order Statistics

Figure 1. Signal processing chain for DTV spectrum sensing in the vicinity of pilot using higher order statistics (HOS).

Following is the description of the DTV spectrum sensing algorithm in the vicinity of pilot using HOS.

Step 1. Downshift (Downconvert) the received samples collected in the DTV channel from Radio Frequency (RF) or the Intermediate Frequency (IF) to the base band,

Step 2. Pass the down-converted signal through an image rejection Low Pass (LP) of total Bandwidth (BW1 = 8 MHz) to suppress the image.

Step 3. Upshift (Upconvert) the signal by approximately 2.69 MHz so that if an ATSC DTV signal was present, its Video carrier would be shifted closer to the d.c. or 0 Hertz frequency

Step 4. Pass the resultant signals through a Low Pass (LP) of bandwidth (BW2 = NFFT / ((Tsensing) Z)) followed by downsampling of the signals by a factor of floor(Fs/BW2) where Tsensing  = Sensing Duration, Z = 1, 2, 3, …determines the multiples of the sensing duration. For example, Tsensing = 0.005 and Z = 1 implies the total sensing duration of 5 mS. Similarly, Tsensing = 0.005 and Z = 2 implies the total sensing duration of 10 mS. Fs = sampling frequency of the original received signal. and NFFT  = Size of the FFT  used.

Step 5. Convert the input samples from serial to parallel and convert them to the frequency domain using a Fast Fourier Transform (FFT) of length NFFT. As an example, keeping NFFT = 2048 is advantageous since the same FFT implementation may be used for spectrum sensing as well as OFDMA demodulation of WRAN.

Step 6. Collect the samples at the output of the FFT and store them in a buffer. Determine the higher order moments and cumulants of the real and imaginary portions of the stored samples using Equations (1), (2) and (3). Perform the frequency domain Gaussianity check and hence DTV detection by applying the following steps.

Step 7. Let R be the number of moments mrreal,mrimaginary and cumulants  crreal,crimaginary of the order greater than two available for computation of the real and the imaginary parts of each of the segments (X) of data at the output of the FFT respectively,

- Choose a  Probability Step Parameter 0 < δ < 1; For example let  = 0.5 / R.

- Let PSignalreal=PSignalimaginary=0.5;

- Choose some γ>0;    γtypical=1

- for r = 3 to (R + 2);

if 	 crreal<γm2realr2      ⇒    PSignalreal=PSignalreal−δ

elseif   crreal≥γm2realr2      ⇒    PSignalreal=PSignalreal+δ

end,

if   crimaginary<γm2imaginaryr2      ⇒    PSignalimaginary=PSignalimaginary−δ,

elseif   crimaginary≥γm2imaginaryr2      ⇒    PSignalimaginary=PSignalimaginary+δ

end,

end

- PSignal=a⋅PSignalreal+b⋅PSignalimaginary where a and b weight parameters. As an example a=b=0.5

- Due to the constant false alarm rate (CFAR) nature of the detector, the threshold is built into the system, and NO presetting is required. However, in order to carry out fine adjustment, we have provided the fine sensing threshold parameter , which is used to make fine adjustments of Pfalse alarm if needed. As  increases Pfalse alarm decreases and vice-versa. In most cases  is kept close to unity.

Step 8. if PSignal≥0.5 then X belongs to Signal, and the DTV Signal is detected.

if PSignal<0.5 then X belongs to Noise and the DTV Signal is not detected.

4. Performance Results

The spectrum sensing algorithm using HOS was tested on the 12 DTV signals that were provided. The simulations were carried out for sensing times in multiples of 5 ms however any length sensing time may be used for the actual implementation. The length of the FFT was kept at NFFT = 2048 with the thought that the same harware may be used for spectrum sensing as well as OFDMA demodulation of WRANs. Due to the constant false alarm rate (CFAR) nature of the detector, the threshold is built into the system, and NO presetting is required. However, in order to carry out fine adjustment, we have provided the fine sensing threshold parameter , which enables fine adjustment of the Pfalse alarm at little degradation in Pdetection. Figure 2 shows the average (Pdetection and Pfalse alarm) of the 12 provided DTV signals vs the fine sensing threshold parameter ,  at SNR = -20 dB and sensing duration of 20 mS. Based on this simulation, the value of the fine sensing threshold parameter  was estimated in order to obtain a certain Pfalse alarm. Similar simulations were carried out for other sensing durations as well. Based on the results from Figure 2, the parameter  was kept at 0.8 in order to obtain Pfalse alarm0.05. Figures 3, 4, 5 and 6 show the Pdetection vs SNR performance of the 12 provided DTV signals for 5, 10, 20 and 40 mS sensing times respectively.

Figure 2. Average Pdetection and Pfalse alarm of the 12 provided DTV signals vs the fine sensing threshold parameter ,  at SNR = -20 dB and 20 mS of sensing duration.

Figure 3. Pdetection vs SNR performance for the 12 provided DTV signals for 5 mS sensing using the algorithm using HOS in the vicinity of the pilot.

Figure 4. Pdetection vs SNR performance for the 12 provided DTV signals for 10 mS sensing using the algorithm using HOS in the vicinity of the pilot.

Figure 5. Pdetection vs SNR performance for the 12 provided DTV signals for 20 mS sensing using the algorithm using HOS in the vicinity of the pilot.

Figure 6. Pdetection vs SNR performance for the 12 provided DTV signals for 40 mS sensing using the algorithm using HOS in the vicinity of the pilot.

The spectrum sensing algorithm using HOS was tested on the 12 provided DTV signals as shown above. Table 1 summarizes the results for the average SNR required for the collection of the 12 provided DTV signals at a Pfalse alarm = 0.05 and Pdetection = 0.90 for various sensing durations. This algorithm meets the DTV fine sensing requirements.

| Method | 5 ms (Z = 1) | 10 ms (Z = 2) | 20 ms (Z = 4) | 40 ms (Z = 8) |
| --- | --- | --- | --- | --- |
| Required SNR | -16.6 dB | -18.6 dB | -21.6 dB | -23.7 dB |

Table 1: Required SNR for DTV signal detection (Average over 12 signals)

References

[1] S. K. Shanmugan, A. M. Breipohl, Random Signals, Detection Estimation and Data Analysis, John Wiley and Sons, 1988.

[2] S. M. Kay, Fundamentals of Statistical Signal Processing, Detection Theory, Prentice Hall Signal Processing Series, 1993.

[3] S. M. Kay, Fundamentals of Statistical Signal Processing, Estimation Theory, Prentice Hall Signal Processing Series, 1993.