IEEE P802.22
Wireless RANs

| Robustness of the WRAN signal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Gerald Chouinard | CRC | 3701 Carling Avenue, Ottawa, Ontario, Canada K2H 8S2 | 1-613-998-2500 | gerald.chouinard@crc.ca |

Abstract

This contribution summarizes the relative performance of the different portions of the 802.22 WRAN signal in the WRAN channel model B to establish the system failure mode (22-07-0319-01-0000-FEC_simulation_results.xls).  This can be used to verify the modulation and error correction performance for each portion of the signal.

It can be seen, the DS/US-MAP portion will fail in the same channel conditions as the data payload if the basic convolutional coding is used but it will fail before the data payload if an advanced FEC codec is used.  More protection, such as using a two-time repetition, would therefore be required for the DS/US-MAC or a CRC would need to be included at the end of the frame header to discard an erroneous frame rather than allowing collisions due to capacity allocation errors in the upstream subframe.

The figure also shows that there is no advantage in increasing the robustness of the data payload towards extending the coverage in the downstream direction for more remote CPEs using less sub-channels than that required for the nominal upstream capacity at the edge of coverage while using the maximum allowed transmit EIRP since the DS/US-MAP will fail before.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Figure 1:  Relative performance of the different portions of the 802.22 WRAN signal in the WRAN channel model B