IEEE P802.22
Wireless RANs

| Suggestions on Frame Control Header, DS-MAP and US-MAP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-08-23 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Wendong Hu | STMicroelectronics | 1060 East Brokaw Road, San Jose, CA 95131 | 1-408-467-8410 | Wendong.hu@st.com |

Abstract

This document provides suggested changes to the Frame Control Header, the DS-MAP, and the US-MAP incorporating the group’s decisions and comments arise in the face-to-face meeings and ad hoc teleconferences.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- Frame Control Header Format

| Syntax | Size | Notes |
| --- | --- | --- |
| Frame_Control_Header_Format() { |  |  |
| DIUC | 4 (or 5) bits | The DIUC that defines the burst profile of the DS-MAP/US-MAP message. |
| DS-MAP or US-MAP Length | 12 bits | Size in bytes. This field specifies the Length of either the DS-MAP or the US-MAP message that immediately follows the FCH. In the case DS-MAP is specified, the US-MAP information will be desribed in the first DS-MAP burst. In the case US-MAP is present, there will not be any DS burst in the current frame. DCD and UCD messages are carried by DS bursts specified by the DS-MAP. |
| MAP Length Indicator | 1 bit | 0 – DS-MAP Length is specified, 1 – US-MAP Length is specified. |
| Repetition Code | 1 bit | 0 – no repetition on DS-MAP/US-MAP, 1 – repetition-2 on DS-MAP/US-MAP. |
| Reserved | 6 bits |  |
| HCS | 8 bits | Header Check Sequence |
| } |  |  |

- DS-MAP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| DS-MAP_Message_Format() { |  |  |
| Management Message Type = 1 | 8 bits |  |
| Frame Number | 4 bits |  |
| DCD Count | 8 bits | Matches the value of the configuration change count of the DCD, which describes the downstream burst profiles that apply to this map. |
| BS ID | 48 bits | See Error: Reference source not found |
| Begin PHY Specific Section { |  |  |
| for (i = 1; i  n; i++) { |  |  |
| DS-MAP_IE() | Variable | PHY specific (Error: Reference source not found) |
| } |  |  |
| } |  |  |
| Reserved | 4 bits | All bits set to zero. |
| } |  |  |

- DS-MAP information elements

| Syntax | Size | Notes |
| --- | --- | --- |
| DS-MAP_IE() { |  |  |
| DIUC | 5 bits | Error: Reference source not found |
| If (DIUC == 30) |  |  |
| Extended DIUC Dependent IE | Variable | Error: Reference source not found |
| else { |  |  |
| If (INCLUDE_CID) { |  |  |
| N_CID | 4 bits | Number of CIDs assigned for this IE |
| for (i=0; i<N_CID; i++) |  |  |
| CID | 12 (or 16) bits | Unicast/Multicast/Broadcast CID. |
| } |  |  |
| Length | 12 bits | Number of OFDMA slots linearly allocated to the DS burst specified by this IE. |
| Boosting | 3 bits | 000: normal (not boosted) 001: +6dB 010: -6dB 011: +9dB 100: +3dB 101: -3dB 110: -9dB 111: -12dB |
| } |  |  |
| If (!byte_boundary) |  |  |
| Padding Nibble | 0-7 bits |  |
| } |  |  |

- DIUC values

| DIUC | Usage |
| --- | --- |
| 0 | Self-Coexistence (Passive Mode) |
| 1-28 | Burst Profiles |
| 29 | PAPR Reduction |
| 30 | Extended DIUC |
| 31 | End of MAP |

- DS-MAP extended IE general format

| Syntax | Size | Notes |
| --- | --- | --- |
| DS_Extended_IE() { |  |  |
| Extended DIUC | 4 bits |  |
| Length | 4 bits |  |
| Unspecified Data | Variable | All bits set to zero. |
| } |  |  |

- US-MAP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| US-MAP_Message_Format() { |  |  |
| Management Message Type = 3 | 8 bits |  |
| UCD Count | 8 bits | Matches the value of the Configuration Change Count of the UCD, which describes the upstream burst profiles that apply to this map. |
| Allocation Start Time | 5 bits | Effective start time (in OFDM symbols) of the upstream allocation defined by the US-MAP |
| Begin PHY Specific Section { |  |  |
| for (i = 1; i  n; i++) { |  |  |
| US-MAP_IE() | Variable | PHY specific (Error: Reference source not found) Define upstream bandwidth allocations. Each US-MAP message shall contain at least one IE that marks the end of the last allocated burst. (UIUC=31 as defined in Table 8). |
| } |  |  |
| } |  |  |
| Reserved | 11 bits | All bits set to zero. (Reserved for FDD) |
| } |  |  |

- US-MAP information elements

| Syntax | Size | Notes |
| --- | --- | --- |
| US-MAP_IE() { |  |  |
| CID | 12 (or 16) bits | Error: Reference source not found |
| UIUC | 4 (or 5 ) bits | Error: Reference source not found (See Table 8) |
| If ((UIUC  0) && (UIUC  1)) { |  |  |
| Number of symbols | 3 bits | Number of symbols for the coexistence window at the end of the frame including the one symbol buffer before the CBP burst and the one symbol buffer following the CBP burst to absorb the propagation time (4 or 5 symbols) |
| US-MAP CBP Channel IE | 16 bits | 6.8.4.1.2.3 |
| } else if (UIUC 2) && (UIUC  3) ) { |  |  |
| Number of Subchannels | 4 bits | [Align Figure 11 for allocation spanning the total width of the upstream sub-frame.] |
| } else if (UIUC 4) && (UIUC  6) ) { |  |  |
| Number of Subchannels | 4 bits |  |
| Number of symbols | 5 bits | [31 symbol width could be coded with 5 bits] |
| } else if (UIUC == 30) { |  |  |
| US_Extended_IE() | Variable | Error: Reference source not found |
| } else if (UIUC == 29) { |  |  |
| CDMA_Allocation_IE() | 32 bits | Error: Reference source not found |
| } else { |  |  |
| Burst_Type | 1 bits | This value specifies the burst type for the burst specified by this US-MAP IE. 0 = Bursts are mapped in the time axis over the full width of the upstream sub-frame before incrementing in the frequency axis; 1 = Bursts are mapped in the time axis over segments of 7 symbols before incrementing in the frequency axis and then re-tracing to the lowest unused subchannel in the next 7 symbol segment. The width of the last segment is to be between 7 and 13 symbols depending on the width of the upstream sub-frame |
| Duration | 12 bits | Number of OFDMA slots linearly allocated to the US burst specified by this IE. (up to 60 by 30 slots can be allocated to a US burst.) |
| MDP | 1 bit | Measurement Data Preferred Used by the BS to indicate to the CPE that this upstream allocation is to be preferably used by the CPE for the specific purpose of reporting back any measurement data. The measurement data to be reported is in connection to the specified Transaction ID. In case the CPE does not have anything to report, it can use this allocation for any other data. This is useful, for example, after a quiet period. 0 = Measurement data not required (default) 1 = Measurement data preferred |
| MRT | 1 bit | Measurement Report Type In case MDP == 1, this field indicates which type of report the BS wants the CPE to send back. 0 = Detailed (see Error: Reference source not found thru Error: Reference source not found) 1 = Consolidated (see Error: Reference source not found) |
| CMRP | 1 bit | Channel Management Response Preferred Used by the BS to indicate to the CPE that this upstream allocation is to be used for confirming or not the receipt of the channel management command with the Transaction ID specified. 0 = Channel management response not required (default) 1 = Channel management response required |
| Transaction ID | 16 bits | The transaction ID of the corresponding channel management or measurement request for which a response/report is required by the BS. |
| } |  |  |
| If (!byte_boundary) |  |  |
| Padding Nibble | 0-7 bits |  |
| } |  |  |

- —UIUC values

| UIUC | Usage |
| --- | --- |
| 0 | Self-Coexistence (Active Mode) |
| 1 | Self-Coexistence (Passive Mode) |
| 2 | UCS Notification |
| 3 | BW Request |
| 4 | CDMA UCS Notification |
| 5 | CDMA BW Request |
| 6 | CDMA Ranging |
| 7-28 | Burst Profiles (Total: 4 codecs*4 rates*3 modulations + 1/4QPSK= 49) |
| 29 | CDMA Allocation IE (see Table 9) |
| 30 | Extended UIUC |
| 31 | End of Map |

- CDMA allocation IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| CDMA_Allocation_IE() { |  |  |
| Duration | 5 | Indicates the duration, in symbols, of the allocation. |
| Repetition Coding Indication | 2 bits | Indicates the repetition code used inside the allocated burst. 0b00 – No repetition coding 0b01 – Repetition coding of 2 used 0b10 – Repetition coding of 4 used 0b11 – Repetition coding of 6 used |
| Code | 8 bits | Indicates the Code sent by the CPE. |
| Symbol | 8 bits | Indicates the PHY symbol used by the CPE. |
| Channel | 7 bits | Identifies the channel used by the CPE to send the Code. |
| Usage | 1 bit | This field can mean two different things depending upon the type of Code used. If (Code = Incumbent) This field indicates whether the CPE shall transmit only the MAC header with the notification. 1 = yes; 0 = no |
| } |  |  |

[place document body text here]

References: