IEEE P802.22
Wireless RANs

| NPD Schemes: The 2-bit NPD Indication Field vs. the SPD monitoring NPD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-08-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Baowei Ji | Samsung Telecommunications America | USA | +1 972-761-7167 | BJi@sta.samsung.com |

Abstract

There two proposals regarding the NPD operation in TG1. We present a comparion of the two schemes. To use the 2-bit NPD Indication Field offers much more functions in a straightforward manner.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- Scheme A: The PPD broadcasts various scenarios using the 2-bit NPD Indication Field, which is monitored by other PDs [3].

- Scheme B: The SPDs monitor the 1-bit field of the NPD beacon frame [2].

- Definition:

- Cluster: for the purpose of discussion here, we define a cluster as one PPD together with all the SPDs and the NPD whose information has been aggregated, or to be aggregated into the PPD’s beacon frame.

| - # | - Functions | - Scheme A | - Scheme B | - Comment |
| --- | --- | --- | --- | --- |
| - 1 | - Who monitors the NPD? | - Only the PPD. | - The PPD and all the SPDs. | - Generally, the PPD can monitor the NPD reliably. - However, the SPDs may not be able to reliably monitor the NPD due to fading issue. the NPD could be ‘on’ and ‘off’ to some SPDs from time to time even though the NPD is always ‘on’ to the PPD. |
| - 2 | - Who is monitored by the SPDs? | - Only the PPD. | - Both the PPD and the NPD. |  |
| - 3 | - How do the SPDs in the cluster know there is the NPD? | - The PPD broadcasts ‘01’ in the 2-bit Indication field. | - The NPD broadcasts ‘1’ in the 1-bit NPD field. |  |
| - 4 | - How do the SPDs know that the PPD decides not to use any NPD at all? | - The PPD broadcasts ‘11’ in the 2-bit Indication field. | - ? | - The SPDs do not have to volunteer transmission in this scenario. Unnecessary volunteer transmission is useless, but increases traffic and the probability of collision. |
| - 5 | - How do the SPDs know there is no NPD, and the PPD is requiring volunteers? | - The PPD broadcasts ‘00’ in the 2-bit Indication field. | - ? | - If Scheme B relies on the periodic transmission from SPDs to resolve this issue, there could be no NPD for as long as 8.6 minutes, because many people comment that the SPD updating period shall be long to reduce traffic. |
| - 6 | - How do the SPDs know the PPD has disapproved the previous NPD? | - The PPD changes the 2-bit Indication field from ‘01’ to ‘00’ or ‘11’. | - ? | - The PPD may find a better candidate for the NPD given it knows the locations of all the PDs in the cluster and given the comes and goes of the PDs. |
| - 7 | - How do the SPDs behave after the the PPD ceases transmission without notice? | - If there is not any beacon transmission for macMaxMissedBeaconsSPD superframe periods, the SPDs know there is neither PPD nor NPD, so the SPDs will contend to be the new PPD(s). | - Case a: If there is NPD, the SPDs do nothing. - Case b: f there is no NPD, the SPDs contend to be the new PPD(s). | - In Case a of Scheme B, the NPD could fail to be the new PPD. The SPDs learns this after - macMaxMissedNPDCodes * macNPDPeriod superframe periods. - Logically, macMaxMissedNPDCodes * macNPDPeriod shall be larger than macMaxMissedBeaconsNPD. This is another piece of evidence that Scheme B, if adopted, has to be designed very carefully to avoid the parameters violate each other. - In Scheme A, some SPDs may not be covered by the new PPD. The wasted time for those SPDs is the minimum of macMaxMissedBeaconsSPD and macMaxMissedNPDCodes * macNPDPeriod superframe periods. - Most likely, macMaxMissedNPDCodes * macNPDPeriod is far larger than macMaxMissedBeaconsSPD because a small macMaxMissedNPDCodes would dramatically increase traffic and the probability of collision. Actually, the collision could be very high to use 5 for macMaxMissedNPDCodes as in the draft. |

# References:

[1] Part 22.1: Enhanced Protection for Low-Power, Licensed Devices Operating in Television Broadcast Bonds, P802.22.1/D1, May 2007

[2] 22-07-0xxx-00-0001_SPDs monitoring the NPD.doc

[3] 22-07-0338-02-0001 Comment-on-the-ceasing-transmission-of-NPD-SPD-and-PPD.doc