IEEE P802.22
Wireless RANs

| Text for Binary Interleaving |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-08-28 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Isabelle Siaud | Orange Labs | Rue du Clos Courtel 35510 Cesson-Sévigné France | +33 299 124 629 | isabelle.siaud@orange-ftgroup.com |
| John Benko | Orange Labs | 801 Gateway Blvd. Suite 500, South San Francisco, CA 94080 | 650-875-1593 | john.benko@orange-ftgroup.com |

Abstract

Definition of the proposed binary interleaver which is applied to the encoded bits (blocks) after the FEC.  This has been verified to perform well with both BCC and the Duo-Binary Turbo Code.  This scheme should also improve performance of the other advanced FECs.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

  - Introduction (informative)

Encoded bits at the output of the convolutional encoder of the FEC code are interleaved to break error bursts due to channel multi-path and fast fading.  The same basic interleaver L(k), used for sub-carrier interleaving is used (see sub-carrier allocation clause).   The selection of {p,q,j} interleaving parameters is oriented on a maximization of the interleaving spreading  to enlarge the interleaving spreading in a multi-level scale between encoded bits separated by s-1. Several s values are considered to maximise interleaving spreading between adjacent bits within every data symbol and between data symbols at the binary level.  The interleaving spreading between adjacent bits within every data symbol consists in selecting {p,q,j} values giving the largest L(s) for s values ranged from 1 to NCBPS-1. NCBPS is the number of encoded bits per data symbol (data sub-carrier).  The maximization of the binary interleaving spreading between adjacent symbols consists in selecting {p,q,j} values giving the largest L(s) for s values multiple of NCBPS ([s]NCBPS=0, ie between equally weighted bits associated adjacent data symbols.  Common interleaving parameters {p,q,j} fulfilling these two technical requirements are selected for the binary interleaving process performed upon K encoded block sizes. Associated binary interleaving parameters of the algorithm that provide the highest interleaving spreading are given in the next section.

  - Binary (bit) interleaving

The same basic interleaver L(k), used for sub-carrier interleaving is used (see sub-carrier allocation clause). L(k) is the position of the k-th bit after the binary interleaving (k=0, 1, …, K-1).  The interleaving algorithm L(k)=I(j)p,q(k) (detailed in the sub-carrier allocation clause) is described by the block size K and three integer parameters {p,q,j}.

| Size K (bits) | p | q | j | DI(s=1) | DI(s=2) | DI(s=3) | DI(s=4) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2304 | 16 | 2 | 2 | 799 | 706 | 93 | 892 |
| 2112 | 16 | 2 | 2 | 799 | 514 | 285 | 1028 |
| 1824 | 16 | 2 | 2 | 737 | 350 | 387 | 700 |
| 1728 | 36 | 2 | 1 | 793 | 142 | 651 | 284 |
| 1680 | 40 | 2 | 2 | 559 | 562 | 3 | 556 |
| 1632 | 16 | 2 | 2 | 545 | 542 | 3 | 548 |
| 1440 | 40 | 2 | 2 | 559 | 322 | 237 | 644 |
| 1248 | 16 | 2 | 2 | 319 | 610 | 291 | 28 |
| 1152 | 36 | 2 | 1 | 359 | 434 | 75 | 284 |
| 1008 | 36 | 2 | 1 | 361 | 286 | 75 | 436 |
| 960 | 16 | 2 | 2 | 415 | 130 | 285 | 260 |
| 864 | 16 | 2 | 2 | 223 | 418 | 195 | 28 |
| 672 | 16 | 2 | 2 | 223 | 226 | 3 | 220 |
| 576 | 36 | 2 | 1 | 217 | 142 | 75 | 284 |
| 480 | 40 | 2 | 2 | 79 | 158 | 237 | 164 |
| 336 | 16 | 2 | 2 | 113 | 110 | 3 | 116 |
| 288 | 16 | 2 | 2 | 65 | 130 | 93 | 28 |
| 192 | 16 | 2 | 2 | 31 | 62 | 93 | 68 |
| 96 | 16 | 2 | 2 | 31 | 34 | 3 | 28 |