IEEE P802.22
Wireless RANs

| Proposed Solutions for Primitives for Channel Scanning |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-09-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Soo-Young Chang | Huawei Technologies | Davis, CA, U. S. A. | 916 278 6568 | sychang@ecs.csus.edu |
| Jianwei Zhang | Huawei Technologies | Shanghai, P. R. China | 86-21-50993036 | zhangjianwei@huawei.com |

Abstract

In the current draft, a couple of primitives are missing. Thus in this document, these missing primitives are proposed for channel scanning.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

1. INTRODUCTION

In the current draft of TG1 standard [1], there are several primitives in each of the MAC and PHY layers. Among them, a primitive should be modified and a couple of primitives should be added to accommodate all the functions needed in the draft as follows. Some comments were suggested to point out these missing primitives [2].

Channel scanning

In the current draft, only two MLME primitives are defined to request the MAC to listen a channel or channels. Thus more primitives are needed for PLME.

In Section 2, more primitives are proposed. Newly proposed primitives and their parameters are marked in red which should be added to the current draft.

2.  PRIMITIVIES IN THE CURRENT DRAFT AND ADDITIONALLY NEEDED

In the current draft two primitives are defined as follows in the channel scanning area. One of them should be modified and two primitives should be added.

In MAC sublayer

MLME-SCAN.request

This primitive is issued from the next higher layer of a protecting device to its MLME to initiate a listening period on a given channel or channels.

Two parameters are needed for this primitive – ChannelList and Duration (Table 33 on page 39).

MLME-SCAN.confirm

This primitive is issued from the MLME of a protecting device to its next higher layer to notify the channel scanning result upon the receipt of PLME-SCAN.confirm.

One parameter should additionally be defined– ChannelList (Table 34 on page 40) for channels heard.

In PHY

PLME-SCAN.request: 6.2.2.9

This primitive is issued from the MLME of a protecting device to its PLME to initiate a listening period on a given channel or channels.

Two parameters are needed – ChannelList and Duration (will be included on page 24).

PLME-SCAN.confirm: 6.2.2.10

This primitive is issued from the PLME of a protecting device to its MLME to notify the channel scanning result.

One parameter is needed – ChannelList for channels heard (will be included on page 24).

3.  PROPOSED TEXT OR TEXT CHANGES

Modification of  Table 7 (page 21)

Table 7—PLME-SAP primitives

| Name | Request | Confirm |
| --- | --- | --- |
| PLME -GET | 6.2.2.1 | 6.2.2.2 |
| PLME –INITIATE-RTS-BURST | 6.2.2.3 | 6.2.2.4 |
| PLME -SET | 6.2.2.5 | 6.2.2.6 |
| PLME –SET-RTX-STATE | 6.2.2.7 | 6.2.2.8 |
| PLME-SCAN | 6.2.2.9 | 6.2.2.10 |

6.2.2.9 PLME-SCAN.request

The PLME-SCAN.request primitive is generated by the MLME and issued to its PLME to initiate a listening period on a given channel or channels. Table X specifies the parameters for the PLME-SCAN.request primitive.

Table X—PLME-SCAN.request parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ChannelList | Bitmap | 69-bit field | The list of channels scanned. The 69 bits (b0, b1, …, b68) indicate which channels are scanned (1 = scan, 0 = do not scan). |
| Duration | Integer | 0-100 | A value used to calculate the length of time to spend scanning each channel. The length of time spent scanning each channel is 2 + 0.01m seconds, where m is the value of the Duration parameter. |

6.2.2.10 PLME-SCAN.confirm

The PLME-SCAN.confirm primitive is generated by the PLME and issued to its MLME when the channel scanning operation initiated with the PLME-SCAN.request primitive has been completed. Table (X+1) specifies the parameters for the PLME-SCAN.confirm primitive.

Table (X+1)—PLME-SCAN.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Status | Enuneration | SUCCESS or FAILURE | The result of the attempt to scan the channel list |
| ChannelList | Bitmap | 69-bit field | The list of channels scanned and found. The 69 bits (b0, b1, …, b68) indicate which channels are detected (1 = detected, 0 = not detected). |

On receipt of the PLME-SCAN.confirm primitive the MLME is notified that the request to scan the specified list of channels was accepted and the operation was completed with detected channel information. A Status parameter equal to COMPLETE indicates that the scan is complete.

Modification of  Table 34 (page 40)

Table 34—MLME-SCAN.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Status | Enuneration | SUCCESS or FAILURE | The result of the attempt to scan the channel list |
| ChannelList | Bitmap | 69-bit field | The list of channels scanned and found. The 69 bits (b0, b1, …, b68) indicate which channels are detected (1 = detected, 0 = not detected). |

4.  CONCLUSIONS

A couple of new primitives and parameter modification of a primitive are proposed which are missing in the current draft to accommodate all the functions suggested in the draft for channel scanning.

References

[1] Part 22.1: Enhanced Protection for Low-Power, Licensed Devices Operating in Television Broadcast Bands, P802.22.1/D1, May 2007

[2] P802.22.1d1.0_cmts_007, Aug. 2007