IEEE P802.22
Wireless RANs

| Burst Profile Definition |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-30 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Gerald Chouinard | CRC | 3701 Carling Avenue, Ottawa, Ontario, Canada K2H 8S2 | 1-613-998-2500 | gerald.chouinard@crc.ca |

Abstract

This contribution describes the two options for defining the burst profiles between the base station and its CPEs and the different aspects to be considered in selecting a dynamic burst profile definition or a station definition.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Burst Profile Definition

During the San Francisco plenary session of the 802.22 WG, the number of active profiles at any specific time, as defined in the downstream and upstream definition tables of the DUIC and UIUC respectively, was considered.  It was suggested that, because of the large variability of transmission and reception conditions in a WRAN 802.22 operation, which can cover typical distances of up to 30 km, the number of active burst profiles should be augmented from what it was originally proposed in the 802.16 standard (i.e., DIUC and UIUC profiles defined by 4 bits, thus limiting the number of active profiles at any time to 16, including the specific burst profiles reserved for the system management).

Further discussion took place when it was discovered that the scheme that was proposed, taken from the 802.16 standard, was defining the profiles in a dynamic fashion, i.e., that these profiles and their related parameters (entry and exit signal level thresholds) were downloaded form the base station to the CPEs.  A new static scheme was proposed where each CPE would upload a list of profiles to the base station based on its capabilities when it associates with it and these profiles, by definition known to the CPE would be used for any further communication with the CPE rather than using the dynamic scheme where generic profiles are downloaded to the CPEs.  This way, the number of indices for the burst profiles needed at any time for operation would be reduced.

This contribution tries to explore the pros and cons of each method.

### Total number of burst profiles

The total number of burst profiles that can be used by a WRAN system is defined as follows:

Number of codecs:	4	(convolutional/Viterbi, DBTC, LDPC, SBTC)

Number of code rates:	4	(1/2, 2/3, ¾, 5/6)

Modulation levels:	3	(QPSK, 16QAM, 64QAM)

Total:	4*4*3= 48

+ 1	(QPSK, rate: ½, repetition of 2 for US-MAP, DCD and UCD)

49

### Upstream burst profiles

What has been brought from 802.16 is that the base station will download its table of upstream burst profiles to the CPEs from time to time using the UDC management packet.  A specific UIUC index will refer to the same burst profile across all CPEs.  For full flexibility, the 49 burst profiles would need to be defined and active at the same time to accommodate all transmission/reception situations in the WRAN cell.  This would mean a minimum of 6 bits to define the UIUC indices for each packet transmitted.  It would also mean that the base station would need to downtload all these profiles dynamically to all CPEs eventhough the CPEs would be able to implement only a subset of these profiles.

In the case of static profiles, the CPEs would signal the profiles that they can handle when they associate with the base station and the meaning of each burst profile index for each CPE would be tabulated at the base station and then used in the US-MAP.  Since it is unlikely that a CPE would implement more that the two codec schemes, the mandatory convolutionan/Viterbi and one of the advanced codecs, the total number of indices used per CPE would be 25 which could fit in a 5-bit UIUC table (22 burst profile indices are currently available in the UIUC table.  The entry and exit signal level threshold for each level of modulation/coding would still need to be downloaded to the CPEs from the base station.  These could be specific to the type of CPE to take into account their specific performance and behaviour.

### Downstream burst profiles

In the case of the downstream, it is assumed that the same burst profiles as for the upstream would be applicable for the same CPEs.  The same set of indices could then be used as defined at the time of association of the CPE.  However, since the downstream will also have to carry broadcast and multicast packets, burst profiles using the mandatory convolutional/Viterbi scheme will need to be defined as common to all CPEs.  This represents a maximum of 13 burst profiles for the broadcast/multicast packets.  The indices for these burst profiles could use indices in the DUIC table that are needed for other special purposes in the UIUC table, i.e., indices 1 to 6.

_______________________________________